Improvement of Models of Effective Use of Motor Vehicles in Transporting Wheat Grain from Crop Fields

M.N. Jurayev1, a) and D.K. Khakimov1, b)

1Tashkent State Transport University, 1 Temiryulchilar St., Tashkent 100167, Uzbekistan

a) Corresponding author: mdjurayev82@mail.ru
b)xakimovdiyorjon1817@gmail.com

Abstract. The article develops an analytical expression of the dependence of the number of vehicles required for transporting wheat grain on the technical and operational indicators of combines used for harvesting in the field and vehicles involved in transportation, field productivity, and the distance from the field to the warehouse, and the correlation of the indicator is depicted in the form of graphs. The problem of distributing the identified number of motor vehicles to their routes was posed as an optimization problem, expressed in the form of a mathematical model, and a solution algorithm was presented.

Keywords: wheat, motor vehicles, combine harvester, transportation, route, distribution, discrete, aggregate, combination

# INTRODUCTION

Research is being conducted to optimize freight transportation in the national economy, aiming to develop low-cost transport vehicles, efficiently meet consumer transportation volume needs, reduce transportation costs, and enhance the productivity of automotive transport vehicles [1, 2, 3, 4].In this regard, in the Republic of Uzbekistan, it is necessary to fundamentally improve the system for implementing comprehensive measures related to the efficient use of convenient transport vehicles for freight flows in the agricultural sector, while considering transport-operational and social indicators in line with the growth dynamics of future freight flows. In particular, the "Concept for the Development of the Transport and Logistics System of the Republic of Uzbekistan until 2030," approved by the Decree of the President of the Republic of Uzbekistan No. PQ-28 dated January 27, 2025, on measures to further develop the transport and logistics system, sets out tasks to continue reforms aimed at fully meeting the economy and business's need for high-quality freight transportation services. To fulfill these designated tasks, based on the objectives of reforming the economy of New Uzbekistan, it is deemed relevant to conduct systematic research on improving economic, technical, and organizational methods for enhancing the freight transportation system in our country, based on logistics principles, particularly in the delivery process using automotive transport.

THE MAIN PART

Various brands of automotive transport vehicles are used to provide transportation services during the wheat harvesting process. It is necessary to organize the comprehensive and efficient operation of transport and handling processes by balancing the ratio of automotive transport vehicles to combines. Studies have shown that the coordinated operation of automotive transport vehicles and combines depends on numerous factors. These include the productivity of the combine, the yield of grain crops, the moisture content of grain and straw, downtime due to technical-technological reasons, and other factors. Additionally, the performance of automotive transport vehicles depends on factors such as their technical condition, the qualifications of drivers, average speed, road conditions, and other factors [3, 4, 5]. According to data, the financial loss per hour of combine downtime is 3 to 4 times higher than that of automotive transport vehicles. Therefore, to eliminate or reduce the downtime of combines waiting for transport vehicles, it is necessary to increase the number of transport vehicles within a limited range.

The amount of economic loss (F) resulting from the downtime of combines during the working day and the ineffective use of daily working time of motor vehicles, where the objective function is expressed as follows (Formula 1):

F= ck∙ λ∙ tk+ (sа1+ sа2)∙ na→min 	(1)

In this, ck – damage from combines left idle in the field during the working day, soums; λ – average number of bunkers filled during the working day; tk – the sum of the average waiting times for each order, hours; ca1 – the cost of economic damage caused by a vehicle waiting for a combine harvester during a working day, so‘m/day; sa2– The cost of economic damage resulting from the ineffective use of a road transport operator's daily working time during the working day, so‘m/day; na – number of vehicles in the transportation process.

When determining the number of vehicles required to transport the grain using combines, the productivity of the combines must be taken into account. Therefore, we calculate the net cycle time of the grain harvesting combine (Formula 2).

tks=tloadb+tunloadingb, clock 	(2)

In this, tks - net cycle time of the combine, clock; tloadb - time to load the combine's bunker, clock; tunloadingb - time to empty the combine's hopper, clock;

As a result of determining the cycle time of the combine and the vehicle, we can see the following two states of the harvesting-transportation process:

In the first case, , tas<tks will be, that is, each combine harvester will have one vehicle with the appropriate body size and the appropriate load capacity (Formula 3).

Vna=mnaγ, m3 	 (3)

In this, Vna – vehicle capacity, m3; mna - load capacity of the relevant motor vehicle, t; γ - grain density, t/m3.

In the second case, tas>tks will be, that is, the required number of motor vehicles is obtained from calculating the volume of crushed grain, taking into account the round trip time of the vehicle.

The number of combine bunkers per vehicle used for transportation is (Formula 4):

nb=qγ∙Q 	(4)

In this, q– Rated load capacity of a motor vehicle, t; γ- wheat density, t/m3; Q– combine harvester bunker capacity, m3.

As a result of the research, an analytical expression for determining the number of vehicles required to transport wheat grain from the field was developed, depending on the technical performance indicators of the combines used for harvesting, the average technical speed of the vehicle involved in the transportation, the transportation distance, and the loading and unloading times (Formula 5):

NА=gd∙Vk∙Bish10∙Q∙γ∙2∙tha+ty−t;	(5)

In this, considering that , 2∙tha+ty−t=tayl;;, the following expression is formed (Formula 6):

NА=1Q∙); 	(6)

Thus, based on the research results, it becomes possible to determine the rationally justified necessary number of motor vehicles for transporting grain crops from the fields to the grain storage facilities. This will allow for the rational exploitation of agricultural organizations and vehicles hired from outside for the harvest period. This takes into account not the entire rolling stock of vehicles, but the load capacity of each vehicle, the number of combines deployed to harvest grain crops, and the technical specifications of each combine. As a result of the calculations, we obtain the minimum number of vehicles of each model, while achieving the maximum payload, which leads to an increase in the productivity of work in the "harvest-harvest-transport" system.

It is also important to note that when determining the number of vehicles for a separate combine harvester, the vehicles spend part of their working time waiting for the combine's bunker to fill [3]. The reduction in the number of vehicles leads to the idle time of the combines. Recently, there has been a trend of increasing the capacity of combine harvester bunkers. Therefore, in order to effectively organize transportation processes, it is necessary to study these processes.

Other conditions with the use of the expression identified during the study (productivity, mower coverage, vehicle and combine speed, etc) is equal, we will see the effect of changing the bunker capacity on the required number of cars.

We assume the capacity of the bunker to be 6; 9; 10; 11; 12 m3..

To make it convenient to calculate the number of cars NАi depending on the capacity Q of the combine's bunker, we define the remaining parameters of expression (Formula 7) by some constant M.

NАi=1Q∙) 	(7)

In this gd- cropland productivity, Вish - combine harvester coverage width, Vk - combine harvester operating speed,
γ- density of wheat grain.

In expression (2.10) we insert the values of the quantities inside the brackets:

gd=36 c/h; Вwork = 5 m; Vk = 7 k/h; γ = 800 k/m3 or 0,80 t/m3.tayl12=2.58h; in this case M=40.63.

The graphs show the dependence of the required number of vehicles used to transport wheat grain on the volume of the combine bunker (Figure 1) and on crop yield (Figure 2), as well as on the coverage width of the combine harvester's sickle (Figure 3).

FIGURE 1. Change in the number of vehicles depending on the size of the combine bunker

FIGURE 2. Graph of changes in the number of vehicles used for transportation depending on productivity

FIGURE 3. Change in the number of cars depending on the width of the combine harvester blade

Thus, an increase in the capacity of the combine harvester's bunker leads to a decrease in the need for vehicles, other things being equal (yield, width of coverage of the harvester, speed of the vehicle and the combine, etc.) (Figure 2). As can be seen from Figure 2, with an increase in yield, the number of vehicles required to ensure a continuous harvesting process increases linearly. The resulting analytical relationship allows us to calculate the required number of cars at a given productivity. To calculate the required number of vehicles depending on the coverage width of the combine harvester's sickle, sickles with coverage widths of 5.0; 6.0; 6.7; 7.0; 9.0; 10.7; 11.0 m were selected. The information obtained is represented by a polynomial function and can be used to calculate the optimal number of vehicles needed to ensure a continuous harvesting process.

Research is conducted to divide the organization of the transportation process during the harvesting process into the following stages loading the combine bunker, emptying the combine bunker, moving the loaded vehicle from the field to the grain receiving area, unloading and receiving it, and moving the vehicle back to the field for the next trip. In this case, the composition of the technical and technological processes being carried out and the time spent on their execution sequence, as well as the indicators of the equipment used during the season in the processes of harvesting (harvesting) wheat grain from the studied fields and transporting it by road to the grain receiving areas, are determined. In this case, the minimum number of motor vehicles required to transport wheat grain from the field to the grain receiving facility in the studied area is determined and distributed to various transportation routes based on daily productivity and transportation costs.

The fulfillment of the specified cargo transportation volumes, cargo flows transported to it from shippers along the routes and these flows are based on the effective distribution of road transport vehicles to the routes, the number of road transport vehicles distributed to the routes, the full use of the capabilities of their equipment for sending (or receiving) cargo, and the minimum transportation costs in accordance with the selected criteria.

The problem of optimal distribution of road transport vehicles allocated for transportation on routes is posed and solved in the form of a linear programming model with a mathematical nature, which leads to the following shortcomings and inconveniences:

1. It is necessary to determine the optimal number of loading and unloading mechanisms required to service the vehicles allocated to each route.

2. The solution to the distribution problem requires rounding off non-integer numbers, which further reduces the optimality of the solution.

3. The requirement for full loading of loading and unloading mechanisms moving along each transportation route with road vehicles during the daily working hours is not taken into account.

Such requirements cannot be taken into account in the form of a linear programming model [2, 6, 7]. Therefore, the problem of distributing vehicles on routes requires the use of combinatorial models and heuristic algorithms for solving it.

In the discrete formulation of the problem of allocating vehicles to routes, the solution obtained will consist of values that belong to a discrete set containing the possible solutions, rather than any arbitrary values, representing the number of vehicles assigned to the routes [1, 8, 9]. The elements of the discrete set consist of the number of vehicles that can be allocated for each route. In general, solving the problem involves determining the number of vehicles assigned to each route, expressed as elements of a specific discrete set.

The volume of cargo transported in each direction of each collection or distribution route (number of vehicles) is determined in accordance with the technological needs of the connecting destination where the cargo is collected or distributed. When justifying the issue of distributing motor vehicles on routes, it is necessary to take into account the following requirements:

- Vehicles of different carrying capacities are distributed to routes with different transportation distances from the point of view of minimizing total transportation costs;

- The vehicles are allocated to transport routes in such a way that the cluster destination’s demand for cargo volume is met. The management of the cargo volume being delivered to or picked up from the cluster destination is adjusted according to its demand. This is achieved by changing the number of vehicles allocated to routes with varying distances, without changing the total number of vehicles used. In other words, if the cluster destination has a higher demand for cargo delivery (or distribution), more vehicles are allocated to shorter routes, or conversely, if the demand is smaller, the vehicles are allocated to longer routes.

The first requirement above does not apply in situations where cargo transportation on routes with different distances is carried out by road vehicles with the same carrying capacity. Because the total transportation costs will be the same in any option within the second requirement above for distributing the same type of motor vehicles over routes of different lengths.

Distribution of vehicles with different carrying capacities into routes - this is the allocation of vehicles for cargo transportation on each route. The number of motor vehicles consists of determining them by type and determining the number of trips each motor vehicle should make on a given route[10, 11].

Transportation to the consumer's address is carried out in routes. These routes are numbered 1, 2, 3, etc. in order of increasing transportation distance. We denote the set of route numbers by J. The planned transportation process is carried out by road vehicles of various carrying capacities. Road vehicles are numbered in order of increasing load capacity, and this number set is given as I For each j-th route, the set of loading and unloading mechanisms (σj ) that can be involved in cargo loading (receiving) operations is given as Σj ={0,1,2…σj}. When there are (σj=1) shipping (or receiving) mechanisms operating on a route, the number of vehicles γi that must be allocated for efficient loading during the day is given for each i-th type. The number of motor vehicles allocated to each j-transport route should be determined in such a way that the Xjσji is determined in such a way that it provides for the efficient use of motor vehicles used for each i-type and rational loading of a certain σj-number of sending (or receiving) mechanisms: That is, the number of vehicles jϵJ that can be allocated to each j-th route must belong to the set Gjσji given by Xjσji. This set is given in the form of a matrix with the following order (Formula 8):

Gjσji=XjσjiJ x10i, x20,i,... xj0i x11i, x21i,..., xj1i	 (8)

When setting the problem, we should also be given the following parameters:

Number of motor vehicles of typeAэi− i-;

Daily productivity in tons of road vehicles of type Qi j− on the j-th transportation route i:

Qmin, Qmax - the minimum and maximum level of the consumer's need for transportation volume.

We introduce the following concepts to develop a method and algorithm for solving the problem.

The solution to this discrete problem is a collection of sequences of Xjσjielements that meet certain requirements. Therefore, a set of Xjσji elements is any sequence of them obtained from the XjσjiJ matrix. For a set to be called a feasible set, the following conditions must be met:

Only elements with the same index i are included in the set;

The number of accumulated Xjσji elements is equal to the number of incoming (outgoing) routes to the cluster destination;

The set does not contain two elements with the same index j.

When forming and analyzing various combinations of elements of the Xjσji assembly of motor vehicles, using their numbers instead of their sums greatly simplifies the problem. For this reason, we introduce the concept of the sum number.

A possible set of numbers is a set of indices indicating the values of the elements in the 1st, 2nd,..., eth...cth positions in its sequence. A possible set number is a set of ψ1i,...,ψei,...ψci indices indicating the values of the elements in the 1st, 2nd,..., eth...cth positions in its sequence. It is self-evident that sets differ from each other in their numbers by at least one ψei index having a different value. A set of Xjσji elements is considered defined if its number is given.

For full use of motor vehicles put into operation during the day, the number of motor vehicles in the Aэi category should be fully distributed to routes, that is, in the form of a set of corresponding Xjσji motor vehicles. Therefore, we introduce the concept of the set of motor vehicles (mg) within the demand - the set of motor vehicles that meets the demand for full use of motor vehicles. The aggregate (mg) within the scope of the requirement for motor vehicles of type i is defined as the aggregate that meets the following condition (Formula 9):

j∈JXjσji=Aэi, хjσj∈Gjσj(M), i∈I 	(9)

here, The set of possible combinations of the elements Gjσj(м)− and Xjσji.

The solution to the problem is to determine the combination of the aggregates (Xjσji) of automobile vehicles in the demand range for all its types iϵI such that the following condition is met for this combination (Formula 10):

Qmin≤i∈Ij∈JXjσjimgQi jQmax, Xjσjimg∈Gjσjmg, 	(10)

here, Xjσjimg− elements of the collection within the scope of the request. Gjσjmg− a set of collections within the scope of the request.

The solution to the problem is determined from among the possible combinations of vehicles in the demand range, each of which satisfies the conditions (3.32) and takes into account the various properties of the problem.

# APPLICATIONS

Nine combines serving 320 hectares of cultivated land in the Kamashi district, with a yield ranging from 38 to 44 quintals, are operating at 3 harvesters per hour, with a daily working time of 13 to 14 hours. The minimum number of motor vehicles (N_Ai) required to transport 12,780 tons of wheat grain planned in the region from the field to the grain receiving facility was determined to be 11. To distribute Kamaz 65207 , HOWO 6*4 and  MAN TGS 40.400 vehicles to different transportation routes, a matrix of their daily productivity and transportation costs was compiled (Table I), and based on this data, using an algorithm for effectively distributing vehicles of different carrying capacities to routes, vehicles allocated for grain transportation were distributed to routes (Table II).

TABLE 1. Indicators

| Performance and transportation cost indicators by type of motor vehicle on routes of various distances | Route numbers and distances |  |  |
| --- | --- | --- | --- |
|  | j=1.Transportation distance - 12 km | j=2. Transportation distance - 20 km | j=3 Transportation distance - 30 km |
| Kamaz 65207 The work efficiency of motor vehicles, Q1j (t) | 60 | 48 | 36 |
| HOWO 6*4 The work efficiency of motor vehicles, Q2j (t) | 60 | 45 | 45 |
| MAN TGS 40.400 The work efficiency of motor vehicles, Q3j (t) | 60 | 60 | 60 |
| Kamaz 65207 The work efficiency of motor vehicles, S1j (so’m/t) | 19600 | 27600 | 38600 |
| HOWO 6*4 The work efficiency of motor vehicles, S2j (so’m/t) | 28000 | 36000 | 47000 |
| MAN TGS 40.400 The work efficiency of motor vehicles, S3j (so’m/t) | 42000 | 50000 | 61000 |

TABLE 2. Indicators

| № | Types of motor vehicles | Route numbers and distances |  |  | XjiQij | ∑Sj (so‘m) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | j=1. Transportation distance -12 km | j=2. Transportation distance -20 km | j=3. Transportation distance -30 km |  |  |
| 1. | Kamaz 65207 | 0 |  |  | 2100 | 92400000 |
|  | HOWO 6*4 |  | 20 |  |  |  |
|  | MAN TGS 40.400 |  |  | 20 |  |  |
| 2. | Kamaz 65207 | 10 |  |  | 2400 | 88560000 |
|  | HOWO 6*4 |  | 10 |  |  |  |
|  | MAN TGS 40.400 |  |  | 20 |  |  |
| 3. | Kamaz 65207 | 10 |  |  | 2100 | 69360000 |
|  | HOWO 6*4 |  | 20 |  |  |  |
|  | MAN TGS 40.400 |  |  | 10 |  |  |
| 4. | Kamaz 65207 | 20 |  |  | 2160 | 68496000 |
|  | HOWO 6*4 |  | 10 |  |  |  |
|  | MAN TGS 40.400 |  |  | 10 |  |  |
| 5. | Kamaz 65207 | 20 |  |  | 2160 | 86496000 |
|  | HOWO 6*4 |  | 0 |  |  |  |
|  | MAN TGS 40.400 |  |  | 20 |  |  |
| 6. | Kamaz 65207 | 20 |  |  | 1860 | 58896000 |
|  | HOWO 6*4 |  | 20 |  |  |  |
|  | MAN TGS 40.400 |  |  | 0 |  |  |
|  |  | 80 | 80 | 80 | 12780 | 464208000 |

# CONCLUSION

The article focuses on optimizing the efficient use of motor vehicles for transporting wheat grain from agricultural fields in Uzbekistan. The study aims to reduce transportation costs, enhance the productivity of transport vehicles, and develop the logistics system. It is based on the Decree of the President of Uzbekistan dated January 27, 2025 (PQ-28) and the "Concept for the Development of the Transport and Logistics System Until 2030," which emphasize systemic improvements in economic, technical, and organizational methods for transport logistics. The research considers the coordinated operation of harvesters and transport vehicles, factoring in variables such as harvester productivity, grain moisture, road conditions, and driver qualifications. Analytical models and a target function (F) were developed to minimize economic losses due to harvester downtime and inefficient vehicle use. These models account for parameters like harvester bunker capacity, vehicle load capacity, and transport distance to determine the required number of vehicles. Findings indicate that increasing harvester bunker capacity reduces the need for transport vehicles, while higher crop yields linearly increase the required vehicle count. Additionally, optimal vehicle allocation to routes was addressed using linear programming and combinatorial models. In a practical case in Qamashi district, covering 320 hectares and transporting 12,780 tons of wheat, a minimum of 11 vehicles was determined, with their allocation to routes based on daily productivity and transportation costs. Overall, the article proposes practical solutions for agricultural organizations to rationally utilize transport vehicles and enhance the efficiency of the harvest-transport process.

REFERENCES

- Sh. A. Butaev and M. N. Juraev, Models and methods of effective distribution of motor vehicle transportation capacity on radial routes (National Library of Uzbekistan named after A. Navoi, Tashkent, 2012), 186 p.

- A. V. Velmozhin, V. A. Gudkov, and L. B. Mirotin, Theory, organization and management of automobile transportation: the logistic aspect of the formation of transportation processes (RPC “Polytechnic,” Volgograd, 2001), 179 p.

- A. A. Popov and I. V. Bumbar, “Determination of the optimal number of vehicles in the harvesting and transport complex,” Far Eastern Agrarian Bulletin 3(59), 99–105 (2021).

- E. G. Veremeyenko, Improving the level of motor transport services for the port’s grain terminal, Dissertation (Moscow, 2016), 167 p.

- S. V. Delz, Development of the transport and logistics system for export grain transportation, Dissertation (Moscow, 2012), 189 p.

- “Solving transport problems using programming in the MathCAD system,” Young Scientist 5, 8–13 (2014).

- E. I. Vatutin, V. S. Titov, and S. G. Emelianov, Fundamentals of discrete combinatorial optimization (Argamak-Media, Moscow, 2016), 270 p.

- M. Zhuraev, J. Togaev, and Z. Yusufkhonov, “Management of consumers needs for volume of transportation, taking into account the probable nature,” E3S Web Conf. 401, 01066 (2023). doi:10.1051/e3sconf/202340101066.

- A. Shermukhamedov, M. Jurayev, and B. Ergashev, “Combination method for determining individual solutions to the problem of rational distribution of vehicles on radial routes,” AIP Conf. Proc. 2432, 030016 (2022). doi:10.1063/5.0089743.

- M. Zhuraev, J. Togaev, and Z. Yusufkhonov, “Management of consumers needs for volume of transportation, taking into account the probable nature,” E3S Web Conf. 401, 01066 (2023), CONMECHYDRO–2023. doi:10.1051/e3sconf/202340101066.

- J. M. Velarde, S. García, M. López, and A. Bueno-Solano, “Implementation of a mathematical model to improve sustainability in the handling of transport costs in a distribution network,” Sustainability 12, 5189 (2020). doi:10.3390/su12125189.