Mathematical Modeling of Changes in Wear Resistance Properties of Aluminum Alloys Under the Influence of Modifying Elements

Sarvar Tursunbaev1, a), Azizakhon Turakhujaeva1, b), 
Sharofuddin Mardonakulov1, c), Shokhista Saidkhodjaeva1, d) and 
Mokhinur Rakhmonova1, e)

1Tashkent State Technical University, 2 University Street, 100095 Tashkent, Uzbekistan

a) Corresponding author: anvarovichsarvar908@gmail.com
b)Azizakhon.TN@gmail.com
c)Sharofiddin8611@mail.ru
d)saidxodjayevaSH@gmail.com
e)rahmonovamohinur40992@gmail.com

Abstract. Improving the properties of aluminum with the help of various modifying elements is one of the most common methods. This article analyzes the change in wear-resistant properties of aluminum alloys when adding different amounts of Ge and Si to the samples. The wear resistance of the cast samples was determined and a graph of its dependence on the amount of germanium in the composition was constructed. Based on the obtained results, an analysis of the change in wear resistance was carried out using mathematical modeling. The article shows that it is theoretically possible to determine the change in wear resistance without conducting additional experiments using mathematical modeling.

Keywords: Germanium, modifier, silicon, temperature, mathematical modeling, mechanical properties, aluminum

INTRODUCTION

Nowadays, non-ferrous alloys are the most widely used alloys after ferrous alloys in the production of machine parts [1, 2, 3, 4]. One of the common technologies is to improve the properties of the alloy by introducing modifying elements into it [5, 6, 7, 8]. In this research paper, germanium and silicon were included as modifying elements in the composition of aluminum.

Wear resistance, unlike the modulus of elasticity or thermal conductivity, is not an intrinsic property of the metal, although these and other properties can significantly affect wear resistance. Wear resistance can only occur as a result of friction of one surface against another, which can be solid, liquid or gaseous, and can occur both under load and in motion. Wear resistance occurs depending on the specific friction pair and the environmental conditions present on the interacting surfaces. [9]. The most common type of wear resistance is abrasive wear resistance. This type of wear resistance is found in mining, agricultural, drilling equipment and tools, working bodies and chassis of road construction and transport equipment, crushing, mixing machines and equipment in the processing industry, etc. [10].

MATERIALS AND METHODS

There are several methods for determining the wear resistance of metals, one of which is to determine the wear resistance of samples using a specific device. In this case, the samples are held in a device with a diamond disk under a certain force (5 or 10 N). The contact between the sample and the disk must be adjusted accordingly to ensure better friction with the surface. The wear rate (ω) of each sample volume was tested at a load of 10 N and calculated using the formula below.

ω=m2-m1ρFS                                                		       (1)

FIGURE 1. Structure drawing with diamond disc measuring stability: 1-control buttons, 2 - electrodvigatel, 3 - diamond disc, 4-sample, 5 - sample holder, 6 - balancing load, 7 - rollers, 8 - loads (5 or 10 N), 9-10-supports.

Where m1 and m2 are the masses of the samples before and after determining the wear resistance, ρ is the density of the main element (Al) in each sample, F (N) is the standard load force, S (m) is the wear path [11]. Figure 1 shows the design of the device used for the tests.

EXPERIMENTS AND RESULTS

Experiments

The samples were melted in foundry furnaces and poured into sand-clay molds at a temperature of 750 °C . The samples cast in sand-clay molds were cut and prepared on a lathe to determine wear resistance. The wear value of the prepared samples was determined using the device shown in Figure 2. The wear value was determined based on the mass of the original sample and the samples after testing. The sample testing process took 10 minutes. Each sample was kept on a diamond wheel in the device for 10 minutes. The results obtained during the tests are presented in Tables 1 and 2.

By putting the results obtained in Formula 1, the rate of wear resistance of the sample volume (ω) was determined. In this, the density of the aluminum element ρ= 2.71 gr/cm3 was equal, F=10 N was taken, and the edible path S = 54 m was taken. Based on the above results, the rate of wear resistance (ω) of the sample volume was calculated.

FIGURE 2. Wear resistance determining device

TABLE 1. Alloy results in aluminum-copper system

| № | The amount of Ge added to the sample (%) | The amount of Si added to the sample (%) | The initial mass of the sample is m1 (mg) | Sample mass after test m2 (mg) | Sample mass difference (mg) | The wear rate of the sample volume (ω) N·m |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 0 | 50.96 | 49.50 | 1.46 | 0.99 |
| 2 | 1 | 0 | 50.57 | 48.28 | 2.29 | 1.56 |
| 3 | 2 | 0 | 49.45 | 46.78 | 2.67 | 1.82 |
| 4 | 3 | 0 | 47.26 | 44.38 | 2.88 | 1.96 |
| 5 | 1 | 5 | 46.80 | 45.56 | 1.24 | 0.84 |
| 6 | 2 | 5 | 50.27 | 49.11 | 1.16 | 0.79 |
| 7 | 3 | 5 | 49.38 | 48.06 | 1.32 | 0.90 |

TABLE 2. Alloy results in the aluminum-magnesium system.

| № | The amount of Ge added to the sample (%) | The amount of Si added to the sample (%) | The initial mass of the sample is m1 (mg) | Sample mass after test m2 (mg) | Sample mass difference (mg) | The wear rate of the sample volume (ω) N·m |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 0 | 50.88 | 49.26 | 1.62 | 1.10 |
| 2 | 1 | 0 | 49.68 | 47.11 | 2.57 | 1.75 |
| 3 | 2 | 0 | 50.30 | 46.12 | 4.18 | 2.85 |
| 4 | 3 | 0 | 49.16 | 42.17 | 6.99 | 4.77 |
| 5 | 1 | 5 | 51.55 | 50.07 | 1.48 | 1.01 |
| 6 | 2 | 5 | 48.79 | 48.03 | 1.36 | 0.92 |
| 7 | 3 | 5 | 49.86 | 48.34 | 1.52 | 1.03 |

Based on the obtained results, a graph of the dependence of the mechanical properties  on the amount of modifying elements  was developed. Figures 3 and 4 show the results of the samples with GeO itself in the charges, and Figures 5 and 6 show the dependence of the samples with the inclusion of germanium and silicon in the composition of aluminum alloys.

FIGURE 3. Graph of the influence of the element Ge on the wear resistance of aluminum alloys [by weight loss (mg)]

FIGURE 4. Graph of the influence of the element Ge and Si on the wear resistance of aluminum alloys [by weight loss (mg)]

FIGURE 5. Graph of the influence of the element Ge on the wear properties of aluminum alloys [wear rate of the sample volume (ω)]

FIGURE 6. Graph of the influence of Ge and Si elements on the wear properties of aluminum alloys [wear rate of the sample volume (ω)]

MATHEMATICAL MODELING

For the mathematical modeling of the effect on the wear resistance, the graphs in figures 3,4,5,6 above were taken as a basis. These graphs are based on the dependence of indicators such as the lost mass of the sample and the rate at which the volume of the sample is eaten (ω) on the amount of germanium and germanium-silicon in the sample. Mathematical modeling was developed using the PTC MathCad Prime 10.0.1.0 program.

As a result of experiments, the initial form of the cubic polynomial function for 4 points has the following form:

P(x)=k=03yk∙Lk(x)                                                                                     (2)

here, Lk(x) - Lagrange basic polynomials, yk - the y-coordinates of points.

A mathematical expression is developed in the following order:

1. At the initial stage of calculation, we can calculate the Lk(x).

2. In the next step, we write down P(x), which we express as follows: P(x)=y0⋅L0(x)+y1⋅L1(x)+y2⋅L2(x)+y3⋅L3(x)

3. Then, we simplify the resulting expression.

4. Using the results obtained through mathematical modeling, the dependence function of the wear resistance of the alloy under study on the percentage of germanium and germanium-silicon added to its composition was derived as follows:

Dependence functions found on lost mass

For Al-Cu alloy (Ge): P1(x)= 0.0467x3 - 0.365x2 + 1.1483x+1.46;

For Al-Mg alloy (Ge): P2(x)=0.09x3 +0.06x2+ 0.8x+1.62;

For Al-Cu alloy (Ge+Si): P3(x) = 0.0167x3+ 0.02x2- 0.2567x +1.46;

For Al-Mg alloy (Ge+Si): P4(x)= 0.0433x3 -0.12x2- 0.0633x+1.62;

Dependence functions for volumetric wear rate were found

For Al-Cu alloy (Ge): P1(x)= 0.0317x3-0.25x2+0.7883x+0.99;

For Al-Mg alloy (Ge): P2(x)= 0.0617x3+0.04x2+0.5483x +1.1;

For Al-Cu alloy (Ge+Si): P3(x) = 0.01x3+0.02x2+0.18x+0.99;

For Al-Mg alloy (Ge+Si): P4(x)= 0.0333x3 -0.1x2- 0.0233x+1.1;

CONCLUSION

1. As can be seen from the results presented in the graphs and tables, the inclusion of germanium alone reduced the wear properties of the aluminum alloys, but with a combination of germanium and silicon, the wear resistance increased. At the same time, the wear properties of the aluminum alloys in a combination of germanium and silicon increased by an average of 9% - 16%.

2. Polygonal functions obtained using mathematical modeling make it possible to obtain changes in wear resistance without conducting practical experiments.

3. When comparing the results of the research based on the modifying elements  (Ge and Si) included in the alloy to improve the wear properties of aluminum alloys, it was found that the difference between the results of the comparative graph developed through mathematical modeling and the results was 5-6%.

Based on the above conclusions, it is recommended to add germanium to aluminum alloys in an amount of 1% to 2% of the alloy mass, and silicon - up to 5%. It is recommended to study the properties of aluminum alloys in further research taking these factors into account.

REFERENCES

- S. S. Dash and D. Chen, "A review on processing–microstructure–property relationships of Al-Si alloys: Recent advances in deformation behavior," Metals 13, 30609 (2023). https://doi.org/10.3390/met13030609.

- A. Ivannikov, V. Fedorov, A. Abramov, M. Penyaz, D. Bachurina, T. Uhlig, A. Suchkov, G. Wagner, P. Morokhov, and O. Sevryukov, "Development of rapidly-quenched Al-Ge-Si filler alloys for the joining of stainless steel AiSi 304 and aluminum alloy AA6082," Metals 11, 1926 (2021). https://doi.org/10.3390/met11121926.

- T. Sarvar, T. Nodir, U. Mardonov, B. Saydumarov, D. Kulmuradov, and M. Boltaeva, "Effects of germanium (Ge) on hardness and microstructure of Al-Mg, Al-Cu, Al-Mn system alloys," Int. J. Mechatronics Appl. Mech. 16, 179 (2024). https://doi.org/10.17683/ijomam/issue16.21.

- W. S. Miller, L. Zhuang, J. Bottema, A. J. Wittebrood, P. de Smet, A. Haszler, and A. Vieregge, "Recent development in aluminum alloys for the automotive industry," Mater. Sci. Eng. A 280, 37 (2000). https://doi.org/10.1016/S0921-5093(99)00653-X.

- C. Contatori, N. I. Domingues, R. L. Barreto, N. B. de Lima, J. Vatavuk, A. A. C. Borges, G. F. C. Almeida, and A. A. Couto, "Effect of Mg and Cu on microstructure, hardness and wear on functionally graded Al–19Si alloy prepared by centrifugal casting," J. Mater. Res. Technol. 9, 13217 (2020). https://doi.org/10.1016/j.jmrt.2020.11.050.

- D. M. Berdiev, A. A. Yusupov, R. K. Toshmatov, and A. K. Abdullaev, "Increasing die durability in cold stamping by quenching with intermediate tempering," Russ. Eng. Res. 42, 1011 (2022). https://doi.org/10.3103/S1068798X22100057.

- S. Tursunbaev, N. Turakhodjaev, X. Wang, H.-L. Shi, X. Li, K. Karimov, U. Mardonov, S. Mardonakulov, N. Tadjiev, A. Turakhodjaeva, A. Turakhodjaeva, and M. Rakhmonova, "Increasing the corrosion resistance of aluminum alloys (Al-Cu, Al-Mg, Al-Mn) by adding microalloying elements in its composition," Int. J. Mechatronics Appl. Mech. 19, 133 (2025). https://doi.org/10.17683/ijomam/issue19.15.

- V. Hutsaylyuk, M. Student, K. Zadorozhna, O. Student, H. Veselivska, V. Gvosdetskii, and H. Pokhmurska, "Improvement of wear resistance of aluminum alloy by HVOF method," J. Mater. Res. Technol. 9, 16367 (2020).

- M. K. Abbass, M. M. Radhi, and R. S. A. Adnan, "The effect of germanium addition on mechanical properties and microstructure of Cu-Al-Ni shape memory alloy," Mater. Today Proc. 4, 224 (2017). https://doi.org/10.1016/j.matpr.2017.01.016.

- X. Qi, J. Li, Y. He, Y. Liu, R. Liu, and R. Song, "Study on the wear resistance and corrosion behaviour of self-sealed MAO/ZrO₂ coatings prepared on 7075 aluminum alloy," J. Alloys Compd. 969, 172436 (2023). https://doi.org/10.1016/j.jallcom.2023.172436.

- S. Tursunbaev, N. Turakhodjaev, F. Odilov, S. Mardanokulov, and R. Zokirov, "Change in wear resistance of alloy when alloying aluminium alloy with germanium oxide," E3S Web Conf. 401, 05001 (2023). https://doi.org/10.1051/e3sconf/202340105001.