Structure of the Mathematical Support System for Automated Design of Assembly Tooling for Aircraft Airframe Structure Assembly

Sagdiyev Tulkun1,𝑎) and Mamirov Shukhrat1,𝑏)

1Tashkent State Transport University, Temiryolchilar Street 1, Tashkent 100167, Uzbekistan

𝑎) Corresponding Email: sagdievtulkun@gmail.com

𝑏) mamirovsuhrat@gmail.com

Abstract.  This paper presents the mathematical, informational, and software support of an automated design (AD) system for assembly tooling used in integrating aircraft airframe structural units. The approach formalizes relations between design– technological properties of production objects and elements of the assembly technological system (bases, base-fixing elements, clamping–fixing elements, and frame components) using a hierarchy of set-theoretic, logical, and quantitative models. These models drive procedure libraries and databases to synthesize feasible fixture concepts, dimension nonstandard elements, and verify rigidity and accuracy. The system has been implemented as a software–methodological complex (SMC “CAD AF”) and piloted at the Joint Stock Company “Tashkent Mechanical Plant”, demonstrating adequate decision-making and notable reductions in manual iteration during technological preparation.

Keywords: CAD, assembly fixture, airframe, mathematical modeling, automated design, rigidity analysis, tolerance analysis

# INTRODUCTION

The design and production of modern aircraft require high-precision assembly processes supported by advanced engineering methodologies and specialized tooling [1, 9]. Within the aircraft manufacturing workflow, the assembly fixture plays a critical role in ensuring the accurate spatial positioning, reliable fixation, and dimensional stability of structural units such as panels, compartments, sections, and aggregates. A well-designed fixture not only maintains geometric accuracy throughout the assembly process but also allows compensation for manufacturing deviations in thin-walled or flexible parts, thereby improving overall product quality.

The growing complexity of airframe structures and the demand for reduced production lead times have driven the adoption of Computer-Aided Design (CAD) and Computer-Aided Engineering (CAE) systems for fixture development [2]. Object-oriented analysis and design methodologies provide a robust framework for modeling both the product and the associated technological systems, enabling systematic reuse of design logic and procedures across different fixture configurations. By integrating standard mathematical models into CAD environments, as proposed in the RD 50-464-84 guidelines [8], diverse design processes can be transformed into a set of unified operations that are both automatable and verifiable.

Recent research has addressed the formalization of design–technological relationships between airframe assemblies and fixture components [3, 4]. These studies have proposed hierarchical modeling frameworks that operate at logical, structural, and quantitative levels, facilitating automated selection of assembly methods, referencing schemes, and fixture element dimensions. Earlier work has explored the development of mathematical and software support for CAD systems in aircraft manufacturing, including dedicated procedure libraries and network models for component selection [5]. Furthermore, specialized software–methodological complexes such as CAD AT SMC have been officially registered and deployed in industry to support the design of assembly tooling for aircraft panels [6].

In addition to fixture design for panels, the classification of airframe structural elements and panels for tooling design has been the focus of prior object-oriented approaches [7], providing a basis for modular database structures in automated design systems. The theoretical foundation for these developments is also grounded in established aerospace production technologies [1, 9] and formal engineering methodologies for fixture modeling [8]. International experience in automated fixture design is reflected in studies such as Shokirov et al. [10], which demonstrate the applicability of CAD-based automation for other aerospace subsystems, and in programming-related optimization approaches [11] that can be leveraged for computational efficiency.

Despite these advancements, there remains a need for integrated systems that combine formalized mathematical models, comprehensive component databases, and modular software architectures to fully automate the design of assembly fixtures for complex airframe assemblies. The present work builds on the established body of knowledge [1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11] to develop and implement the Software–Methodological Complex (SMC) “CAD AF,” an automated design system tested under industrial conditions at the Joint Stock Company “Tashkent Mechanical Plant.” The system integrates hierarchical mathematical models with standardized databases and procedural modules, enabling accurate, efficient, and repeatable design of aircraft assembly fixtures.

# PROBLEM STATEMENT AND REQUIREMENTS

From an engineering perspective, the problem involves developing an automated design (AD) methodology and toolset capable of selecting, sizing, and validating assembly fixture (AF) components—including base-fixing elements (BFEs), clamping-fixing elements (CFEs), and frame structures—based on the design–technological characteristics of the assembly unit. This requires the formalization of relationships between product geometry, tolerances, and assembly method selection to ensure repeatable accuracy and strict compliance with technical specifications. Furthermore, the methodology must integrate design rules, catalogs of standardized components, and computational models into a unified decision-making framework to minimize reliance on manual iteration. To meet these objectives, several key requirements must be fulfilled: the fixture must guarantee assembly accuracy so that panels, compartments, sections, and aggregates conform to positional and dimensional tolerances; maintain dimensional stability throughout its service life despite operational loads or thermal variations; and provide sufficient rigidity to prevent deformation of the assembly unit during installation, alignment, and joining. Additionally, the design should prioritize the use of standardized components to reduce costs, simplify procurement, and ensure interchangeability; optimize overall dimensions to balance accessibility for assembly operations with spatial constraints; allow unobstructed access to assembly zones to facilitate installation, fastening, and inspection; and support error compensation principles to correct dimensional deviations in parts and subassemblies, particularly in thin-walled or flexible components.

# GENERAL AD WORKFLOW

The automated design (AD) process for aircraft assembly fixtures follows a sequence of interlinked decision-making and computational stages that mirror the workflow of process and tooling design engineers, as illustrated in Fig. 1. It begins with the formation and input of initial data into the system, including the assembly fixture design specification, technical requirements for the assembly unit, geometric and functional parameters, and any constraints related to interchangeability methods or manufacturing conditions. Once the initial data is provided, the system determines the primary assembly method by evaluating the applicability of available techniques using procedural knowledge and mathematical models, with selection criteria based on technical constraints, required assembly accuracy, and economic efficiency. It then defines the complete assembly base composition, covering both primary and auxiliary references, and identifies all base-fixing elements (BFEs) required for the chosen method. The number and type of BFEs and clamping-fixing elements (CFEs) are calculated using matrix and library models (ML1–ML3, PL1), with optimization criteria applied to minimize cost, mass, or complexity. Standardized BFEs and CFEs are retrieved directly from catalogs, while non-standard components are designed parametrically (ML4, PL2) to generate working drawings and technical specifications. The frame structure of the fixture is established using MN2, taking into account load-bearing capacity and rigidity, after which the system selects frame elements from the database via MN3 and verifies their stiffness and strength through analytical calculations. If requirements are not met, the system modifies the frame scheme or proposes an alternative layout. The design then proceeds to the specification of installation elements, including their composition, dimensions, and technical details, before generating detailed drawings, specifications, and a complete bill of materials. All project data are archived in the Project Database (PDB) for future reuse or modification. At any stage, if technical feasibility criteria—such as the availability of suitable standard elements or compliance with rigidity requirements—are not met, the system issues prompts for design modification or suggests adjustments to the technical specifications, ensuring that the final fixture design is manufacturable, compliant with requirements, and optimized for production efficiency.

FIGURE 1. Generalized block diagram of the algorithm and stages of automated design (AD) for assembly fixture structures

# MATHEMATICAL MODEL STRUCTURE

The suggested automated design (AD) system of the aircraft assembly fixtures (AFs) is based on hierarchical models of classification of the mathematical models reflecting the logical, structural, and quantitative connections between the object of assembly and the elements of the fitting. Such a structure can be subdivided into two major groups, conditionally constant and variable models, as Fig. 2 demonstrates.

## Conditionally Constant Models

These models have information that is constant over time in the performance of the models. They form the central structure of the system and comprise of:

- ML1 – Instructor, design matrix to find out base positioning schemes of airframe units and assemblies.

- ML2 – Matrix of interconnections between assemblies and individual base elements.

- ML3 – Matrix for selecting calculation formulas to determine the number of base-fixing elements (BFEs) and clamping-fixing elements (CFEs).

- ML4 – Matrix for selecting design procedures for BFEs.

- PL1 – Library of formulas and procedures for calculating BFE and CFE quantities.

- PL2 – Library of procedures for designing BFEs.

Other constant resources include tolerance tables for dimensional transfers, structural diagrams of dimensional chains for accuracy analysis, technical–economic indicator tables for different assembly methods, and stiffness coefficient tables for analytical beam models.

## Variable Models

Variable models adapt to changes in product range, available standardized elements, and design requirements:

- MN1 – Network model for selecting standard sizes of CFEs from catalogs.

- MN2 – Matrix for determining the structural scheme of the AF frame.

- MN3 – Matrix for determining the standard sizes of frame components.

## Logical Representation of Object Properties

The initial state of the model is defined by the initial logical vector:

v0 =Ai,FA,Fiai                                                                  (1)

where 𝐴𝑖 is the identifier of the assembly unit, 𝐹(𝐴) is its functional class, and 𝐹𝑖 (𝑎𝑖) is the set of technological and geometric attributes of each element.

From this, the system determines the set of feasible assembly bases:

B(A)=k=1nBi(ak)                                                                             (2)

where ∧ denotes a logical AND over the compatibility of all required base elements.

## Compatibility and Procedure Selection

The compatibility of base elements with a given referencing scheme is expressed by:

π1=k=1nπ1k,    N(π1i)=mi(π4i),                                                                       (3)

where π1k is the compatibility predicate for the 𝑘-th element, 𝑚𝑖 (·) maps predicates to procedure identifiers, and 𝜋4𝑖

is the procedure index.

## Quantitative Determination of CFE/BFE Counts

Once a referencing scheme is fixed, the number of clamping-fixing elements of type 𝑐 is determined by:

NC=Freqfcmaxηc ,                                                                          (4)

where:

- Freq — resultant clamping force required.

- fcmax— maximum clamping force for subtype 𝑐 from the catalog.

- ηc — efficiency/safety factor (0 < ηc ≤ 1).

## Rigidity Verification of Frame Elements

The frame design is checked using classical beam theory. For a simply supported beam of length L under uniform load w:

δmax= wL48El ,                                                                          (5)

σmax=MmaxCl,   Mmax= wL48                                                     (6)

where 𝐸 is Young’s modulus, 𝐼 is the second moment of area, and 𝑐 is the distance from the neutral axis to the extreme fiber. The section is acceptable if:

𝛿max ≤ 𝛿allow,  𝜎max ≤ 𝜎allow.                                                        (7)

## Tolerance Accumulation for Accuracy Verification

Dimensional accuracy is verified using either the Root-Sum-Square (RSS) or worst-case method, depending on the interchangeability policy:

TRSS=j=1mtj2,          TWC=j=1mtj,  	                                           (8)

where 𝑡 𝑗 are the individual tolerance contributors from each element in the dimensional chain.

## Model Integration

These logical, set-theoretic, and quantitative models are integrated via the modular architecture shown in Fig. 2, ensuring consistent data flow between procedure libraries, databases, and calculation routines.

# RESULTS AND DISCUSSION

The Software–Methodological Complex (SMC) “CAD AF” represents the practical implementation of the mathematical, informational, and procedural framework described in the preceding sections. It provides a computer-aided environment that manages the complete lifecycle of aircraft assembly fixture design, beginning with the specification of requirements

FIGURE 2. Structure of mathematical models: ML1–ML4 (logical/matrix layers), PL1–PL2 (procedure libraries), and MN1–MN3 (network/matrix selection for sizes and frame schemes)

and ending with the generation of final engineering documentation. By embedding formalized relationships between the design–technological properties of the assembly object and the fixture’s structural elements, the system ensures that every stage of the design process is guided by explicit engineering rules rather than subjective decisions.

The system is built on a set of interconnected databases containing both standardized component information and specialized engineering reference data. The frame element database stores geometric and stiffness parameters for a wide variety of beam and frame profiles, including section dimensions, second moments of area, section moduli, linear mass, and material properties. The locating element database records standardized supports, saddles, fixators, and their terminations, each linked to catalog entries for compatible connection hardware. In addition, databases of assembly unit elements provide data on standard cross-sections of stringers, spars, frames, ribs, and reinforcements, together with their allowable materials. Tolerance and accuracy tables define permissible dimensional deviations and provide dimensional chain diagrams for verifying compliance of fixture designs with specified accuracy requirements.

The operational logic of SMC “CAD AF” is implemented in plenty of meter modules which are structured functionally and connect with shared databases and the libraries of mathematical models. This starts in the Input Data Module where the designer documents the purpose of the fixture, the parameters of the assembly unit and the constraints that are placed by technical specifications or interchangeability requirements. There is also a graphical display of the input geometry allowing inspection to be performed on the data before continuing further.

Subsequently, the Assembly Method Selection Module analyses the possible methods of assembly against compatibility, analytical accuracy (computed by root-side-square or worst-case tolerance summation) and cost-effectiveness. The methods that fulfill all the accuracy and manufacturing ability requirements are kept. When a technique has been chosen, the Referencing Scheme Module decides on the assembly base composition and also the needed base-fixing elements (BFEs) and clamping-fixing elements (CFEs) based on and procedural matrices and formula libraries. The standardized parts are extracted based on catalogs whereas the non-standard designs are designed through the BFE and CFE Design Modules parametric modeling techniques in order to establish geometries, dimensions, and technical specifications.

The Frame Structure and Rigidity Analysis Module can then choose a layout of a frame in the MN2 matrix, calculate the rigidity and strength of the frame via beam theory and store an appropriate section in the frame structure element database. In the case that deflection or stresses are beyond the permitted limits, there is a change in layout or even other profiles are recommended so as to ensure stability under working loads. The Foundation Strength Calculation Module checks out the stability of the supporting structure, whereas the Installation Element Module calculates how many installation components are required. The Output and Documentation Module assembles engineering drawings, bills of materials, and specifications and stores the finished project in Project Database (PDB) to be reused.

Each of the modules is supplied by the mathematical infrastructure presented above: logical matrices ML1 to ML4 specify interactions between objects and components, procedural libraries PL1 to PL2 represent methods of calculation, and network / matrix models MN1 to MN3 assist in the selection of conventional components as well as in determining their sizes. The integration ensures conformity among engineering regulations, computing process, and sources of resources.

Pilot testing of the Joint Stock Company Tashkent Mechanical Plant demonstrated that the system renders correct and manufacturing friendly fixation designs of panels, fuselage components and large-size aggregate. The SMC decreased the necessity of manual calculations, shortened the design-cycle, and cut back trial-and-error iterations. The quality of communication between the design and manufacturing teams was enhanced by consistency in data format and legibility of records of the calculations, resulting in design verification and approval being processed much more quickly.

FIGURE 3. Structure and informational interconnections of SMC “CAD AF” modules, including databases, calculation libraries, design/verification modules, and input/output subsystems

SMC “CAD AF” is modular and hierarchical in architecture and thus can conform to the changing manufacturing practices. The workflow can accommodate new assembly methods and component catalogs as well as innovative methods to analyse. Centralized engineering information eradicates duplicacy, enforces the standardization, and enhances traceability process. Although the procedure was developed in the aerospace industry, the technique could also be applied to the shipbuilding, railway and automotive manufacturing industries where large and complex assemblies need to be designed with precise rigid and cost effective fixtures to ensure the accuracy of the assemblies.

# CONCLUSION

The study in this paper has led to creation of a thorough-going automated design environment, the Software(mological) Complex (SMC) “CAD AF” explicitly devoted to an assembly-fixture development within a structure of an aircraft airframe. A system of mathematical support in the hierarchy is at its core and combines sets of the most efficient engineering models (set-theoretic, logical and quantitative) and extensive databases of normative components, tolerance information, and tested engineering processes. The inbuilt nature of these models into the actual working process means that the system makes sure that each design decision can be tracked and recreated and is in line with the predefined manufacturing regulations.

At the Joint Stock Company, as “Tashkent Mechanical Plant” SMC proved viable practicality by means of pilot deployment. The system was able to consistently generate fixture designs, which could meet the requirements of extreme accuracy and rigidity needs, shorten the overall design cycle time and enhance the relationship between design and the manufacturing departments. Its programmatic automation also reduced the use of manual calculations and subjectivity in decision making and reduced the likelihood of error as well as increased data reproducibility.

Modularity and extensibility of the architecture is one of the main advantages of the SMC “CAD AF” product. This enable new assembly processes, component databases and calculation routines to be adapted without upsetting the current work flows. The validated engineering data is focused and provides accuracy to all modules which supports the standardization activity in the production. Also, the method is not restricted to use in aerospace, as the methodology is flexible enough to adapt to the other industries in which large scale and precision critical assembly fixtures are needed, namely, ship building, rail transportation or auto manufacturing industries.

In general, the work has proved that an automated design system based on models, properly organized, can highly improve the efficiency of the entire task, the quality of acquired results, and the scale of the entire process of designing the fixtures. The SMC “CAD AF values therefore give not only a convenient solution to the existing manufacturing requirements but also a stable platform where new technologies such as optimization through AI, digital twins and complete integration of a PLM, CAD and CAM solution can be added in future.

# REFERENCES

- V. I. Ershov, V. V. Pavlov, M. F. Kashirin, and V. S. Khukhorev, Aircraft Assembly Technology. Moscow: Mashinostroenie, 1986, 456 pp., illustrations.

- G. Booch, Object-Oriented Analysis and Design with Applications in C++. Addison-Wesley Longman, 1995.

- T. A. Sagdiyev and F. I. Umurzakov, “Features of Selecting Positioning Methods for Assembling Aircraft Structural Units in Assembly Tooling,” Scientific and Informational Journal “Heavenly Shields”, no. 10(2), pp. 206–211, 2024.

- T. A. Sagdiev and D. K. Kambarov, “Object-Oriented Approach to the Analysis and Classification of Aircraft Frame Structures,” Theory and Practice of Modern Science, no. 6(84), June 2022, doi:10.46566/2412-9682_2022_84_220.

- T. A. Sagdiyev and V. V. Pavlov, “Some Features of Developing Mathematical and Software Support for CAD Systems for Designing Assembly Process Technologies,” Information Technologies in Design and Production, Scientific and Technical Journal of FGUP “VIMI”, Moscow, no. 2, pp. 35–40, 2008.

- T. A. Sagdiyev and A. A. Bubnov, Software–Methodical Complex “Computer-Aided Design System for Assembly Tooling of Aircraft Panels” (CAD AT SMC), Official registration certificate for computer software No. DGU 01220, State Patent Office of the Republic of Uzbekistan, Tashkent, Jan. 30, 2007.

- T. A. Sagdiyev and A. A. Bubnov, “Object-Oriented Approach to the Analysis and Classification of Panels for Designing Assembly Tooling,” Issues of Cybernetics, Institute of Informatics, Academy of Sciences of Uzbekistan, issue 168, pp. 135–139, 2003.

- Standard Mathematical Models for Design Objects in Mechanical Engineering: Methodological Guidelines RD 50-464-84. Moscow: Standards Publishing, 1985, 200 pp.

- V. G. Kononenko, P. N. Kucher, Yu. A. Boborykin, et al., Aircraft Production Technology. Course Design. Kyiv: Higher School, 1974, 224 pp.

- R. Shokirov, N. Abdujabarov, J. Takhirov, and S. Bobomurodov, “Automated Design of the Appearance of an Unmanned Aerial Vehicle,” AIP Conference Proceedings, vol. 2432, 030088, 2022, doi:10.1063/5.0090313.

- S. Abdukarimov and R. Shokirov, “Team competitions on programming format ACM ICPC,” International Journal of Scientific and Technology Research, vol. 9, no. 4, pp. 1932–1935, 2020.