Moving Load Action on the Ribble Cylindrical Shell with a Viscous-Elastic Filling

I.I. Safarov1, a), Z.I. Boltayev2, b), G.T. Mirzoyeva2, c), B.Sh. Zaripov2, d) and 
A.A. Jumayeva3, e)

1Tashkent Institute of Chemical Technology, 32 Navoi Street, Tashkent 100011, Uzbekistan
2Bukhara State Technical University, 15 Murtazaev Street, Bukhara 200100, Uzbekistan
3Samarkand State University of Architecture and Civil Engineering, Samarkand, Uzbekistan

a) Corresponding author: Safarov45@mail.ru
b) boltayev-z@mail.ru
c) gulimirzoyeva1992@gmail.com
d) begzodzaripov331@gmail.com 
e) jumayevaanora4@gmail.com

Abstract: Cylindrical shells are used in aviation technology as well as in rocket engineering. This paper aims at examining the behavior of an infinitely long cylindrical shell under moving loads which is reinforced by longitudinal stiffening ribs on the exterior and having a viscoelastic inertial fill inside. The mass in the motion is transferred to the shell only with the help of its edges having no loads outside the edges. The segmented arrangement of the ribs in the examined structure is interpreted by formulating equations that model their behavior like beams, incorporating continuity and connection constraints between elements..Variation of the number and stiffness of the body ribs on the distribution character of shell movement and contact pressure at the interface with a viscoelastic filler has been illustrated. Classical equations of the Kirchhoff-Love hypothesis control the movement of the shell and dynamic equations of the theory of viscoelasticity are used to include the aggregate. The Kirchhoff-Love hypothesis is the basis of classical equations describing the shell motion. Equations of the theory of viscoelasticity are used as concerns the group in the whole. In the situation of two opposite facing edges, a small effect of the material added has been found in regards to the extent of bending of the shell in locations far away to the areas where the force is acting upon, especially in the event of peeling contact.

Keywords: thin shell, filler material, stiffness, rigidity, edge surface, contact conditions, interaction conditions, contact stress

INTRODUCTION

Cylindrical shell and plate-shaped structural components are extensively used in engineering and building applications.. In recent times, the speed of surface and subsurface transport, aircraft, and waterborne vehicles, whose structural components consist of cylindrical shells, has significantly increased [1, 2, 3, 4]. In this regard, new dynamic problems arose, and requirements for refining the mathematical model - the cylindrical shell plus the moving body - increased. It clarifies not only the shell model (specified equations of motion and boundary conditions, geometric and physical nonlinearity, non-uniformity, etc.), but also the models of contacting objects (contact conditions, inertia of moving bodies, clarification of the equations of motion of the medium, nonlinearity, non-uniformity, dissipativity, thermal phenomena, etc.) [5, 6, 7, 8]. By applying the Fourier transform and decomposing all given quantities into Fourier series, we introduce potential functions in accordance with formulas (3). Examples of such problems include the design of underground and underwater reservoirs and pipeline networks, subway tunnel linings and capital mining workings, cleaning (blowing) the cavity of main industrial gas and oil pipelines before their commissioning or during operation [9, 10] (the movement speed of cleaning devices reaches 20-25 m/sec).

The linear dynamics of smooth cylindrical shells under the action of various moving loads has been studied in several works. The axially symmetrical oscillations Of a hollow cylindrical body of finite length influenced by the operation of uniformly distributed pressure wave moving with a constant velocity are considered. The solution is sought in trigonometric rads using the cylinder's natural vibration functions. It was found that consideration of transverse displacement and rotational inertia clarifies the magnitude of the bending moment by %. Up to 10 members are held in rads. These problems are studied in more detail in the article, analytical formulas for calculating (in several approximations) critical velocities of massive circular motion, as well as systems of circular loads, using the method of O.A. Goroshko, are given.

METHODS

Definition of the problem and approaches to its solution.

Let us take an infinite cylindrical shell of infinite length, the cross-section ribbed with a circular shape and subjected to the moving load as well as a viscoelastic medium. The radius of the outermost surface of the aggregate here denoted as R1 and the radius of the innermost surface is R0. The material is considered to be linear viscoelastic, uniform, and the same in all directions. It is defined in a known cylindrical system with coordinates r, θ, z, and also in a set of known Cartesian coordinates x, y, z, where one of the axes, z, runs along the center of the cavity.

FIGURE 1: Calculation scheme of ribbed shells

The axially symmetric behavior of the shell is governed by the corresponding motion equations [20, 21, 22]:

(1)

Here,  concerning the inner layer  - for external layer,  it represents the displacement vector of the points located on the mid-surface of the supporting layer.

In the case of Kirchhoff-Love shells, the vector representing positional change is three-dimensional. To satisfy the According to the Timoshenko-type hypothesis, the number of displacement components (i.e., displacements along the coordinate axes) is determined  the vector is five-dimensional

The motion of the stiffness edges obeys the equations of the beam theory.

(2)

where  force exerted by the shell per unit length k -th rib:  The normal force that acts on the k -th beam will be multiplication of the magnitude of normal force by l where l is the number of reinforcing rein.

Neglecting volumetric forces, the linear motion of the mechanical system is represented by the following vector equation:

(3)

where; ;  are relaxation nuclei;  is instantaneous modulus of elasticity;  displacement vector;  density; k  is the layer number,  is the Poisson's ratio, which we consider to be a non-relaxing quantity [6].

We take the integral terms in (1) as small. Then the integral function has the form, where  slowly changing function,  actual frequency. Next, we apply the freezing procedure. The following notation for displacements is adopted for convenience:

for Kirchhoff-Love shells,

for Timoshenko shells.

In the Timoshenko shell theory, as well as the three displacements axial, circumferential, and transverse displacement, the rotational angles of the normal to the mid-surface in the axial direction are also considered. Conversely, in the case of shells that satisfy the Kirchhoff-Love assumptions, the following form of the differential operator may be established:

(4)

Here, ,  represent the thickness and the curvature radius of the mid-surface of the cylindrical shell,  the relaxation core;  refers to the time-independent (instantaneous) modulus of elasticity.

Given a constant velocity of load motion and a steady-state behavior, the analysis is performed in a co-moving coordinate system ,  where Н is the characteristic size of the structure (length),  is the speed of load movement. Applying the Fourier transform by  and applying Fourier series expansion to all the given quantities by , Potential functions that satisfy the wave equations are introduced according to formulas (3). Based on the solutions of these equations, when the load velocity is lower than the propagation speed of displacement waves in the filler, the displacement Fourier coefficients are expressed as follows:

(5)

where

By means of formulas (5), we obtain the harmonic transform of the stress field within the filler. By  The transformed boundary conditions are satisfied by writing the functions in the terms of the Fourier coefficients of the radial displacement of the shell.

(6)

In this case, the functions  are calculated using formula (6), and the elements of the determinants  are found if the elements  of these formulas include , and the elements are calculated  using the formulas:

Using the functions found through equation (6), we added them to the expressions, and then applied the resulting  outcomes to the specified conditions  .The correlation between the filler's reactive force and the shell's radial displacements is established, with the function  being computed according to the given formulas.

(7)

where  is algebraic additions of the determinant elements.

RESULTS AND ANALYSIS

Simulations were run for a range of dimensionless parameter values as follows:

FIGURA 2. Change in shell deflection along a circle for a different number of stiffness edges

It has been determined that as the number of stiffening ribs grows, the structural stiffness becomes greater., which leads to a rapid reduction in the effect of discrete reinforcement and loading. When you have two ribs positioned opposite each other, the filler in sliding contact barely affects how deflections spread out in areas far from where the load is applied, which results in a clear difference in the shape of the curves. The analysis of the results shows that contact stresses rapidly reverse sign as the distance from the loading point increases, which, in the case of a one-sided connection, causes the shell to separate from the filler.Moreover, the arrangement and number of stiffening ribs have a direct influence on the distribution of deformations. In particular, the amplitude of deflections decreases sharply at distances far from the loading center, which ensures the structural stability of the system against localized loads. When you look at this behavior together with the viscoelastic properties of the filler material, you can see that the deformation is restricted to a certain area, which means the filler isn’t fully active along its whole length.In addition, under the action of moving loads, transient resonance phenomena may occur in the shell-filler system. It was found that adding more stiffening ribs and positioning them wisely helps reduce resonance effects and improves how stable the structure behaves under dynamic loads. The research also shows that how the shell and filler interact with each other—whether they are bonded together or can slide—has a strong effect on how the contact forces are distributed. When sliding is allowed, small gaps or separation zones may appear in some areas, which reduces how well stress is passed from the shell to the filler. But when the filler is bonded to the shell, they move more uniformly together, which leads to smoother deformation and avoids the appearance of negative contact pressures. This shows that if we want to control how the structure reacts to different types of loads, we need to pay close attention not only to how many stiffening ribs are used and where they are placed, but also to how the shell and the filler are connected.

FUTURE SCOPE

Future Scope Studying how moving loads affect ribbed cylindrical shells with a viscoelastic filling offers many useful directions for future research. As engineering systems grow more advanced and face different kinds of moving or repeated forces — like vehicles on pipelines, high-speed trains in tunnels, or fluids inside transport systems — knowing exactly how these structures respond becomes even more important. Future work may target more realistic models of the filler material by adding properties which vary with temperatures, or have non-linear viscoelastic behavior. This would assist in getting nearer to the right outcomes that answer to the actual operating conditions. Moreover, the effect of the various rib shapes, sizes, and positions on the response of the shell structure to its moving loads could also be examined and lead to enhancing the shell structure. The other good pointer is enhancing simulation method to be merged with real life test information. That would allow the predictions to be more convincing and engineers to have more tools to design safely (and smartly). Future work may also look at the ageing of these structure, how they cope with damage, repetition or load variation over a long period of time, particularly in longer term civil, aerospace, or marine, engineering projects. On the whole, this study can serve as a good basis. It can be used with the contribution of the various disciplines that provide more effective, stable and resistant structures that could be applicable in actual environment.

REFERENCE

- V. I. Pozhuev, “The action of a moving load on a cylindrical shell in an elastic medium,” Constr. Mech. Comput. Struct. 1, 44–48 (1978). https://doi.org/10.1007/BF00884609.

- V. P. Maiboroda, I. E. Troyanovskii, I. I. Safarov, G. M. Vazagashvili, and I. V. Katalymova, “Wave attenuation in an elastic medium,” J. Sov. Math. 60, 1379–1382 (1992). https://doi.org/10.1007/BF01679642.

- S. A. Bochkarev, S. V. Lekomtsev, and V. P. Matveenko, “Natural vibrations and stability of elliptical cylindrical shells containing fluid,” Int. J. Struct. Stab. Dyn. 16, 1550076 (2016).

- M. Amabili, “Free vibration of partially filled, horizontal cylindrical shells,” J. Sound Vib. 191, 757–780 (1996). https://doi.org/10.1006/jsvi.1996.0154.

- V. P. Maiboroda, I. I. Safarov, and I. E. Troyanovskii, “Free and forced oscillations of a system of rigid bodies on inhomogeneous viscoelastic snubbers,” Sov. Mach. Sci. (Engl. Transl.) 3, 25–31 (1983).

- I. I. Safarov and M. Kh. Teshayev, “Unsteady motions of spherical shells in a viscoelastic medium,” Bull. Tomsk State Univ. Math. Mech. (June 2023).

- I. I. Safarov and M. Kh. Teshayev, “Nonlinear oscillations in the dynamics of multilayer viscoelastic shells,” News High. Educ. Inst. Appl. Nonlinear Dyn. 31, 63–73 (2023).

- I. I. Safarov, M. Kh. Teshayev, A. M. Marasulov, and B. Z. Nuriddinov, “Propagation of own non-axisymmetric waves in viscoelastic three-layer cylindrical shells,” Eng. J. 25, 97–107 (2021). https://doi.org/10.4186/ej.2021.25.7.97.

- M. Kh. Teshaev, I. I. Safarov, N. U. Kuldashov, M. R. Ishmamatov, and T. R. Ruziev, “On the distribution of free waves on the surface of a viscoelastic cylindrical cavity,” J. Vib. Eng. Technol. 8, 579–585 (2020).

- M. Kh. Teshaev, I. I. Safarov, and M. Mirsaidov, “Oscillations of multilayer viscoelastic composite toroidal pipes,” J. Serbian Soc. Comput. Mech. 13, 105–116 (2020). https://doi.org/10.24874/jsscm.2019.13.02.02.08.