Adaptive Tuning of Non-Line Search Gradient Algorithm for Ultra-Wide Band Microwave

Tareq F A Zanoon1, a)

1 Department of Computer Systems Engineering,  Arab American University, Jenin, 240, Palestine.

a) Corresponding author: hngoh@mmu.edu.my

Abstract. Ultra-Wideband (UWB) microwave imaging is an advanced imaging technique that utilizes penetrating microwaves to investigate unknown structures. The scattered wave response, governed by the composition and characteristics of the medium, is used to reconstruct these structures by solving the inverse scattering problem. However, solving the inverse electromagnetic scattering problem remains challenging due to its inherent ill-posedness, non-linearity, and high computational complexity. Various solution strategies have been proposed, with gradient-based algorithms being a common approach. These methods iteratively update solutions along the descent direction of the cost function, but they often require extensive line search calculations to determine the optimal step size. Additionally, they are susceptible to local minima, which can hinder further cost function reduction. In this work, we propose an alternative approach that eliminates the need for traditional line search by leveraging the Barzilai-Borwein (BB) method’s step size. To further enhance stability and convergence speed, we introduce a tuning parameter and an adaptive function to the mathematical formulation. The tuning parameter stabilizes early iterations, while the adaptive function dynamically adjusts the step size in later iterations to accelerate convergence. The effectiveness of the proposed technique is assessed through comparisons with the unmodified BB method and steepest descent approaches. Results reveal that the adaptive function significantly improves overall performance in terms of accuracy and computational time. We present and discuss test cases with various medium compositions, providing insights into the method’s applicability

# Introduction

Ultra-Wideband (UWB) microwave imaging is a non-destructive imaging technique that employs penetrating microwave signals to investigate unknown structures. Its versatility has enabled its adoption across a wide range of modern applications, including medical diagnostics, remote sensing, and structural evaluation, particularly in scenarios where direct or invasive examination is impractical [1-3]. Instead of physically accessing the structure, the interaction of the incident microwaves with the media is analyzed recognizing that the electromagnetic response is governed by the material’s composition. In medical imaging, for instance, the distinct dielectric properties of biological tissues allow for the reconstruction of internal anatomical structures, while in geophysical exploration, subsurface layers can be mapped based on the reflected wave patterns [4]. Recent advancements, have demonstrated the integration of microwave imaging with artificial intelligence tools, significantly enhancing both reconstruction accuracy and imaging performance [5]. This integration leverages the inherent advantages of microwave imaging as a non-ionizing, cost-effective modality. Despite variations in the specific waveforms and frequencies utilized across different applications, the underlying principle remains consistent, where the scattered field data are collected and processed through inverse problem-solving techniques to reconstruct the spatial distribution of the media under investigation.

However, solving the inverse problem is problematic, as it must overcome two critical challenges: ill-posedness and non-linearity [6]. The solution is inherently non-unique due to the incomplete observed field, inaccuracies in data acquisition, and background interference. Therefore, the existence of a unique solution that continuously depends on the data cannot be guaranteed. While regularization and prior information related to the known characteristics of the solution may mitigate ill-posedness issues, addressing the second challenge requires overcoming the non-linear nature of the problem. The relationship between the scatterer and the corresponding field is highly non-linear due to multiple scattering effects, which intensifies with stronger scatterers. Ignoring this non-linear relationship may work for weak scattering scenarios, but applications requiring full reconstruction of the imaging domain necessitate an optimization-based inversion strategy.

The local optimization inversion strategy aims to find a local minimizer of a predefined error cost function. Although the solution obtained may not always be the most accurate, it is computationally less demanding compared to global methods. The nonlinear solution is achieved through a series of linearized steps that are iteratively updated in the descent direction of the cost function. While the gradient of the cost function indicates the descent direction, it does not specify the descent distance. One might assume that maximizing the descent in each iteration would lead to faster convergence (as in the steepest descent method), but this assumption requires rigorous line search calculations and may lead to local minima traps, thereby limiting further reduction of the cost function.

An alternative approach is to eliminate the need for line search altogether by determining a fixed or optimal step size.   This strategy not only reduces computational costs per iteration but also improves convergence speed, particularly in the case of complex cost functions [7,8]. Among such methods, the Barzilai-Borwein method uses information from the previous iterations to compute an optimal step size. This dynamic adjustment leads to faster convergence, making it suitable for large-scale  and ill-conditioned problems. The balance between simplicity, computational efficiency, and fast convergence continues to make the Barzilai-Borwein method a popular choice in optimization problems [9].

Recently, the Barzilai-Borwein non-line search approach was observed to outperform other methods in terms of accuracy, overall convergence, and susceptibility to local minima traps [10]. While considered stable for such types of problems compared to other non-linear descent alternatives, its stability may be affected by the properties of the cost function, leading to unstable oscillatory behavior. In this work, we investigate the Barzilai-Borwein method for imaging setups of various degrees of target complexities. Furthermore, an adaptive tuning parameter is introduced to accelerate the convergence of the minimization process.

# Mathematical Formulation

The primary objective of a forward scattering solver is to compute the response of a known incident field when all the electric properties of the problem space are known. The inverse scattering solution on the other hand, seeks to identify the electrical parameters that lead to a known response from a known incident field. This known response is usually obtained from a set of measurements collected from observation points distributed outside the problem space in a geometry that forms the imaging plane.

A wideband of frequencies illuminating source ensures the transmission of wide frequency spectrum instead of a single monochromatic signal. In turn, objects scatter energy at very long as well as very short wavelengths, which provides more information about the shape and material structure of the objects. The low frequency spectrum aids in the linearization of the problem while the high frequency spectrum maintains the spatial resolution. With this in mind the ultra-wide band excitation pulse of the UWB is more suitable solving in time domain, rather than frequency.

## The Forward Problem

The propagation of electromagnetic waves in linear, isotropic, and none dispersive media is governed by Maxwell’s curl Equations (1) and (2),

∇xE+μ∂H∂t=0						(1)

∇xH-ε∂E∂t-σE=0 						(2)

Here, E and H denote the electric and magnetic fields in the x, y, and z directions, while ε, μ, and σ represent the electric permittivity, magnetic permeability, and electrical conductivity of the medium, respectively. The finite-difference time-domain (FDTD) technique employs finite differences as approximations to both the spatial and temporal derivatives that appear in Maxwell’s equations. For transverse magnetic (TM) polarization, and assuming a non-conductive medium with unity relative permeability for simplification, FDTD equations at the nth time step and the (i, j) spatial location in a two-dimensional domain can be expressed in Equation (3) to (5).

Ezn+12 i,j-Ezn-12 i,jΔt=1ε0εri,jHyn i+12,j-Hyn i-12,jΔx-Hxn i,j+12-Hxn i,j-12Δy 			(3)

Hxn+1 i,j+12-Hxn i,j+12Δt= -1μo Ezn+12 i,j+1-Ezn+12 i,jΔy				(4)

Hyn+1 i+12,j-Hyn i+12,jΔt= 1μo Ezn+12 i+1,j-Ezn+12 i,jΔx    				(5)

Using the equations presented above, a computational domain can be constructed by discretizing both space and time. A uniform spatial grid is assumed (Δx=Δy=Δz), where each grid cell represents the local dielectric properties of the medium. The computational domain can accommodate different media by a spatially varying the dielectric distribution [11]. The imaging space is formed by placing a series of transceiver points placed equidistantly along the perimeter. This configuration enables full-view scanning of the target region. The background medium within the imaging domain is assumed to be homogeneous with known electromagnetic properties. To effectively simulate an open space and prevent artificial reflections at the domain boundaries, Perfectly Matched Layers (PML) are employed to absorb outgoing waves. This arrangement is illustrated in Figure 1.

Figure 1. Topology of the 2-D bounded imaging domain illustrating the imaging plane, and transceiver distribution

Each transceiver sequentially transmits an UWB pulse to illuminate the domain, while the remaining transceivers function as receivers, capturing the resulting electromagnetic field response. This process yields a total of N projections, withM=N2-N unique measurements recorded. Image reconstruction is performed based on the field responses corresponding to the individual projections, measured over the time interval [0,T].

## The Inverse Problem

The inverse problem is formulated by solving an error cost function F is defined as the least squares between the calculated (Ecalc) and measured (Emeas) data sets for each transmitter-receiver pair composing the full view geometry. The goal of the optimization process is to solve the material properties φ =(ε) of the medium over the domain 𝛺, which minimizes the cost function F defined as the least square between the calculated field Ecalc and measured field Emeas. Assuming a total of M transmitters and N receivers, the residual error r of the N receiving locations for the mth projection over the time T in TM mode can be expressed in Equation (6).

rφ=12n=1NTΩEzcalcnφ-Ezmeasn2dΩdtδn 			(6)

where δ is the Dirac delta function which is valid in the discrete observation space point and none otherwise. The cost function of all M transmitters (projections) can be then expressed in Equation (7).

Fφ=m=1Mrmφ						(7)

where m=1,2,..,M, is the transmitter location.

For a multivariable vector function φ, the gradient g = ∇F(φ) indicates the direction of the steepest ascent of the scalar function F at the point φ. Consequently, the negative gradient points toward the direction of descent, guiding the optimization process toward a local minimum, which defines the search direction for our solution. In inverse scattering problems, the gradient is computed efficiently using the adjoint state method, which involves incorporating Maxwell’s equations as constraints into the objective functional via Lagrange multipliers. This leads to the formulation of an adjoint problem, which is solved by propagating the residuals backward in time (Figure 2). The adjoint fields can be computed using the Finite-Difference Time-Domain (FDTD) method, analogous to the forward (direct) problem simulation [12]. The gradient of the cost function F for the ith and jth position is given as Equation (8).

g=∇Fφ=m=1Mnt=1NT-1i,j ϵΩNi  NjezdEzdtΔxΔy Δt  			(8)

where ez, and Ez are the electric fields obtained from backward and forward time stepping of the FDTD solver respectively, Δt is the length of the time step, NT is the total time steps, Ni and Nj are total unknowns in the ith and jth direction respectively.

The iterative process begins with an initial guess φ0, which is typically chosen to represent a homogeneous medium, reflecting the predominant composition of the imaging domain under investigation. The steepest descent utilizes the gradient descent at each iteration to achieve the maximum amount of decrease in the cost function, resulting in a zig-zag like path from the initial guess φ0 and gradually sliding down the gradient until it converges orthogonally to the point of minimum, mathematically (see Equation (9)).

φk+1=φk-αk∇Fφk 					(9)

where at each iteration k, the step size αk ≥0  is chosen to minimize (see Equation (10))

αk=minαFφk-α∇Fφk 					(10)

Figure 2. Utilizing the forward and backward time stepping for calculating the gradient

The steepest descent algorithm proceeds by conducting a line search in the direction of -∇Fφk  until a minimizer φk+1 is found. The process is iteratively repeated until the gradient is equal to zero ideally. However, this is not the usual case, and a stopping criterion has to be specified from which F(φk)-F(φk-1) is less than the designated accuracy value for convergence, or F(φk) is equal a certain percentage of F(φ0).

Meanwhile, the Barzilai-Borwein two-point gradient method derives the step size from a two-point approximation of the secant equation underlying quasi-Newton methods, here has a fixed size given as:

α1k=Sk-1Tyk-1yk-1Tyk-1  							(11)

and         α2k=Sk-1TSk-1Sk-1Tyk-1          						(12)

where Sk-1=φk-φk-1   and    yk-1=gk-gk-1. The fixed-steps α1  and α2  given in Equations (11) and (12) can

be used interchangeably as in Equation (13).

The spatial dielectric distribution φ can then be updated as Equation (13) to (14)

φk+1=φk+θkαkgk						(13)

where        θk=η+1-ηFφk-1-Fφk-2     					(14)

Here, η is an adaptive tuning parameter introduced to accelerate convergence. It dynamically adjusts based on the convergence behavior observed in previous iterations and can range from 0% to 100%. In this formulation, the update step is partitioned such that one portion is derived from the Barzilai-Borwein step, while the other is informed by the rate of error reduction observed as the iterations proceed

# Methods and Setup

The forward problem is configured using a multistatic arrangement consisting of eight UWB transceivers. Each transceiver sequentially transmits, illuminating the imaging domain one at a time. For each transmission, the remaining seven transceivers capture the scattered waveforms, yielding a total of 56 unique transmitter-receiver measurements. The transceivers are positioned equidistantly along the perimeter of the imaging domain, as illustrated in Figure 1.

The imaging domain is discretized into a grid of 54 × 54 cells, with a spatial resolution of 2.5 mm in both dimensions. The temporal resolution is set to Δt = 4.17 ps. To minimize reflections and emulate an open-space environment, the computational domain is enclosed by a perfectly matched layer (PML) of 5 cells in all directions. In this context, two setups are examined. The first setup features a 10 mm square object with a relative permittivity of 4, positioned off-center. In the second setup, two additional objects are added: one measuring 5 mm with a relative permittivity of 6, and another measuring 2.5 mm with a relative permittivity of 8. Both setups are illustrated in Figure 3.

| (a) | (b) |
| --- | --- |

Figure 3. Imaging setups featuring (a) a single square10 mm target with a relative permittivity (eps_r) of 4, and (b) multiple square targets with dimensions of 10mm, 5mm, and 2.5mm, and relative permittivity (eps_r) of 4,6, and 8 respectively

A 5 GHz ultra-wideband (UWB) source is used to illuminate the targets. Figure 4 presents the resulting disturbance in the electric field transmitted from source 1, caused by the presence of the 10 mm object in the first setup.

Figure 4. Electric field as illuminated from source 1, due to the presence of the 10 mm target taken at: top-left 125 ps, top right 250 ps, bottom left 375 ps, and bottom right 458 ps

The final image is based on the reconstruction of the inverse solution where an iterative procedure takes place. The initial guess, known to significantly influence the convergence and accuracy of the iterative process, is typically chosen to represent a homogeneous medium [14], which, in this case, is free space. Based on this initial guess, the first iterations are computed using the steepest descent method to determine the step sizes α1 and α2 required to start the Barzilai-Borwein descent. The subsequent iterations then follow according to Equations (13) and (14).

While lower values of η may promote faster convergence, they can also introduce instability or oscillatory behavior prior to meeting the stopping criteria thereby terminating the procedure prematurely. The choice of η is a tradeoff between accuracy and speed as illustrated and discussed in the next section.

# Results and discussion

To assess the effectiveness of the proposed methodology, the inversion strategy is applied to both testing scenarios. The second scenario, involving multiple targets with high dielectric constants, introduces a higher degree of nonlinearity, making the inversion process significantly more challenging.

Initially, the parameter η is set to 1, corresponding to the use of the unmodified Barzilai-Borwein update formula. For experimental benchmarking, the iterative process is executed for 50 iterations without applying a termination criterion, thereby establishing a baseline for subsequent η variations. In the following experiments, the process is repeated with a progressive sweep of η, where the value is reduced by 10% in each run.

Figure 5 depicts the convergence behavior for the first test configuration, which features a single 10 mm × 10 mm target. The plot compares the convergence trends corresponding to different selected values of the tuning parameter η. From the figure, it is evident that decreasing η leads to a faster convergence rate, with fewer iterations required to reduce the error below 1% of its initial value, a threshold that could serve as a practical stopping criterion in certain applications.

Figure 5. Convergence trends for various η values in the reconstruction of a single 10 × 10 mm offset target over 50 iterations

However, when η is reduced beyond approximately 0.7 in this case, convergence instabilities begin to emerge, eventually leading to divergence, even after reaching very low error levels. This behavior underscores a fundamental trade-off in tuning: while smaller η values can significantly accelerate convergence; they simultaneously increase the risk of numerical instability. Therefore, selecting an appropriate η requires a careful balance between convergence speed and the desired level of solution accuracy, both of which should be tailored to the specific requirements of the imaging setup.

Figure 6 presents the reconstructed images of the single 10 × 10 mm target, both generated after 50 iterations. While both reconstructions successfully identify the target’s location, size, and dielectric properties, the image produced using the adaptive method exhibits enhanced clarity, with sharper edges and more distinct target boundaries. In contrast, the image reconstructed using the Barzilai-Borwein method appears more blurred, suggesting a loss in spatial resolution. This visual improvement demonstrates the advantage of the adaptive approach in preserving structural details during the inversion process. Nonetheless, the Barzilai-Borwein method could achieve comparable accuracy with more iterations and time.

Figure 6. Reconstructed images of a single 10 mm target, taken at: (left) η=1, and (right) η=0.7, after 50 iterations. Color map representing relative permittivity

In comparison to the steepest descent method, both the Barzilai-Borwein algorithm and its adaptive modification exhibit improved performance. Figure 7 shows the convergence trend plotted against time. Upon examination, the steepest descent method requires over 167 seconds to converge to a solution that still maintains a higher error of the cost function compared to that of the proposed method. In contrast, the adaptive approach achieves a more accurate solution in just 33 seconds, corresponding to approximately 20% of the time required by steepest descent. This represents a fivefold reduction in computational time, coupled with enhanced solution accuracy. This is mainly due to the elimination of the lines search procedure in the steepest descent. All calculations were performed on an M2 processor with 8 Gbytes of RAM.

Figure 7. Convergence trends over time for the steepest descent, Barzilai-Borwein, and adaptive tuning methods (with η=0.7) in the reconstruction of a single 10 × 10 mm offset target

The second test case poses a greater challenge due to the presence of multiple targets and higher dielectric contrasts, which introduce increased nonlinearity into the problem. Figure 8 presents the convergence behavior for this scenario, comparing performance across various η values. Consistent with the first case, lower η values promote faster convergence. However, at η = 0.8, early symptoms of over tuning is observed, despite all configurations reducing the cost function error below 1% prior to the emergence of instability. Due to the stronger nonlinear effects in this case, a higher number of iterations is required. Each convergence curve in the figure corresponds to 100 iterations.

Figure 9. Convergence trends for various η values in the reconstruction of multiple dielectric targets. Color map representing relative permittivity

As in the first case, Figure 9 presents the reconstructed images at η=1, and η=0.9, after 100 iterations. Both reconstructions successfully identify target location, size, and dielectric properties. Both reconstructions also show the 2.5x2.5 mm target clearly, highlighting the capability milli-meter scale detection of microwave tomography, even with the presence of other larger targets nearby which usually cast a shadowing effect. The image produced using the adaptive method also shows enhanced clarity, sharper edges and more distinct boundaries when compared to the none-adaptive approach. The color map also shows that the dielectric target values have been accurately mapped and restored, with the adaptive approach being more accurate.

Figure 9. Reconstructed images of multiple targets, taken at: (left) η=1, and (right) η=0.9, after 100 iterations. Color map representing relative permittivity

# Conclusion

In this study, we propose an adaptive tuning strategy to accelerate the convergence of the Barzilai-Borwein optimization method. The results demonstrate that eliminating traditional line searches for step size selection significantly improves both computational efficiency and solution accuracy. Moreover, the introduction of the adaptive tuning function further enhances convergence speed. The proposed method was evaluated on imaging scenarios involving both single and multiple dielectric targets, consistently yielding accurate and robust reconstructions. The findings also highlight the risks associated with over tuning, which can lead to instability after reaching certain levels on minima. While termination criteria may vary depending on the imaging application, the choice of the adaptive tuning parameter should be carefully tailored to the specific configuration and expected material properties, as nonlinear effects play a critical role in the optimization process.

# References

U. Rafique, S. Pisa, R. Cicchetti, O. Testa, M. Cavagnaro, "Ultra-Wideband Antennas for Biomedical Imaging Applications: A Survey. Sensors, 22(9):3230, 2022.

Singh D, Särestöniemi M, Myllylä T. Microwave Technique for Brain Monitoring: State of the Art, Promising Applications, Challenges and Future Directions. IEEE Access. 2025

Zanoon, T.. "Preliminary Validation of UWB Through-Rubble Detection Measurements for Quasi Real Time Detection of Trapped Survivors", International journal of simulation: systems, science & technology, 2019.

M. Everett, "Electromagnetic Methods in Applied Geophysics", Cambridge University Press, 2013.

N. Khalid, M. Zubair, M. Q. Mehmood, and Y. Massoud, "Emerging paradigms in microwave imaging technology for biomedical applications: unleashing the power of artificial intelligence," npj Imaging, vol. 2, no. 13, Jun. 2024.

X. Cai, Y. Chen, and L. Yang, "Advances in Nonlinear Optimization for Ill-Conditioned Problems," Optimization Letters, vol. 17, no. 4, pp. 1025-1050, 2023.

Y. Huang, Y.Dai, X. Liu, and H. Zhang, On the acceleration of the Barzilai-Borwein method. Comput. Optim. Appl. 81, 3, 717-740, 2022.

X. Guo, C. Xu, Z. Zhu, and B. Zhang, "Nonmonotone Variable Metric Barzilai-Borwein Method for Composite Minimization Problems," AIMS Mathematics, vol. 10, no. 5, pp. 11759-11786, 2025.

Q. Wang and Y. Li, "Efficiency of Gradient-Based Methods in Large-Scale Machine Learning Problems," IEEE Transactions on Neural Networks and Learning Systems, vol. 34, no. 1, pp. 158-170, 2023.

T. Zanoon, "Microwave tomography of thermal breast ablation featuring quantitative image reconstruction: A numerical study," in Proc. 2019 IEEE Int. Conf. Smart Instrumentation, Measurement and Application (ICSIMA), Kuala Lumpur, Malaysia, 2019.

T. Zanoon, S. Binajjaj and M. Abdullah, "Electromagnetic tomography featuring ultra wide band sensor with conformal finite difference (CFDTD) modeling of dispersive media," 2009 IEEE Symposium on Industrial Electronics & Applications, Kuala Lumpur, Malaysia, 2009.

M. Abdullah, S. Binajjaj, T.  Zanoon, and A. Peyton, " High-resolution imaging of dielectric profiles by using a time-domain ultra wideband radar sensor". Measurement, 44(5), pp.859-870, 2011.

Yu-Hong Dai and Roger Fletcher. 2005. Projected Barzilai-Borwein methods for large-scale box-constrained quadratic programming. Numer. Math. 100, 1, 21-47, 2005.

T. Zanoon and M. Abduallah, "A hybrid image reconstruction approach for ultra-wide band microwave tomography featuring radar based and iterative methods," IEEE International Conference on Imaging Systems and Techniques, Santorini, Greece, 2014.