Dual Communication Smart Socket Based on Arduino and ESP32 with Short Message Service and Telegram

Iksan Bukhori1, a), Syafira Fauzia 1, b), and Kok Swee Sim2, c)

1 Department of Electrical Engineering, Faculty of Engineering, President University, Bekasi 17550, Indonesia.

2 Faculty of Engineering and Technology (FET), Multimedia University, Melaka 75450, Malaysia.

Corresponding author: c)sksbg2022@gmail.com 
a) iksan.bukhori@president.ac.id  
b) syafirafauzia24@gmail.com

Abstract. In this study, a novel dual network smart home system is developed using Telegram which is internet-dependent and SMS which is internet-independent. The structure of the system serves as a plug-and-play system, which means that users can simply plug in any electronics they want to control to the system. The proposed system employs two microcontrollers; Arduino UNO for SMS via SIM800L and an ESP32 for Telegram. Users can remotely turn on/off any device linked to the system via ON or OFF command, set a timer for it via TIMER ON or TIMER OFF command, or inquire about its current state via STATUS command. Tests were conducted for Telegram sub-system and SMS subsystem, including initialization time test, communication test, timer test, command accuracy test, co-operation test, and user authorization test. For SMS subsystem, five different SIM card providers in Indonesia, namely Indosat, Telkomsel, 3, XL, and Smarfren were tested. In communication test, both Telegram and SMS subsystems took on average about 1 seconds to execute ON/OFF and STATUS command. In Timer test, both subsystems executed the commands with deviation of about 3 seconds. Command executions occurred with accuracy of 100% for both subsystems. Co-operation test showed that interaction between user and the sys-tem occurs fluidly between SMS subsystem and Telegram subsystem, with any successive re-quests can be executed via separate subsystems.  The results also showed that the proposed system can work reliably in the area with and without internet connection due to the combination of SMS and Telegram.

# INTRODUCTION

Electricity has evolved into a basic need that must be supplied in today's modern world. Electricity now plays a critical role in the welfare and advancement of the economy, as well as a growth engine on both the domestic and global levels. Electrical energy is extremely crucial in a country's socio-economic development. The usage of electrical energy will continue to rise in tandem with population growth, economic growth, and an increase in varied activities and the use of means of living that require electricity [1], [2]. This is reflected in the data from Central Statistics Agency or Badan Pusat Statistik (BPS) on the capacity of electric power installed in Indonesia as shown in Figure 1.

FIGURE 1. Electricity capacity installed in Indonesia

Alongside electricity installation, the usage of Internet in Indonesia has also increased [3], as shown in Figure 2.

FIGURE 2. Trend of ICT indicators in Indonesia, 2020-2023

This trend proposes a promising solution to efficient household electric energy usage in Indonesia. With an increase in cellphone usage in a household, a proper framework utilizing cellphone with/without internet access shall provide a solution for people to control their electricity usage remotely. Similar trend is also happening in multiple countries in the whole world, which propels researches in smart home devices [4]. This feature of enabling remote control is of particular interest since unattended electronic appliances may cause several problems if they are left turned on for a long time. Beside the risk of short-circuit and house fire, the concern regarding energy efficiency also becomes important [5]. In fact, the ability of homeowners to be able to turn on and turn off their electronic utilities remotely may reduce the utility cost by 10% to 25% [6]

Most of the proposed smart home solutions are unfortunately highly dependent on the availability of stable internet connection in the house where the system is implemented and where the user is located. We would argue that this approach really limits the usage of the technology to only be applicable in some area. People in rural area who do not have internet access, for example, will not be able to use the technology. Even in urban area, when people can access internet in their home, a lot of people in Indonesia still depend on individual mobile data instead of a fixed home WIFI [7] (see Figure 3).

FIGURE 3. How people access internet in Indonesia

The same Survey also shows that 74% respondents do not have fixed broadband subscription at home (see Figure 4).

FIGURE 4. Survey on broadband subscription at respondent's home

Most of the proposed smart home solutions are unfortunately highly dependent on the availability of stable internet connection in the house where the system is implemented and where the user is located. We would argue that this approach really limits the usage of the technology to only be applicable in some area. People in rural area who do not have internet access, for example, will not be able to use the technology. Even in urban area, when people can access internet in their home, a lot of people in Indonesia still depend on individual mobile data instead of a fixed home WIFI [7] (see Figure 3).

This dependance on mobile data means that once the owners leave the house for work, meet people, or visit their hometown, any internet-dependent device in their house are unusable. Not only that people cannot use internet-dependent smart home application in villages without internet connection, they also may also not be able to utilize the convenience of that technology when they travel to their rural hometown, which beats the purpose of allowing people to access the electronics when they cannot access them directly.

This research aims at a more flexible smart home application, where users can control the power on their electronic devices remotely with or without the Internet. The installation is designed to be as simple as possible, without requiring the users to have specialized application to run the system.

The rest of the paper has the following structure. The next section presents the related works in the area of smart home system. After that, we discuss the Smart Socket device proposed in this research, and the tests of the Smart Socket and their results. Next, we discuss the future works to improve further. Finally, the last section concludes the paper.

# RELATED WORKS

There are many smart home systems being proposed over the years. However, we have identified some problematic patterns with the existing designs. The first problem is regarding the communication being used in the system. A lot of the proposed smart home/smart appliance systems are solely based on Internet of Things (IoT), and rely heavily on a reliable internet connection. Some implementations rely on Google Assistant ([6], [8]). MQTT is used to communicate between the proposed system and custom mobile app in [9]. Firebase became the main part of the system proposed in [10]. Researchers in ([11], [12])  added data analysis to add some features, while researchers in [13] added voice recognition to the system. Nevertheless, these proposed systems still rely on internet connection. We would argue that even though internet is ever present in this day and age [14], full reliance on the Internet may be counter-productive. For example, in some area the internet connection may be unreliable at best, or nearly non-existent at worst. Being able to reduce this reliability is a good idea to increase the robustness of the system to be applied in many places.

The second problem is regarding the society’s acceptance of the system itself. Some proposed systems require the users to have a software that is developed specifically for the purpose of proposed system, such as specialized Android app ([15], [16], [17]). Some other proposals require the users to use an application that is not commonly used among general citizens, such as Blynk ([18], [19], [20]) and ThingSpeak ([21], [22]). Necessitating a new and unfamiliar application may become obstacle for the user to adopt the system [23]. In fact, we would argue that whenever possible, smart home system should use familiar technology to increase its adoptability among general citizens.

The third problem related to the ease-of-use of the system itself. Many proposed systems can be unfeasible to be implemented directly, since it requires the rewiring of the whole household’s electrical installation. This is why a lot of proposed systems are in the form of prototypes such as the ones in [24], [25], [26] which require further work to actually be implemented in a house.

In addressing the three aforementioned problems, this research focuses on the development of the system called Smart Socket to allow users to remotely turn on and turn off electronic equipment. The design focused on three things. First, the proposed device can be easily implemented without the needs to rewiring process of the whole electrical installation. Users would simply need to connect the device directly to the electronic equipment they want to control. To address the communication part, we took inspiration from [27], [28], [29] where they combine the Short Messaging Service (SMS) and Internet communication for their application. This way, whenever the internet is unreliable, users can simply use SMS, and vice versa. Similar approach has actually been proposed in [30] . However, it uses SMS as the only communication channel.

In order to avoid the needs for specialized software, Telegram messaging application is chosen for the Internet-based communication. In addition, a direct means of control is also provided by a physical switch on the device, which can also be used to prevent remote access if it is deemed necessary to do so.

# MATERIALS AND METHODS

Smart socket device proposed in this research interacts with the user via Telegram and SMS. The device acts as a middle-man between electronic device and main power source. The general design of the Smart Socket can be seen in Figure 5. As one can see in the figure, user only needs to connect household electronic appliance on the socket provided on the front side of the proposed device and connect the plug located on the back side of the device to the main power source. This electronic appliance can be computer, television, washing machine, or any electricity-dependent appliances. After that, user can either use Telegram or SMS to control the connection between the connected household appliance to the main power source.

FIGURE 5. General design of the smart socket

In details, the proposed Smart Socket consists of several features;

The Smart Socket device is a plug-and-play system, where any electronic device to be controlled need to simply be connected directly to the device.

There are three types of command that can be executed by the smart socket, sent either by Telegram or SMS. The first one is the command to turn on/off the electronic appliance attached to Smart Socket. The second one is the command to turn on/off the appliance at the specified time. The third command is the command to request the current status of the appliance connected to the Smart Socket, either on or off.

The device should work seamlessly between Telegram and SMS. It means the user can send a command from Telegram and then send another command from SMS and vice versa. For example, users should be able to turn on the appliance by Telegram and request the current status using SMS and they should receive the correct status.

A physical switch is connected to the device such that when the switch is turned off, no remote access is possible.

Only users with phone numbers and Telegram ID recorded in the program can interact with the device remotely.

## Hardware

There are seven pieces of hardware used in this study. The first is an electrical socket, which is essentially the foundation of a device with a hole to connect a household appliance's socket to this device. This port is directly connected to the relay.

The second is Arduino board, a microcontroller which has been widely used as many smart system brains, from biomedical applications ([31], [32], [33]), monitoring and positioning system ([34], [35], [36]), mobile robot ([37], [38]), to wearable devices [39] and bio signal processing [40]. In this research, it is used to process received SMS messages and forwarding signals to the relay to connect/disconnect the electrical network based on the message. The third module is the GSM SIM800L, which acts as a receiver of SMS from user, which is then forwarded to the Arduino Board for processing.

The fourth module is the 5VDC-250VAC Single Channel Relay Module to connect and disconnect household appliance to/from a power source. A Single Pole-Double Throw (SPDT) Switch which can be used to disable any remote control either from Telegram or SMS is also used in the design.  Real Time Clock (RTC) is attached to Arduino to keep track the current time, such that the device can be used in Timer Mode, where the user can specify when the ON or OFF command should be executed.

The last one is the ESP32 microcontroller which handles the requests from Telegram. The interconnection between these components can be seen in Figure 6.

FIGURE 6. Interconnection of proposed device

## System Design

There are several systems and subsystems working together in this Smart Socket device. The three main systems are the Arduino System, ESP32 System, and Relay Control. Inside Arduino system, there are Initialization, SIM Mode, and Timer Mode. In ESP32 System, there are Initialization, Telegram Mode, and Timer Mode. The flowchart representing the whole system is depicted in Figure 7. The main two subsystems, Telegram and SMS, communicate with each other through three physical lines. These lines have two purposes; to allow ESP32 to control the Relay via Arduino, and to make sure any changes in one subsystem is also recorded in another, e.g., the current status of the appliance connected to the system.

FIGURE 7. Flowchart of the main system

### Relay Control

Relay control’s program is implemented in Arduino. It can be visualized as a flowchart in Figure 8.

FIGURE 8. Relay control flowchart

As one can see, the inner working of relay control is rather simple. In addition to turning on and off the relay, the program also updates the status variable according to the latest relay operation. Not only that, it also sends a status signal to ESP32.

### Arduino Main System

The main system on Arduino side consists of several subsystems and data processing. Figure 9 shows the main program on Arduino’s side.

FIGURE 9. Arduino main system

In its main program, Arduino checks for the timer variable. If it is not empty, then timer mode needs to run. If it is empty, then Arduino will simply listen to SIM800L module to see if there is any command coming in the form of SMS. Also, Arduino checks for the two wires connected to ESP32 to see if there is any request from ESP32 to turn on or turn off the relay.

### Arduino SIM Mode

In SIM Mode, Arduino listens to SIM800L Module. If there is a valid message from verified user incoming, it will parse the message and perform appropriate actions. The whole process is shown in the flowchart in Figure 10.

FIGURE 10. Arduino SIM mode

### Arduino Timer Mode

RTC module plays a crucial role in this part. When the timer variable is not empty (see Figure 4), Arduino will check for the current time recorded by RTC. If the current time matches the timer variable, action according to the timer message (to turn on or turn off) is executed. The process is shown in Figure 11.

FIGURE 11. Arduino timer mode

### ESP32 Main System

ESP32 is mainly responsible for the communication with Telegram. Just like on the Arduino side, the message received by ESP32 (by Telegram) determines whether the system should go to timer mode or Telegram mode. This main system is depicted in Figure 12.

FIGURE 12. ESP32 main system

One thing to note here is that ESP32 does not need RTC to keep track of time. Instead, it can get the current time by accessing ntp server via the internet.

### ESP32 Telegram Mode

In Telegram mode, ESP32 listens and respond using Telegram bot to communicate with user’s Telegram account. It also provides commands to Arduino according to the Telegram message. These commands are sent via the three parallel lines between ESP32 and Arduino (see Figure 6). The Telegram mode is depicted in Figure 13.

FIGURE 13. ESP32 Telegram mode

### ESP32 Timer Mode

The timer mode in ESP32 is pretty much similar to the one in Arduino. The only difference is that once the current time retrieved from ntp server is the same as the timer variable, a signal is sent to Arduino to run an action on the relay, since ESP32 itself is not connected to the relay directly. The ESP32 timer mode program is shown in Figure 14.

FIGURE 14. ESP32 timer mode

## Commands

To use the proposed Smart Socket, users need to use a predefined set of commands to perform specific actions, either via SMS or Telegram. The list of commands is shown in Table 1.

TABLE 1. Formats of the commands

| Command |  | Function | Remark |
| --- | --- | --- | --- |
| SMS | Telegram |  |  |
| On | /On | Turn on relay | Case insensitive |
| Off | /Off | Turn off relay | Case insensitive |
| On hh.mm | /On hh.mm | Turn on at the time is at hh.mm | Case insensitive |
| Off hh.mm | /Off hh.mm | Turn off at the time is at hh.mm | Case insensitive |
| STATUS | /STATUS | Send current STATUS to user via SMS/Telegram | Case insensitive |

All commands are case insensitive. Due to how Telegram bot works, a forward slash is required to prefix any command.

## Final Product

The appearance of the proposed device is shown in Figure 15. The plug which will be connected to the main power source is located on the back side. The explanations for each numbered labels are presented in Table 2.

FIGURE 15. Proposed device (front-side)

TABLE 2. Front side of the device

| Number | Component |
| --- | --- |
| 1 | Electrical socket for connecting electronic appliance into this system |
| 2 | Indicator that the SIM800L v2 is ready for use. If this LED turning ON its interpretation the device successfully carried out the initialization process. |
| 3 | Indicator of the relay position. If this LED is turned ON meaning that the relay is in ON condition. Meanwhile, if this LED is OFF, the relay is in OFF condition. |
| 4 | Switch |

# RESULTS AND DISCUSSION

This research employs an experimental research design. Tests are performed to check that the Smart Socket is operating effectively and in accordance with the objectives that have been established. The experiment consists of Initialization Time Test, Communication Test, Command Accuracy Test, TIMER Command Test, SMS-Telegram Co-test, and Security Test. In Telegram-based communications, all experiments are done under the internet connection provided by Indosat, as it is the network that is deemed most stable in the area where the experiments are conducted. All tests are conducted in the area of President University, Cikarang, Indonesia in August to September 2023.

In the tests involving time, the standard deviation is presented along with the average. Let n be the number of experiments and Xi as the random variable representing ith measurement. The equation used for the average of the measurement is as shown in Equation (1).

| μ=i=1nXin | (1) |
| --- | --- |

Meanwhile, the standard deviation is treated as the sampled standard deviation and is shown in Equation (2).

| σ=i=1nXi-μ2n-1 | (2) |
| --- | --- |

## Initialization Time Test

Initialization time is the interval of time it takes for the Smart Socket from being turned on until it is ready to use. That is, until the system is ready to receive Telegram or SMS. This is indicated by the blue LED on the device (see Figure 16).

The time it takes for the Telegram subsystem to be ready to receive message is compared against five cellular providers in Indonesia for the SMS subsystem. These providers are Indosat, Telkomsel, XL, 3, and Smartfren. These five providers are the ones commonly used in the area where the tests are conducted. The result is as shown in 
Table 3.

FIGURE 16. Successful initialization

TABLE 3. Initialization time between Telegram and SMS using five cellular providers in Indonesia

| Measure | Initialization Time (Seconds) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Telegram | Indosat | Telkomsel | 3 | XL | Smartfren |
| μ | 1.123 | 26.162 | 25.172 | 26.815 | 25.134 | 25.267 |
| σ | 0.004830 | 0.327882 | 0.119425 | 0.111977 | 0.071678 | 0.134416 |

As one can see, in general initialization of Telegram takes the least amount of time, compared to initialization of the SMS-based system.

## Communication Time Test

In this test, two different commands are sent to the Smart Socket for either Telegram and SMS. The first command is the STATUS command, which requested the device to send the current status of the connected appliance (either on or off). The time elapsed between when the message is sent until a reply is received is recorded. The second command is the ON/OFF command which turns on/turns off the connected appliance, accordingly. The time required from when the message is sent until the specified action is executed is then recorded. Each test is repeated 10 times. The result of the Telegram communication test is presented in Table 4.

TABLE 4. Communication test for Telegram

| Measure | Command Type |  |
| --- | --- | --- |
|  | STATUS (seconds) | ON/OFF (seconds) |
| μ | 2.111 | 2.797 |
| σ | 0.788 | 1.093 |

Five different cellular providers in Indonesia are tested against each other in this test, the same ones as the ones used in the Initialization test. The summary of result is shown in Table 5. In the experiments, Smartfren totally failed the test. During the test, the device can still initialize using Smartfren SIM card (as shown in Table 3). However, no communication could be conducted. It is suspected that this SIM card simply does not work well with SIM800L module used in the proposed device. A more thorough investigation may be required to understand this behaviour better.

In a similar vein, an interesting result happened in the test for XL. Somehow, only one communication is allowed, from the user to SIM800L. The SIM card simply cannot send a message back from SIM800L to user’s phone. A further investigation on the compatibility of this particular SIM card and SIM800L module may be required.

TABLE 5. Communication test for SMS

| Provider | STATUS Command |  | ON/OFF Command |  |
| --- | --- | --- | --- | --- |
|  | μ | σ | μ | σ |
| Indosat | 5.415 | 1.023 | 6.517 | 1.093 |
| Telkomsel | 4.724 | 1.262 | 6.502 | 1.961 |
| XL | Failed |  | 6.147 | 1.374 |
| 3 | 6.518 | 1.306 | 6.132 | 1.450 |
| Smartfren | Failed |  | Failed |  |

For the next part of the experiments, only the most stable SIM card needs to be used. Based on the tests conducted, in SMS subsystem, Indosat, Telkomsel, and 3 never fail the test, and we need to determine which one to use in the next set of tests. It is not enough to claim one of the providers is the best one by comparing a single value (such as the average). The one claimed to be the best one should also be statistically significant enough to warrant the claim [41].In this study, we use ANOVA test due to the fact that there are more than two groups of data to be compared. ANOVA test is used to test the performance between the three providers for one-way communication (ON/OFF command) and two-way communication (STATUS command). The results of the ANOVA test for the ON/OFF command and STATUS command are shown in Table 6 and Table 7, respectively.

TABLE 6. ANOVA test for One-Way communication (ON/OFF) command (α=0.05)

| Source of Variation |  | SS |  | df |  | MS |  | F |  | P-value |  | F crit |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Between Groups | 16.37509 |  | 2 |  | 8.187543 |  | 5.651771 |  | 0.008906 |  | 3.354131 |  |
| Within Groups | 39.11405 |  | 27 |  | 1.448669 |  |  |  |  |  |  |  |
| Total | 55.48914 |  | 29 |  |  |  |  |  |  |  |  |  |

TABLE 7. ANOVA test for Two-Way communication (STATUS) command(α=0.05)

| Source of Variation | SS | df | MS | F | P-value | F crit |
| --- | --- | --- | --- | --- | --- | --- |
| Between Groups | 0.951167 | 2 | 0.475583 | 0.199703 | 0.820173 | 3.354131 |
| Within Groups | 64.29933 | 27 | 2.381457 |  |  |  |
| Total | 65.2505 | 29 |  |  |  |  |

As can be seen from Table 6, there is significant difference between groups in the one-way communication test. However, in two-way communication test there is not significant difference (see Table 7). Therefore, we can choose the best SIM card to use based on the one-way communication test.

Using the data from Table 5, in the one-way communication (ON/OFF command), Indosat provides the least variance. In fact, if we have a look at the data from two-way communication (STATUS command) test, Indosat also provides a more stable result with the lowest variance.

Using these results, the next set of tests focuses on the Indosat SIM card for the SMS subsystem. It should be noted, however, that this test is done merely to simplify the next tests that follow by using the SIM card that yields the most stable connection. This result is closely tied to where the experiments are conducted. Other areas may yield different result, based on the signal coverage in the region.

### Command Accuracy Test

The next test is to check the accuracy of the ON/OFF command and the STATUS command. In this part, the device is tested to see if it can execute the command correctly. In this experiment, each of the command (On, Off, and Status) are sent 10 times via SMS and Telegram. The number of correct command execution (i.e., the accuracy) is then counted. If the accuracy is less than 100%, then that means there may be some errors in the program or a mistake in parsing the incoming message. The result of the test is shown in Table 8, where it can be seen that both the Telegram and SMS subsystem performs perfectly.

TABLE 8. Command accuracy test result

|  | Commands |  |  |
| --- | --- | --- | --- |
|  | ON | OFF | STATUS |
| SMS Accuracy | 10/10 | 10/10 | 10/10 |
| Telegram Accuracy | 10/10 | 10/10 | 10/10 |

### Timer Test

In this set of tests, the TIMER command is sent one by one to Telegram and SMS subsystem to turn on and turn off the relay at the time specified in the message. The deviation between the specified time and the actual time the action (on/off) is executed is then recorded. The tests are repeated 10 times with different specified time. The result is presented in Figure 19.

As can be seen in the figure, the SMS subsystem performs very stable. The deviation is consistent at 3 seconds. This is because the time is taken from the RTC module using wired connection directly. Meanwhile, there are variations in the Telegram subsystem. This is simply because in this subsystem, the current time is not obtained from the RTC, but instead from the ntp server. Thus, the time required to retrieve this information may vary depending on the internet traffic at that moment. However, the deviation itself is not much different from the SMS subsystem, averaging at 2.8 seconds for the turning on timer and 3.3 seconds for the turning off timer.

FIGURE 19. Timer command test

### SMS-Telegram co-test

The aim of this research is to develop Smart Socket that can work seamlessly between Telegram and SMS. In order to validate this objective, a set of tests are conducted. In these tests, a command is sent to one subsystem and then continued with another command being sent to another subsystem. Special attention should be paid between the ON/OFF command and STATUS command. If the two subsystems work seamlessly, the STATUS command will result in the correct status according to the ON/OFF command that has been sent before, even if the ON/OFF command is sent to a different subsystem. The result is shown in Table 9. In the table, Command #1 denotes the first command being sent to the system and Command #2 denotes the second command being sent after the Command #1 is executed.

TABLE 9. SMS-Telegram co-test

| No | Command #1 | Device Response | Command #2 | Device Response | Remark |
| --- | --- | --- | --- | --- | --- |
| 1 | ON via SMS | Relay ON | /OFF via Telegram | Relay OFF | Successful |
| 2 | OFF via SMS | Relay OFF | /ON via Telegram | Relay ON | Successful |
| 3 | /ON via Telegram | Relay ON | OFF via SMS | Relay OFF | Successful |
| 4 | /OFF via Telegram | Relay OFF | ON via SMS | Relay ON | Successful |
| 5 | ON via SMS | Relay ON | /STATUS via Telegram | Send STATUS = ON | Successful |
| 6 | /ON via Telegram | Relay ON | STATUS via SMS | Send STATUS = ON | Successful |
| 7 | OFF via SMS | Relay OFF | /STATUS via Telegram | Send STATUS = OFF | Successful |
| 8 | /OFF via Telegram | Relay OFF | STATUS via SMS | Send STATUS = OFF | Successful |

As shown in Table 9, the SMS and Telegram subsystem works seamlessly.

### User Authorization Test

In prior experiments, all commands are sent from authorized number/Telegram ID. In this set of tests, a command is sent from unauthorized number/Telegram ID, to see if the device would be able to reject such command properly. For the SMS test, rather than having to send message to the unauthorized user, their request would simply be ignored. This event can be seen in the Arduino’s serial monitor (see Figure 20).

FIGURE 20. Trying to send SMS command from unauthorized user

In Telegram subsystem, whenever unauthorized Telegram ID tried to access the device, a message is sent back to the user, as shown in Figure 21.

# FUTURE WORKS

The current design of the proposed Smart Socket device can still be improved further. There are some things that can be adjusted and added; one can add current sensor to monitor electricity usage or other sensors which allow automation, for example adding motion sensor to automatically turn off appliances when no one is home while also allowing them to be turned on remotely if needed.

FIGURE 21. Trying to send telegram command from unauthorized user

However, an immediate plan is to remove the limitation of one device one appliance. By using Master-Slave architecture. we can make one master device controlling multiple slaves, each is connected to an appliance. Note that in this plan, we try to avoid providing multiple connections on one slave to allow user to attach multiple appliances which are physically placed further apart to the system. In a Master-Slave architecture, user can access each appliance by specifying the slave device where the appliance is plugged in inside the message sent to the master device. Another avenue to take in the future is with the security. At the moment, the security measure is limited to identifying whether the ID or phone number is the one registered in the system. In the future, a security measure can be implemented prior to the message being sent, for example by using biometrics [42], can be implemented. During the data transmission, research on securing the data being transferred can also be explored [43]. Both of these, will ensure that the person sending the command is really the one with the authorization to send the control messages, and the message itself is not being hijacked in transmission.

# CONCLUSION

A Smart Socket device is presented in this paper. Unlike many other smart appliances/smart home systems, the proposed device does not solely depend on Internet connection. It uses two types of communications; Internet-based via Telegram and SMS-based via SIM800L module. This way, the proposed device should work with or without Internet connection. In addition, the use of Telegram and SMS removes the needs for user to install/learn new software to use the device since Telegram and SMS are commonly used for many people. The device is also easy to use, since users simply need to connect any electronic appliance to the device.

It should be emphasized that since there is no rewiring needed to integrate the proposed Smart Socket to the target electronic device, a wide array of applications is possible. In household environment, any appliance such as television, air condition, washing machine, etc. can be controlled using the device. Not only that, the device should also work to control devices in office environment such as for computer, microwave, etc. Other applications for any other electronic devices/appliances should also be possible, due to the simplicity of the integration with the proposed Smart Socket.

There are three different types of command the users can send to this Smart Socket, either by Telegram or SMS. The first one is ON/OFF command to immediately turn on and turn off the connected appliance remotely. The second one is STATUS command, which sends the current status of the appliance to the user. The third one is the TIMER command, where the user can ask the device to turn on or turn off the appliance at later time.

A set of tests is conducted to test the performance of the proposed device. In the initialization test, Telegram subsystem requires on average 1.32 seconds to be ready. Meanwhile, SMS subsystem requires much more time to initialize, the fastest of which is 25.13 seconds by XL SIM card. In the communication tests, however, XL fails to send a response to STATUS command. The best performance for this test is achieved by Telkomsel with 4.7 seconds for two-way communication (STATUS command) and 6.5 seconds for one-way communication (ON/OFF command). However, Telegram communication still performs much better with the average around 2 seconds. Even though Telkomsel achieves better result in the communication test, it is still less stable than Indosat SIM card.

The accuracy test shows that the device accurately performs the requested actions correctly 100% of the time. Timer test shows that the SMS subsystem is more consistent than the Telegram subsystem. This is simply because Telegram subsystem needs to retrieve current time from ntp server rather than the direct process between Arduino and RTC in the SMS subsystem. The SMS-Telegram Co-Test also shows that the device works seamlessly between two subsystems. Finally, the user authorization tests show the device is able to ignore requests sent by unauthorized user.

# References

Y.S. Akil, S. Mangngenre, S.M. Said, K. Amar, and A.M.S. Yunus, “Factors influencing urban consumers on selecting electricity saving home appliances for managing energy consumption in Indonesia,” Cogent Engineering 10(2), 2287299 (2023).

BPS-Statistics Indonesia, Statistik Listrik 2018-2022 (BPS-Statistics Indonesia, Indonesia, 2023), p. 96.

BPS-Statistics Indonesia, statistik-telekomunikasi-indonesia-2020 (BPS-Statistics Indonesia, Indonesia, 2024), p. 346.

S. Khan, H. Ali, and Z. Shah, “Systematic analysis of smart homes: Current trends and future recommendations,” Cogent Engineering 11(1), 2344452 (2024).

M.Z. Fakhar, E. Yalcin, and A. Bilge, “A survey of smart home energy conservation techniques,” Expert Systems with Applications 213, 118974 (2023).

S. Imran Hussain, S. Deepalakshmi, R.J. Benilla, and V. Charu Nivetha, “Automation of Smart Home using Smart Phone via Google Assistant,” in 2023 5th International Conference on Smart Systems and Inventive Technology (ICSSIT), (IEEE, Tirunelveli, India, 2023), pp. 499–505.

Asosiasi Penyelenggara Jasa Internet Indonesia, “Survei Penetrasi & Perilaku Internet 2023,” https://survei.apjii.or.id/survei/group/8, (2024).

R. Rishi, and P. Doloi, “Home Automation Using Google Assistant,” International Journal of Innovative Research in Electrical, Electronics, Instrumentation and Control Engineering 9(6), 116–127 (2021).

K. Mohamed, and A. El Shenawy, “A Smart IoT-Based Home Automation System for Controlling and Monitoring Home Appliances,” IREACO 16(5), 228 (2023).

P.K. Goyal, S. Verma, and M. Giri, “A Firebase-Based Smart Home Automation System Using IoT,” in Innovations in Electrical and Electronic Engineering, edited by R.N. Shaw, P. Siano, S. Makhilef, A. Ghosh, and S.L. Shimi, (Springer Nature Singapore, Singapore, 2024), pp. 229–237.

S. Murugesan, A. Cm, A. Bhuvanesh, S. Manoj, P.E. Darney, and K.S. Rajan S, “Smart Home Automation with Smart Metering using ZigBee Technology and Deep Belief Network,” in 2024 2nd International Conference on Intelligent Data Communication Technologies and Internet of Things (IDCIoT), (IEEE, Bengaluru, India, 2024), pp. 988–992.

C. Lin, “An intelligent illumination system with personalized quality of services in smart home,” in Ninth International Symposium on Sensors, Mechatronics, and Automation System (ISSMAS 2023), edited by L. Pan and Z. Zhou, (SPIE, Nanjing, China, 2024), p. 84.

T. Bhuvaneswari, V. Chitra, and G.C. Cheng, “Voice Controlled Home Automation System Design,” IJORAS 5(2), 94–100 (2023).

C. Palanisamy, “Smart Manufacturing with Smart Technologies – A Review,” IJORAS 5(2), 85–88 (2023).

L. M. Satapathy, S. K. Bastia, and N. Mohanty, “Arduino based home automation using Internet of things (IoT),” International Journal of Pure and Applied Mathematics, vol. 118, no. 17, pp. 769–778, 2018.

E.I. Davies, and V.E.I. Anireh, “Design and Implementation of Smart Home System Using Internet of Things,” AIMS Research Journal Publication 7(1), 33–42 (2019).

A.H. Hussian, K. Ali, A.N.M. Hat, F. Atan, W.S.W. Mohd, M.A.M. Ayub, and M.R. Shaharudin, “Smart Control Led Downlight, Socket and Switch Using IOT With QR Security,” J. Phys.: Conf. Ser. 1874(1), 012058 (2021).

N.A. Rabbani, and Y.-L. Foo, “Home Automation to Reduce Energy Consumption,” IJTech 13(6), 1251 (2022).

R.O. Dwiansyah, and D.A. Prasetya, “SMART HOME SYSTEM WITH RADIO FREQUENCY USING BLYNK APPLICATION,” Emitor 22(2), 177–184 (2022).

H. Durani, M. Sheth, M. Vaghasia, and S. Kotech, “Smart Automated Home Application using IoT with Blynk App,” in 2018 Second International Conference on Inventive Communication and Computational Technologies (ICICCT), (IEEE, Coimbatore, 2018), pp. 393–397.

A.D. Suffian Ahmad Taufik, R. Abdullah, A.N. Jaafar, N.N.S. Nik Dzulkefli, and S.I. Ismail, “A low-cost Wi-Fi smart home socket using internet of things,” Bulletin EEI 13(2), 930–937 (2024).

A. Akhtar, T. Ahmad, N. Sabahat, and S. Minhas, “IoT Based Home Automation System Using ThingSpeak,” in 2019 International Conference on Computing, Electronics & Communications Engineering (iCCECE), (IEEE, London, United Kingdom, 2019), pp. 163–168.

S.V. Ramisetty, P. Akkineni, O.G. Sai, and P. Sai, “Home Automation using Thingspeak,” IJITEE 10(1), 219–223 (2020).

S. Chang, and K. Nam, “Exploring the Sustainable Values of Smart Homes to Strengthen Adoption,” Buildings 12(11), 1919 (2022).

C. Sree Vignesh , C.K. Jeevan, E. Kishore, and V. Murali Bhaskaran, “Smart Home Automation,” IJFMR 5(6), 9387 (2023).

S.A.B. Siddik, W.N.A.-A. Wan Husin, and T. Muniandy, “The Enhanced Speech Recognition in Automated Home Lighting System using Adaptive Time-Frequency Domain Noise Removal Algorithm Filter,” IJORAS 6(1), 20–28 (2024).

S. Budijono and Margaretta, “Smart Warning System Using SIM800L and ESP32,” IOP Conf. Ser.: Earth Environ. Sci. 794(1), 012132 (2021).

A.T. Chafa, G.P. Chirinda, and S. Matope, “Design of a real–time water quality monitoring and control system using Internet of Things (IoT),” Cogent Engineering 9(1), 2143054 (2022).

K.M. Al Abdouli, A. Rai, G. Tenzin, U. Gayleg, N. Chhetri, and A. Chhetri, “Real-time flood forecasting in Amo Chhu using machine learning model and internet of things,” Cogent Engineering 11(1), 2370900 (2024).

M.D. Artawan, A.A. Ngurah Gunawan, and M. Sumadiyasa, “Use of short message service (SMS) based ATmega328 microcontroller and SIM800L modules as on / off control electronic equipments,” Aap 6(1), 19–24 (2018).

W. Indhika Fauzhan, H. Alexander, H. Jens, and S. Eko, “Performance Evaluation of Capacitive Based Force Sensor for Electroencephalography Head Caps,” IJORAS 2, 4–8 (2020).

H. Long Pua, and K. Beng Gan, “Development of Continuous Blood Pressure Measurement System Using Photoplethysmograph and Pulse Transit Time,” IJORAS 3, 8–12 (2021).

M.-H. Wong, B.-C. Yeo, P.-K. Ng, and W.-J. Choong, “Data Acquisition System and Pattern Image Generations for Hand Grip Device,” IJORAS 3, 13–18 (2021).

Y.-S. Bong, and G.-C. Lee, “A Contactless Visitor Access Monitoring System,” IJORAS 3, 33–41 (2021).

B. Chin Yeo, M. Alvin Song Chong, W. Soong Lim, and S. Yi Lee, “Zone-Based Indoor Positioning System in Faculty Building with Neural Networks,” IJORAS 4, 8–12 (2022).

Y.H. Wong, G.C. Lee, and H.K. Sim, “RFID and Facemask Detector Attendance Monitoring System,” IJORAS 5(2), 14–24 (2023).

B. Thangavel, C. Venugopal, S. Immanuel, J.E. Raja, and W.C. Chua, “Design and Development of an Arduino Based Automated Solar Grass Trimmer,” IJORAS 6(1), 46–58 (2024).

J.H. Ang, and T.S. Min, “A Review on Sensor Technologies and Control Methods for Mobile Robot with Obstacle Detection System,” IJORAS 6(1), 78–85 (2024).

M.T. Soe, D.C.L. Yang, K. S. Sim, and W. K. Wong, “Design of wearable social distancing card for enhancing public health safety compliance,” 2576-8484 9(4), 362–378 (2025).

K. S. Sim, and Z. Y. Lim, “Fast fourier analysis and EEG classification brainwave controlled wheelchair,” in 2016 2nd International Conference on Control Science and Systems Engineering (ICCSSE), (IEEE, Singapore, Singapore, 2016), pp. 20–23.

D. C. Y. Ong, I. Bukhori, K.S. Sim, and K. B. Gan, “Adaptive Optimizable Gaussian Process Regression Linear Least Squares Regression Filtering Method for SEM Images,” IEEE Access 13, 93574–93592 (2025).

C.C. Teo, H.F. Neo, G.K.O. Michael, C. Tee, and K.S. Sim, “A Robust Iris Segmentation with Fuzzy Supports,” in Neural Information Processing. Theory and Algorithms, edited by K.W. Wong, B.S.U. Mendis, and A. Bouzerdoum, (Springer Berlin Heidelberg, Berlin, Heidelberg, 2010), pp. 532–539.

M.N. Al-Andoli, S.C. Tan, K.S. Sim, P.Y. Goh, and C.P. Lim, “A Framework for Robust Deep Learning Models Against Adversarial Attacks Based on a Protection Layer Approach,” IEEE Access 12, 17522–17540 (2024).