Estimation of Internal Forces of Three-Layer Plates Under Generally Symmetrical Loading

Mirziyod Mirsaidov1,2, Kazokboy Mamasoliev3,a, Jamshid Sindarov3,a

1Natiоnal Researсh University-Tashkent Institute оf Irrigatiоn and Agriсultural Meсhanizatiоn Engineers, Tashkent, Uzbekistan;

2M.T.Urazbaev Institute оf Mechaniсs and Seismiс Stability of Struсtures, Uzbekistan Aсademy оf Sсienсes. Tashkent, Uzbekistan;

3Samarkand State Architectural and Civil Engineering Institute, Samarkand, Uzbekistan

a) Corresponding author: q-mamasoliev@mail.ru

Abstract. This article examines estimates of internal force factors for the bending of three-layer plates freely lying on an elastic half-space. An elastic filler layer is installed between the plates, which transmits reactive pressures to the plates of the constituent layer. The top plate is loaded with generally symmetrical external loadings related to the middle of the plate. To study the stress-strain state of three-layer plates interacting with elastic half-spaces, a mathematical model and an analytical method for solving the problem based on orthogonal polynomials have been developed. An analytical solution to the problem has been obtained, which has a refined calculation algorithm for calculation and project work. Based on the results of theoretical studies and numerical calculations of the test example, conclusions were drawn about the influence of the filler on the internal forces of the plates.

Keywords. Three-layer plate, elastic filler, half-space, integra-differential equation, orthogonal polynomials, closed system of equations, internal forces.

# INTRODUCTION

Research into the stress-strain state of mutually contacting bodies, which depends on many structural elements, is one of the pressing issues of mechanics. A structure interacting with a deformable base belongs to such contacting bodies.

Researchers develop a variety of models and solution methods to assess various factors affecting critical parts of engineering structures. The performance of engineering structures is directly related to the need to improve the level and quality of their design. When constructing any structures, it is necessary to take into account the multifactorial interactions of structural elements given in the relevant design and calculation works. The foundations of industrial and civil buildings, as well as the coverings of airfields, railways, roads, pedestrian roads and many others belong to such structures. The noted shows the need to develop multifactor effective mathematical models and calculation methods for the implementation of design and calculation work, as well as during the construction of objects. The proposed models and calculation methods should lead to more economical solutions for the construction of structures.

In the work [1], the study of the time response of an elastic thin plate interacting with multilayer transversally isotropic soils was carried out. This proposes an effective theoretical method for solving the issue.

In the work [2], a study of nonlinear vibrations of pure polymer plates of three types of polymer composite plates was carried out; in [3, 4], both experimental and numerical analyzes were carried out to study the ultimate strength to model stiffeners and a support plate.

In [5, 6], the issues of bending of multilayer strip-plates lying on an elastic foundation were studied. Estimates of internal forces in plates are given based on the approximation of orthogonal polynomials.

In [7], an interaction function was constructed based on axial compression (tension) and shear loads of orthotropic plates, and in [8] an approach to dynamic modeling of a multi-plate structure connected by nonlinear hinges was presented. Linear modes have been proven to play an important role in dynamic analysis.

In [9], an algorithm and program for the numerical solution of wave issues using the method of characteristics and the finite difference method were developed.  In works [10,25], the vibrations of a rod protected from vibrations under the influence of kinematic excitations were studied, and in work [11] the stress-strain state of asymmetrically layered plates with controlled forces interacting with a sandy base was considered.

In [12], the process of free and forced vibrations of two beam systems with intermediate supports was studied. Analytical solutions were obtained to ensure effective results in design work.

[13] investigated the dynamic strain localization of plastic polymer bars under large tensile strains up to failure, and the study [14] proposed a unified procedure to analyze the free and transient vibration behavior of a composite laminated beam subjected to general boundary conditions in a thermal scenario.

In [15], the propagation of oscillatory waves and assessment of the level of their impact with various objects were studied. Mathematical models and methods for assessing the levels of vibration waves at various distances from the soil base have been developed.

In [16,17], a mathematical model was obtained and an analytical method was used to solve the problem. Orthogonal polynomials were used to estimate the internal forces of the plates. Corresponding conclusions are presented on the influence of base and filler pressure on internal forces in plates.

In [18], a mathematical model was developed for assessing the stress-strain state of ground dams using a spatial model based on the variational Lagrange equation, taking into account the real geometry, material properties and heterogeneous design features of structures.

In [19,20], modifications of the structure made from composite laminate plates were studied. The results of experimental and numerical analysis affecting the structural connection relationships are presented.

Article [21] presents a model analysis of a honeycomb structure, structure for various parametric conditions using the finite element method.

The work [22] presents a numerical algorithm for solving odd differential equations using the Runge-Kutte method, which is suitable for dynamic systems.

In [23, 28], based on the bending of the slab, the internal force factors of layer plates in contact with elastic foundations were studied. Using an analytical method, the influence of the filler on the stress-strain states of the plate was determined under various external loads.

It is known that numerous researchers have developed many different calculation methods, which are described by various models. Despite the progress achieved in this area, there is a need to develop analytical calculation methods based on modeling the operation of structures taking into account their interaction with the soil foundation.

# METHODS

# MATHEMATICAL MODEL

Let us consider two rectangular plates lying on an elastic half-space, located symmetrically, one above the other, in the form of a layered system. We assume that an elastic filler layer is installed between the plates, which transmit reactive pressures to the plates of the constituent layer.  Such structures can be called three-layer plates that have contact relationships with elastic half-spaces. For the geometric and mechanical parameters of the plates, we introduce the following notations:

h is height; 2l is width;  b is length; E is modulus of elasticity; ν is Poisson's ratio.

On the upper (second) plate, from the top there is an external load, a generally symmetrical effect relative to the middle of the plate at a certain distance (Fig. 1, a), and from the bottom - the normal reactive pressure of the filler layer pz. The lower (first) plate is affected from above by the filler pressure pz, and from below by the normal reactive pressures of the base p. We assume that the reactive pressure of the filler is proportionally equal to the deflection differences of the plates, i.e.:

pz=k(y2-y1).

where y1, y2 are accordingly, the deflections of the first and second plates; k is the coefficient of proportionality, which in the future we will call the coefficient of stiffness of the filler.

a)

b)

FIGURE 1. Design diagram of three-layer slabs (a) and three-layer beam slabs loaded (b) with symmetrical external loads.

For mathematical modeling of the issue of bending a three-layer plate having continuous, two-sided, contact relationships with an elastic half-space, a fragment of a three-layer plate with a width of one is considered and operating according to the scheme of three-layer beam plates. Then, the task is reduced to the study of the stress-strain state of three-layer beam plates lying on an elastic base when loaded with generally symmetrical external loads (Fig. 1, b).

For convenience, we set the origin of the Cartesian coordinates in the center of the symmetry of the beam plate (Fig. 1, b) and the problem is considered on the segment [-l,l] along the abscissa axis Ox (i.e. on -l≤x≤l). The deflections of the beam platey1, y2, the external load q, as well as the base pressure p, are functions of the variable x.

For deflection of beam plates, the following systems of fourth-order differential equations can be written, under the above assumptions, conditions and notations:

Dy2IV=q-ky2-y1,    Dy1IV=ky2-y1-p.                                         (1)

Where

D= Eh312(1-ν2).

To determine the precipitation of a homogeneous base V, according to the Gorbunov-Posadov hypothesis, we use the following formula:

V=D0-llln1x-s∙p(s)ds.                                                             (2)

Here, D0=2(1-ν02)πE0, E0 and ν0 are respectively, the modulus of elasticity and the Poisson's ratio of the base material.

Let's assume that there is a two-way connection between the surface of the first plate and the base. In this case, the relationship between the structure and the base, as contact conditions, is written as:

y1x=Vx,  -l≤x≤l .                                                               (3)

Thus, the study of the stress-strain state of a three-layer beam plate, according to the problem statement, leads to the solution of a system of integra-differential equations (1), (2) and (3). Equation (1) and relation (2), (3) constitute a closed system of equations with respect to the unknowns of the issue under consideration.

In the future, when solving the issue, we will use the dimensionless coordinate x, which is equal to the ratio of the absolute coordinate to the half-length of the beam plate, i.e. |x|l.

# SOLUTION METHOD

The analytical solution of the system of differential equations (1) in a generalized form is presented in the following form

y1=l42Di=14Cix4-i+fqx-fpx-Dl4i=14Biuiαx+φqx+φpx;                        (4)

y2=l42Di=14Cix4-i+fqx-fpx+Dl4i=14Biuiαx+φqx+φpx.                        5

Where Ci, Bi are constant integrations determined from the boundary conditions of the issue under consideration;

u1x=cshxcosx; u2x=snhxcosx+cshxsinx;

u3x=sinxsnhx; u2x=snhxcosx-cshxsinx;

fqIVx=qx;  fpIVx=px;  α4=kl42D ;                                                             (6)

φqx=14α30xu4α(x-s)q(s)ds;                                                                        (7)

φpx=14α30xu4αx-spsds;                                                                      (8)

Due to the symmetry of the external loads, the reactive base pressure is sought in the form of a series of even terms of Chebyshev polynomials of the first kind [23]:

px=1-x2-12n=0∞A2nT2nx                                                                       (9)

Here, A2n - are the unknown coefficients to be determined; T2nx – an orthogonal Chebyshev polynomial of the first kind.

The equilibrium equations of the beam plate can be written in the following form:

l-11p(x)dx=Ps;    l2-11xpxdx=Ms.                                                     (10)

Here through Ps and Ms – indicated accordingly, the sum of all vertical forces and the sum of their moments relative to the middle of the beam plate.

Substituting (9) into (10), while taking into account the orthogonality of the Chebyshev polynomials, we determine the first unknown coefficients of series (12) in the form:

A0=2(q+P)πl  .                                                                                (11)

Substituting (9) into (2), we obtain the following expression for determining the base settlement:

V=lD0-A0ln2+n=1∞A2n2nT2n(x).                                                         (12)

Here the following formulas were used:

-111-s2-12T0sln1x-sds=-πln2; -111-s2-12T2ksln1x-sds=π2kT2kx.

The deflections of the beam plate (7) and (8), taking into account (12), will take the following form:

y1=l42Di=14Cix4-i+fqx-Dl4i=14Biuiαx+φqx-Dl4n=0∞A2nl4Dfp,2nx+φp,2nx;        (13)

y2=l42Di=14Cix4-i+fqx+Dl4i=14Biuiαx+φqx-Dl4n=0∞A2nl4Dfp,2nx-φp,2n(x).         (14)

Here,

φp,2nx=14α40xu4αx-z1-z2-12T2nzdz;                                       (15)

fp,2nx=132n(2n-1)(2n-2)(2n-3)1-x272P2n-4(72,72)x, n>2,                  (16)

where  Pjβ1,β2(x) – Jacobi polynomials [7,16]. The form of the function fp,nx for n≤3 is determined based on the explicit form of Chebyshev polynomials [23], i.e. for the case when n=0,1 it has the form:

fp,0x=1123x+2x3arcsinx+1364+11x21-x212;

fp,2x=18xarcsinx+11208+9x2-2x41-x212.

Based on the above formulas (13) and (14), the factors of internal forces of three-layer beam plates, i.e. the angle of rotation, bending moments and cutting forces, can be represented as:

φix=Dlyi'x; Mix=-Dl2yi''x; Qix=-Dl3yi''x, i=1,2.               (17)

The draft of the base (12) and the deflections of the beam plate (13) and (14) are expressed in terms of unknown coefficients A2n. To determine the coefficients A2n, the contact condition (3) is used.

# RESULTS AND DISCUSSION

Expressions (16) and (17) defining the deflections of the beam plate, taking into account (21), for each section separately, are presented in the following form:

y1I=l42Di=14CiIx4-i+qx424-12i=14BiIuiαx-q4α41-u1αx-

-12n=0∞A2nl4Dfp,2nx+φp,2nx, x∈-a;a;                                                   (18)

y1II=l42Di=14CiIIx4-i-12i=14BiIIuiαx-12n=0∞A2nl4Dfp,2nx+φp,2nx, x∈-1;-a∪a;1;     (19)

y2I=l42Di=14CiIx4-i+qx424+12i=14BiIuiαx-q4α41-u1αx-

-12n=0∞A2nl4Dfp,2nx-φp,2nx, x∈-a;a;                                             (20)

y2II=l42Di=14CiIIx4-i+12i=14BiIIuiαx-12n=0∞A2nl4Dfp,2nx-φp,2nx, x∈-1;-a∪a;1.   (21)

Here, the symbols I and II indicate the values corresponding (Fig.2b) to the sections -a≤x≤a and -1≤x≤-a,  a≤x≤1. In this case, arbitrary constants CiI, CiII, BiI, BiII,(i=1,2,3,4) are determined from the following boundary conditions:

- When x=0,

Q1I0=0; Q2I0=0; φ1I0=0; φ2I0=0.

- When x=±1,

Q1I±1=0; Q2I±1=0; M1I±1=0; φ2I±1=0.

- When x=±a  (conditions for connecting sections) internal force factors satisfy the following equality:

Q1I±a-Q1II±a=P;  Q2I±a=Q2II±a;

M1I±a-M1II±a=M;  M2I±a=M2II±a;

φ1I±a=φ1II±a; φ2I±a=φ2II±a

y1I±a=y1II±a;  y2I±a=y2II±a.

The relative deflections of the beam plate satisfying the boundary conditions are represented in the following form:

y1x=F1x+n=0∞A2nF1,2nx;                                                               (22)

y2x=F2x+n=0∞A2nF2,2nx.                                                        (23)

The following designations are introduced here:

F1x=l42DC2Ix2+qx424-12B1Iu1αx+B3Iu3αx-q4α41-u1αx,  when x∈-a;a,F1x=l42DC2IIx2-12B1IIu1αx+B3IIu3αx,  when x∈-1;-a∪a;1.    (24)

F2x=l42DC2Ix2+qx424+12B1Iu1αx+B3Iu3αx-q4α41-u1αx,  whenx∈-a;a,F2x=l42DC2IIx2+12B1IIu1αx+B3IIu3αx,  when x∈-1;-a∪a;1. (25)

F1,2nx=-12l4Dfp,2nx+φp,2nx;                                                             (26)

F2,2nx=-12l4Dfp,2nx-φp,2nx;                                                            (27)

C2II=π4A0; C2I=11+2α2π4A0-qa2α22;

B1II=A02α3b1u1αφ0'''1+αu4αφ0''1; B3II=A02α3b1u3αφ0'''1-αu2αφ0''1;

B1I=12α3b2-2P+l4Dqau1αa+-2M+l4D2α32C2I+qα22u3αa-q4α4+

+A02α3b1u1αφ0'''1+αu4αφ0''1;

B3I=12α3b2-2P+l4Dqau3αa+-2M+l4D2α32C2I+qα22u1αa+

+A02α3b1u3αφ0'''1-αu2αφ0''1;

b1=u1αu2α+u3αu4α;  b2=u1αau2αa+u3αau4αa.

As noted above, the contact conditions (3) are used to determine the unknown coefficients A2n. To do this, we substitute expressions (12) and (22) into equality (3), then multiply both parts of the equalities by expression 1-x2-12T2jx and integrate in the range from -1 to 1.

When integrating, we take into account the orthogonality of the polynomials and obtain an infinite system of linear algebraic equations with an infinite number of unknown relatively unknown coefficients A2n, in the form:

a2j+n=0∞a2n,2jA2n=c2jA2j, j=1,2,3,4,…                                                      (28)

Where

c2j=π1-ν02lE0j; a2j=l42D-11F1x1-x2-12T2jxdx;                              (29)

a2n,2j=-l42D-11F1,2nx1-x2-12T2jxdx.                                      (30)

By integrating integrals (29) and (30) in parts, we can get rid of the singularity:

a2j=l42D(-14j)-11F1'x1-x212P2j-1(12,12)xdx,

a2n,2j=-l42D(-14j)-11F1,2n'x1-x212P2j-1(12,12)xdx.

F1'(x)=l42D2C2Ix+qx36-12-B1Iαu4αx+B3Iαu2αx-q4α3u4αx,  when x∈-a;a,F1'x=l42DC2II2x-12-B1IIαu4αx+B3IIαu2αx,  when x∈-1;-a∪a;1.

F2n'x=-12l4Dfp,2n'x+φp,2n'x;

fp,2n'x=116n(2n-1)(2n-2)1-x252P2n-3(52,52)x, n>1;

φp,2n'x=12α30xu3αx-z1-z2-12T2nzdz.

The resulting formulas have a convenient form and it becomes possible to implement calculations using computer technology.

The solution of the system (28) is determined by the reduction method. Based on the principle of the reduction method, we limit ourselves to a few first-order unknowns A2, A4, …, A2r with corresponding r equations, systems of equations (28). We determine the solutions A2, A4, …,A2rfrom the compiled system and, substituting them in (22) and (23), we find the deflections of the beam plate. Then, using deflection formulas, it is possible to calculate the internal forces of the beam plate based on formulas (17). Here it can be easily seen that the constraints, respectively, by r equations with r unknowns in system (28), uniquely correspond to those taken in place of the infinite series (9) in the form of a finite series consisting of r terms.

# TEST CASE

Let's consider a numerical example to illustrate the presented methodology. The calculation is carried out in the following mechanical and geometric parameters:

for soil -  E0=5∙102kgsm2;   ν0=3∙10-1.

for plates -  E=1,25∙105kgsm2; ν=1,67∙10-1; l=4∙102sm;h=2,5∙10 sm.

for the stiffness coefficient of the filler k, having the dimension kgsm3   -

1∙10-1;1,5∙10-1;  2∙10-1;  2,5∙10-1; 3∙10-1; 3,5∙10-1;  4∙10-1;4,5∙10-1;  5∙10-1.

To carry out calculations in series (18), we take the first four terms with unknown coefficients. The numerical values of the unknown coefficients A0,  A2,  A4,  A6corresponding to different values of the filler stiffness coefficient k are given in Table 1.

TABLE 1. Numerical values for solving algebraic equations

|  | A0l(a(q+P))-1 | A2l(a(q+P))-1 | A4l(a(q+P))-1 | A6l(a(q+P))-1 |
| --- | --- | --- | --- | --- |
| 0.10 | 0.636619734 | -0.087965346 | -0.006879273 | 0.000589234 |
| 0.15 | 0.636619734 | -0.088674923 | -0.007182671 | 0.000577423 |
| 0.20 | 0.636619734 | -0.089776138 | -0.007326346 | 0.000576461 |
| 0.25 | 0.636619734 | -0.091682197 | -0.007581275 | 0.000577862 |
| 0.30 | 0.636619734 | -0.093138624 | -0.007813469 | 0.000571349 |
| 0.35 | 0.636619734 | -0.094386526 | -0.008095364 | 0.000569784 |
| 0.40 | 0.636619734 | -0.095163543 | -0.008274618 | 0.000568126 |
| 0.45 | 0.636619734 | -0.096857319 | -0.008337421 | 0.000561237 |
| 0.50 | 0.636619734 | -0.098976734 | -0.008419263 | 0.000557329 |

According to Table 1, it can be noted that:

- Changing the stiffness value of the aggregate does not lead to a noticeable change in the solution of the algebraic equations and, similarly, does not lead to a change in the distribution of the base pressure.

- To ensure the necessary calculation accuracy with a uniform distribution of base pressure in the Chebyshev polynomial, it is sufficient to limit ourselves to the first four terms of the series.

Based on the calculation results, Table 2 shows the maximum values ​​of bending moments in beam plates with comparisons with the results [23], - M1(ql2)-1, M2(ql2)-1.

TABLE 2. Maximum values of bending moments in beam plates

|  | 0.10 | 0.15 | 0.20 | 0.25 | 0.30 | 0.35 | 0.40 | 0.45 | 0.50 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| M1(aql2)-1 | 0.076583 | 0.077361 | 0.079714 | 0.080675 | 0.082305 | 0.082576 | 0.082971 | 0.082987 | 0.083271 |
| M1(a(q+P)l2)-1 | 0.079321 | 0.080638 | 0.081275 | 0.082621 | 0.084019 | 0.085219 | 0.086233 | 0.087683 | 0.088624 |
| M2(aql2)-1 | 0.114672 | 0.112318 | 0.110623 | 0.108937 | 0.098601 | 0.092461 | 0.098119 | 0.098362 | 0.097876 |
| M2(a(q+P)l2)-1 | 0.119864 | 0.118638 | 0.115563 | 0.110695 | 0.109487 | 0.106228 | 0.101278 | 0.099264 | 0.098967 |

Analyzing the data in Table 2, it can be noted that:

- An increase in the rigidity of the filler leads to the maximum values ​​of bending moments in the plates approaching each other.

- Bending moments will increase to 10%.

# CONCLUSION

Analyzing the results of the study, we can conclude:

- A mathematical model and an analytical method have been developed to evaluate the bending of three-layer plates interacting with elastic bases loaded with generalized symmetrical external loads.

- An improved algorithm for calculating three-layer plates lying freely on an elastic base is presented.

- The nature of the influence of the filler layer on the internal forces in three-layer plates lying on an elastic base is determined.

- Limits of numerical limitations have been established to ensure the accuracy of calculating practical calculations of three-layer plates having continuous contact with elastic half-spaces.

# REFERENCES

M. Jia, Y. Yang, Z. Ai. Time history response of an elastic thin plate on a transversely isotropic multilayered medium due to vertical loadings. Computers and Geotechnics Volume 134, (2021).

- https://doi.org/10.1016/j.compgeo.2021.104058

J. J. Mao, S. K. Lai, W. Zhang, Y. Z. Liu, Comparisons of nonlinear vibrations among pure polymer plate and graphene platelet reinforced composite plates under combined transverse and parametric excitations. Composite structures. Volume 265, (2021). https://doi.org/10.1016/j.compstruct.2021.113767

L. Li., C. Liu, W. Zhang, Z. Du, X. Guo. Combined model-based topology optimization of stiffened plate structures via MMC approach. International journal of Mechanical Sciences. Volume 208, (2021). https://doi.org/10.1016/j.ijmecsci.2021.106682

H. Ma, Q. Xiong, D. Wang, Experimental and numerical study on the ultimate strength of stiffened plates subjected to combined biaxial compression and lateral loads. Ocean Engineering. Volume 228, (2021).

- https://doi.org/10.1016/j.oceaneng.2021.108928

M. Mirsaidov., Q. Mamasoliev, Contact problems of multilayer slabs interaction on an elastic foundation.2020. IOP Conf. Series: Earth and Environmental Science 614 (2020) 012089 IOP Publishing https://doi:10.1088/1755-1315/614/1/012089

M. Mirsaidov, K. Mamasoliev, K. Ismayilov, Bending of Multilayer Slabs Lying on Elastic Half-Space, Considering Shear Stresses. In: Vatin N., Roshchina S., Serdjuks D. (eds) Proceedings of MPCPE 2021. Lecture Notes in Civil Engineering, vol 182. Springer, Cham. (2022). https://doi.org/10.1007/978-3-030-85236-8_8

B. Wa, X. Chen, X. Sun, P. Chen, Z. Wang, Y. Chai, Interaction formulae for buckling and failure of orthotropic plates under combined axial compression/tension and shear. Chinese Journal of Aeronautics. Volume 35, Issue 3, Pp. 272-280, (2022). https://doi.org/1016.jcja.2021.01.021

Y. Cao, D. Cao, G. He, X. Ge, Y. Hao, Modelling and vibration analysis for the multi-plate structure connected by nonlinear hinges. Journal of Sound and Vibration. Volume 492, (2021). https://doi.org/10.1016/j.jsv.2020.115809

K. Sultanov, Contact interaction of an underground pipeline with soil under dynamic impacts. Facta Universitatis, Series: Mechanical Engineering, [S.l.], (2024). ISSN 2335-0164. Available at: <http://casopisi.junis.ni.ac.rs/index.php/FUMechEng/article/view/12383>

M. Mirsaidov, O. Dusmatov, M Khodjabekov, Mode Shapes of Transverse Vibrations of Rod Protected from Vibrations in Kinematic Excitations. Proceedings of FORM  Pp 217-227, (2021) https://doi.org/10.1007/978-3-030-79983-0_20

Ph. Schreiber, Ch. Mittelstedt, Buckling of shear-deformable unsymmetrically laminated plates. International Journal of Mechanical Sciences Volume 218, (2022),

- https://doi.org/10.1016/j.ijmecsci.2021.106995.

X. Zhao, P. Chang, Free and forced vibration of double beam with arbitrary end conditions connected with a viscoelastic layer and discrete points. International Journal of Mechanical Sciences. Volume 209, (2021).

- https://doi.org/10.1016/j.ijmecsci.2021.106707

L. Zhang, A. Pellegrino, D. Townsend, N. Petrinic, Temperature Dependent Dynamic Strain Localization and Failure of Ductile Polymeric Rods under Large Deformation. International Journal of Mechanical Sciences Volume 204, (2021). https://doi.org/10.1016/j.ijmecsci.2021.106563

D. Shao, Q. Wang, Y. Tao, W. Shao, W. Wu, A unified thermal vibration and transient analysis for quasi-3D shear deformation composite laminated beams with general boundary conditions. International Journal of Mechanical Sciences. Volume 198, (2021),https://doi.org/10.1016/j.ijmecsci.2021.106357

M. Mirsaidov, M. Boytemirov , F.Yuldashev, Estimation of the Vibration Waves Level at Different Distances. Lecture Notes in Civil Engineering, 170, Pp. 207–215, (2022). https://doi.org/10.1007/978-3-030-79983-0_19

M. Mirsaidov, K. Mamasoliev,  Contact interaction of multilayer slabs with an inhomogeneous base. Magazine of Civil Engineering, 115(7), 11504, (2022) https://doi.org/10.34910/MCE.115.4

M. Mirsaidov, K. Mamasoliev,  Contact interaction of multi-layer plates with a combined base. AIP Conference Proceedings 2637, 050001 (2022). Doi: 10.1063/5.0118870

D. J. Jurayev, N. Vatin, , T. Z. Sultanov, M. M. Mirsaidov,  Spatial stress-strain state of earth dams. Magazine of Civil Engineering, 118(2), 11810, (2023). https://doi.org/10.34910/MCE.118.10

Sh. Ahmed R.M., M. Haneef, Experimental and numerical analysis of woven glass-epoxy composite laminates with single lap joints. Journal: AIP Conference Proceedings, AIP Conf. Proc. 2399, 030020 (2023),

- https://doi.org/10.1063/5.0157163

S. N. Shastry, S. S. Swamy, The effects of design modification in a two edge coupled thin composite laminates on SEA. AIP Conf. Proc. 2399, 030024 (2023), December 2023, https://doi.org/10.1063/5.0132717

C. D. Avinash, Narasimhe Gowda, N. Smitha, Modal analysis of honeycomb structure for different parametric conditions using finite element method. Journal: AIP Conference Proceedings, AIP Conf. Proc. 2399, 030005 (2023), https://doi.org/10.1063/5.0132658

M. Al Safar, K. Ibraheem, Numerical algorithm for solving fuzzy differential equations using Runge-Kutta method RK6, Journal: AIP Conference Proceedings, AIP Conf. Proc. 3036, 040020 (2024), https://doi.org/10.1063/5.0203150

M. Mirsaidov, K. Mamasoliyev, J. Sindarov, Estimation of flexural deformation of three-layer plates interacting with an elastic half-space. AIP Conf. Proc. 3244, 020056 (2024). https://doi.org/10.1063/5.0242467

M. M. Mirsaidov, N. I. Vatin, K. Mamasoliev. Bending of multilayer beam slabs lying on an elastic half-space. Magazine of Civil Engineering. 17(6). Article No. 13004. (2024), DOI: 10.34910/MCE.130.4

O. Dusmatov, M. Khodjabekov, O. Toshov, Determination of modal mass and stiffness in longitudinal vibrations of the rod. AIP Conf. Proc. 3244, 060023 (2024). https://doi.org/10.1063/5.0241687

Kh. Khudoynazarov, K. Mamasoliyev, E. Ismoilov, Non-stationary influence of a transverse-isotropic cylindrical shell with a viscous compressed fluid/AIP Conf. Proc. 3177, 050005 (2025) https://doi.org/10.1063/5.0294882

K. Mamasoliev, M. Mirsaidov, Mathematical model and analytical solution of the contact problem of bending a slab lying on an inhomogeneous combined base/ AIP Conf. Proc. 3177, 050006 (2025), https://doi.org/10.1063/5.0295291

D. Kholikov; Z. Shukurov; E. Ismoilov; K. Xaydarova, Experimental determination of the intensified deformed state of elastic thin-walled shell/ AIP Conf. Proc. 3177, 050020 (2025)  https://doi.org/10.1063/5.029593.