Assessment of Pile Vibrations in Various Soil Layers

Dilshod Kholikov1, a), Dilfuza Ismatova2, b) Elbek Ismoilov3, c), Kamola Khaydarova3, d)

1Zarmed University Samarkand compus, Samarkand, Uzbekistan

2Samarkand State University of Architecture and Construction, Samarkand, Uzbekistan

3Samarkand State University, 140104, University blv. 15, Samarkand сity, Uzbekistan

a) dilshodxoliqov2587gmail.com

b) ismatovadilfuza12@gmail.com

c) eismoilov.samsu@gmail.com

d) Corresponding author: xaydarovakamolaxakimovna@gmail.com

Abstract. The calculation of pile foundations should be carried out using mathematical models that describe their mechanical behavior. The design scheme can be represented both in analytical and numerical forms. When determining the bearing capacity and settlement of single piles, preference should be given to tabulated or analytical solutions provided in construction standards and regulations. All calculations of piles, pile foundations, and their bases should be performed using the design values of material and soil properties. This paper presents the results of an assessment of the influence of the excitation frequency of the pile foundation base on the amplitudes of transverse vibrations of a pile interacting with a two-layer soil according to the Winkler model.

# Introduction

According to the provisions of SNIP II-7-81 “Construction in Seismic Regions”, the design model of the “pile–soil medium” system [1] must be selected taking into account the most significant factors that determine the stress state and deformations of the foundation and structural elements of the building (the structural scheme of the building, construction features, the nature of soil layers, the properties of foundation soils, and the possibility of their changes during construction and operation, etc.).

When designing pile foundations, the following factors should be taken into account: soil conditions of the construction site, hydrogeological regime, features of pile installation, and the presence of slurry beneath the pile tips. The design values of soil properties should be determined in accordance with GOST 20522, and the design values of the subgrade reaction coefficients of the soil surrounding the pile - in accordance with Appendix [1].

Studies [2-5] present analyses and results of investigations on pile foundations interacting with various types of collapsible soils. The main requirements for pile foundation structures are outlined in manuals [6-8]. The methodology for calculating pile foundations under the influence of seismic waves and kinematic excitations is described in works [9].

In this paper, the vibration process of a pile foundation interacting with the base according to the Winkler model, where the base undergoes kinematic excitation, is considered.

# PROBLEM STATEMENT

Let us set the origin of coordinates at the lower cross-section of the beam and direct the OX axis vertically upward. Let  W1x,t and W2x,t denote the deflections of the beam in each zone (Figure 1), which satisfy the following equations

EJ∂4W1∂x4+m∂2W1∂t2+k1W1=0  for  0<x<l 	 (1)

EJ∂4W2∂x4+m∂2W2∂t2+k2W2=0 for  l<x<L   	(2)

The functions W1x,t and  W2x,t satisfy the following boundary conditions

∂W1∂x=0,  EJ∂3W1∂x3=k0W1-W0  at  x=0  	 (3)

∂2W1∂x2=0,  EJ∂3W1∂x3=M∂2W1∂t2  at  x=L  	(4)

under compatibility conditions

W1=W2,  ∂W1∂x=∂W2∂x,∂2W1∂x2=∂2W2∂x2,  ∂3W1∂x3=∂3W2∂x3   at   x=l    	 (5)

where k0 is the stiffness coefficient of the contact during shear at the lower cross-section of the beam with the base, which moves according to the law W0=W0t, M is the mass of the attached body (pile cap) at the upper cross-section of the beam.

Figure 1. Scheme of the transverse bending of a beam carrying a concentrated mass M. Here, EJ is the flexural rigidity of the beam, m is the mass per unit length, k1 and k2 are the subgrade reaction coefficients for each zone of beam-soil contact, with corresponding lengths l and L-l, and L is the total length of the beam.

We assume W0t=U0sinωt (U0 - amplitude, ω - frequency) and represent the solutions of equations (1) and (2) in the form of

W1=U1(x)sinωt,   W2=U2(x)sinωt

where the functions U1(x) and U2(x) satisfy the equations

EJU1IV+k1-mω2U1=0  0<x<l    	 (6)

EJU2IV+k2-mω2U2=0  l<x<L      	   (7)

boundary

U1'0=0,     EJU1'''0=k0[U10-U0] 	   (8)

U2''L=0,    EJU2'''L=-Mω2U2L  	(9)

matching conditions

U1l=U2l,   U1'l=U2'l, U1''l=U2''l,  U1'''l=U2'''l  	 (10)

# METHOD OF SOLUTION

When performing the calculations, we assume that k1≥4k2

- We consider the case ω<k2/m (Case 1).

The general solution of equations (6) and (7) can be written as:

U1=C1Y1β1*(ξ-l1)+C2Y2β1*(ξ-l1)+C3Y3β1*(ξ-l1)+C4Y4β1*(ξ-l1)

U2=C1S1β2ξ-l1+C2β1β2S2β2ξ-l1+B3β12β22S3β2ξ-l1+B4β13β23S4β2ξ-l1

where w=mω2L4EJ,   β1=4w12-w2,  β2=4w22-w2,    w1=k1L44EJ,       w2=k2L4EJ,    l1=l/L.

Yi(βz), Si(βz) are Krylov functions [5], defined by the formulas:

Y1=cosβ1zchβ1z,   Y2=sinβ1zchβ1z+cosβ1zshβ1z,

Y3=sinβ1zshβ1z,    Y4=sinβ1zchβ1z-cosβ1zshβ1z,

S1=cosβ2zchβ2z,  S2=sinβ2zchβ2z+cosβ2zshβ2z,

S3=sinβ2zshβ2z,    S4=sinβ2zchβ2z-cosβ2zshβ2z.

The constants C1,C2,C3 and C4 are determined from conditions (8)-(10), which yield:

C1=-β02s12d0,   C2=β02s11d0,

C3=C1r11+C2r12 ,  C4=C1r21+C2r22,

s21=a21+r11a23+r21a24,      s22=a22+r12a23+r22a24,

d0=s11s22-s12s21    s11=a11+r11a13+r21a14,      s12=a12+r12a13+r22a14,

r11=b21b14-b11b24d,  r12=b22b14-b12b24d,  d0=b13b24-b23b14

r21=b11b23-b21b13d,      r22=b12b23-b22b13d,

b21=αw2S11,w+b01S31,w,  b22=αw2S21,wβ1β2+b02S41,w,

b23=αw2S31,wβ12β22+b03S21,w,  b22=αw2S41,wβ13β23+b04S11,w,

b01=-2β23,    b02=-4β22β1,    b03=-2β12β2,    b04=4β13,

b11=-β23S31,w,    b12=-β22β1S41,w,    b13=β12β2S11,w,b14=β13S2(1,w),

a21=-2β13Y20,w+β02Y10,w,  a22=-4β13Y30,w+β02Y20,w,

a23=-2β13Y40,w+β02Y3(0,w),  a24=4β13Y10,w+β02Y4(0,w)

a11=-β1Y40,w,      a12=2β1Y10,w,   a13=β1Y20,w,  a14=2β1Y30,w.

# Results

The calculations were carried out for the following parameter values:

E=2∙1011Pа,  a=0.3m,  h=0.3m,  L=5m,  ρ0=45kgm,  М=3000kg,

k1=5∙106Nm2,  k2=106Nm2, k0=5∙105Nm2.

Figure 2 shows the curves of deflection and axial stress σ=Ehd2Udx2 distributions along the beam length ξ=x/L for various values of the frequency ω.

Figure 2. Curves of the distribution of the deflection amplitude U(м) and axial stress σ (MPa) along the beam length ξ=x/L for two values of the ratio l1=lL, at different values of the base vibration frequency ωsек-1: 1-ω=1.75,  2-ω=1.78,  3-ω=1.8:  4-ω=1.82,: 1-ω=1.83

Figure 3. Dependence of the deflection amplitude U(м) on the frequency ωsек-1 at sections х=0 (curve 1), х=l  (curve 2), and x=L (curve 3) for two values of the ratio l1=lL

- Let k2m<ω<k14m  (Case 2).

In this case, the general solution of equations (6) and (7) can be written as:

U1=C1Y1β1∙(ξ-l1)+C2Y2β1∙(ξ-l1)+C3Y3β1∙(ξ-l1)+C4Y4β1∙(x-l1)

U2=C1S1β2ξ-l1+C2β1β2S2β2ξ-l1+B3β12β22S3β2ξ-l1+B4β13β23S4β2ξ-l1

where,  β1=4w12-w2,  β2=4w2-w22.   Yi(βz),   Si(βz) are Krylov functions [5], defined by the following formulas:

Y1=cosβ1zchβ1z,      Y2=sinβ1zchβ1z+cosβ1zshβ1z,

Y3=sinβ1zshβ1z,    Y4=sinβ1zchβ1z-cosβ1zshβ1z,

S1=0.5(chβ2z+cosβ2z),      S2=0.5shβ2z+sinβ2z,

S3=0.5(chβ2z-cosβ2z),    S4=0.5shβ2z-sinβ2z.

The constants C1,C2,C3 and C4 are determined from conditions (8)-(10).

Figure 4 shows the curves of deflection distribution along the beam length ξ=x/L for various values of the frequency ω.

Figure 4. Curves of the deflection U(м) and axial stress σ (MPa) distribution along the beam length ξ = x / L for two values of the ratio l1=lL   at different values of the base vibration frequency ω 1sек:1-ω=2.2, 2-ω=2.3, 3-ω=2.4: 4-ω=2.6,:1-ω=3.2

Figure 5. Dependence of the deflection amplitude on the frequency ω1sек  at sections х=0 (curve 1),

х=l (curve 2), and x=L (curve 3) for two values of the ratio l1=lL.

- Now consider the case where the external excitation frequency satisfies the inequality ω>k14m    (Case 3).

In this case, the general solution of equations (6) and (7) can be written as:

U1=C1Y1β1*(ξ-l1)+C2Y2β1*(ξ-l1)+C3Y3β1*(ξ-l1)+C4Y4β1*(ξ-l1)

U2=C1S1β2ξ-l1+C2β1β2S2β2ξ-l1+B3β12β22S3β2ξ-l1+B4β13β23S4β2ξ-l1

where,  β1=4w2-w12,  β2=4w2-w22,

Yi(βz), Si(βz) are Krylov functions [5], defined by the formulas:

Y1=0.5*(chβ1z+cosβ1z),      Y2=0.5*shβ1z+sinβ1z,

Y3=0.5*(chβ1z-cosβ1z),    Y4=0.5*shβ1z-sinβ1z.

S1=0.5*(chβ2z+cosβ2z),      S2=0.5*shβ2z+sinβ2z,

S3=0.5*(chβ2z-cosβ2z),    S4=0.5*shβ2z-sinβ2z.

The constants C1,C2,C3 and C4 are determined from conditions (8)-(10).

Figure 6 shows the curves of deflection distribution along the beam length for various values of the frequency ω.

Figure 6. Curves of the deflection amplitude U(м) and axial stress σ (MPa) along the beam length ξ=x/L for two values of the ratio l1=lL at different base vibration frequencies ω1sек:1-ω=4, 2-ω=4.2, 3-ω=4.5: 4-ω=4.6,:1-ω=5

Figure 7. Dependence of the deflection amplitude on the frequency ωsек-1 at sections х=0 (curve 1), х=l (curve 2), and x=L (curve 3) for two values of the ratio l1=lL.

# conclusion

Analysis of the results shows a significant influence of external excitation frequencies close to resonance on the magnitudes of beam deflections and axial stresses. At low frequencies (Figure 2), near the resonance frequency 1.72<ω<2, the beam deflections along its length increase monotonically and reach high values as the frequency approaches resonance. In this case, the tensile stresses attain their maximum values at the lower section of the beam.

For frequencies in the range k2m<ω<k14m (Case 2), the pattern of deflections and stresses changes: the deflections of the upper section can become negative, and the entire length of the beam is under compression.

Finally, at high values of the external excitation frequency (Figure 6, Case 3), the beam also remains in compression, with deflections along the entire length being practically negative, and the largest deflections occurring at the section where the mass is attached. Increasing the length of the contact zone l between the two soil conditions leads to a reduction in both deflections and stress values along the beam.

# References

Pile Foundations. Updated Edition of SNiP 2.02.03-85. Official Edition. Moscow, (2011).

A.A. Grigoryan, Pile Foundations of Buildings and Structures on Collapsible Soils, Moscow, Stroyizdat, (1984).

V.A. Ilyichev, Geotechnical Handbook: Foundations, Substructures, and Underground Structures. Moscow, ASV Publishing, (2014).

A.T. Iovchuk, On the Use of Piles in Soils with Weak Layers, Foundations and Mechanics of Soils, 2, 32–36 (1967).

R.A. Mangushev, Modern Pile Technologies: Textbook, 2nd edition, 2010.

Guidelines for Designing Pile Foundations of Buildings and Structures on Collapsible Soils. Moscow, Stroyizdat, (1969).

R.A. Mangushev, Piles and Pile Foundations: Structures, Design, and Technologies, Moscow, ASV, (2015).

Guidelines for Designing Pile Foundations. Moscow, NIIOSP named after N.M. Gersevanov, (1980).

A.G. Bulgakov, Formulation and Solution of the Problem of Interaction of Building Structures on Pile Foundations with Seismic Waves, Theory of Engineering Structures, Building Constructions, 2, 3–13 (2014).