Investigation of drilling fluid criteria and parameters determining the effectiveness of cuttings removal during the transition from laminar to turbulent flow regimes in the geological conditions

Umedov Sherali1, Komilov Tolib1,4, a), Begijonov Ilhom, Rakhimov Anvarkhodzha1, Shukurov Sirojiddin1, Eshmurodov Otabek2, Shomurodov Shokhboz2, Sanetullaev Yernazar3, Eshmuratov Anvar3, Ruzmanov Farxod 1, Rakhimov Komilkhodzha1

1 Tashkent state technical university named after Islam Karimov, Tashkent, Uzbekistan

2Karshi state technical university, Karshi, Uzbekistan

3Karakalpak State University, Nukus, Uzbekistan

Termez State University of Engineering and Agrotechnology, Termez, Uzbekistan

a) Corresponding author: komilovtolib87@yandex.ru

Abstract. This article focuses on the research and mathematical substantiation of drilling fluid criteria and parameters that ensure effective cuttings removal during well drilling in the geological conditions of Uzbekistan. The main attention is given to the conditions for the transition of drilling fluid flow from laminar to turbulent regime. The work involves constructing a mathematical model to optimize drilling fluid parameters. The first condition concerns selecting the density of the drilling fluid, which should prevent the inflow of formation fluids while simultaneously preventing formation fracturing. For this purpose, calculation inequalities (2) and (3) are formulated. The second condition defines the rheological parameters necessary to ensure a turbulent flow regime. Based on the analysis of the Reynolds number, which depends on the Hedstrom number, criterion constraints for kinematic viscosity were obtained. As a result of combining the density and viscosity criteria, along with the hydrodynamic conditions of drilling tool movement in (12), a complete mathematical model for the geological conditions of Uzbekistan is obtained in (13). This model serves as the basis for selecting drilling fluid parameters to increase the mechanical drilling rate while maintaining wellbore stability conditions.

INTRODUCTION

The efficiency of the drilling process directly depends on the hydrodynamic parameters and rheological properties of the drilling fluid, with the key aspect being the provision of optimal flow regime for cuttings removal [1]. This research is based on principles developed by leading specialists in the field of drilling and hydrodynamics [2-3].

Classical works in the field of drilling, presented in the studies of A. I. Bulatov, Yu. M. Proselkov, and S. A. Shamanov, laid the foundation for determining the main technological parameters. In particular, these works formulated the basic conditions for ensuring borehole cleaning, defining the minimum required value of the volumetric flow rate Q (formula (1)), as well as methods for calculating the total delivery of drilling pumps.

A key aspect in assessing pressure losses and cuttings removal efficiency is determining the flow regime (laminar or turbulent). According to generally accepted approaches, the transition from structural (laminar) to turbulent flow for viscoplastic fluids is determined by the critical Reynolds number. The formula, which takes into account the Hedstrom number, is also provided in classical sources.

The issues of selecting the drilling fluid density to ensure wellbore stability and prevent emergency situations are fundamental. The literature defines two antagonistic conditions:

- creating a back pressure that prevents the inflow of formation fluids (formula (2)).

- preventing formation fracturing (formula (3)).

While classical approaches provide recommendations for selecting safety factors and differential pressure, this work details and evaluates the minimum density value depending on the formation pressure and depth, which allows for more accurate application of formula (2) in practice.

It is worth noting the contributions of Zh. A. Akilov and M. S. Dzhabbarov, whose works on mathematical modeling of hydrodynamic processes include the pressure balance equation during the movement of drilling tools. This equation (12) is used in the current work to derive inequality (12), which limits the speed of the tool's movement and, consequently, prevents hydraulic fracturing.

The literature acknowledges that numerous rheological conditions for drilling fluids have been developed for various situations. In particular, standardized methods are used to determine rheological parameters:

Kinematic viscosity determination is performed in accordance with GOST 33768-2015, which ensures the accuracy and standardization of the initial data.

The calculation of dynamic yield stress and plastic viscosity is based on the methods described in the works of V. P. Ovchinnikov and N. A. Aksenova, using viscometer readings (FANN and VSN-3).

A criterion inequality (11) has been derived, which relates kinematic viscosity to the wellbore geometric parameters, density, and dynamic yield stress. This inequality guarantees a turbulent flow regime in the annular space, provided that the minimum flow rate required to clean the borehole is met.

A complete mathematical model (13) has been created, which combines rheological requirements for turbulence (inequality (11)), density criteria for preventing hydraulic fracturing and inflow (conditions (2) and (3)), and dynamic constraints on tool movement speed (inequality (12)).

Thus, the work not only relies on fundamental knowledge but also develops it by creating a comprehensive apparatus for the rapid selection of drilling fluid parameters, specifically adapted to improve drilling efficiency in the geological conditions of Uzbekistan.

###### EXPERIMENTAL RESEARCH

To ensure the cleaning of the borehole bottom from cuttings, the value of the volumetric flow rate Q is determined, which is then verified by checking the following condition [4]:

Q≥πdс24a                                                                                           (1)

where a ∈ [0.35; 0.5] m/s for rotary drilling and electric drilling; a ∈ [0.5; 0.7] m/s for drilling with hydraulic downhole motors.

For the flow rate value Q, the diameters of the cylinder liners of the drilling pump are selected from reference tables, focusing on cuttings removal. The total pump output is determined [4] by the formula:

Q=mnQn

where m is the filling coefficient; n is the number of pumps; Qn is the pump flow rate at a given bushing diameter, m3/s.

The coefficient m is selected depending on the suction conditions of the liquids. When there is positive suction head, m=1. If suction is carried out from containers in the soil during water flushing, m=0.9, and with a clay solution, m=0.8 [4].

When selecting the density of the drilling fluid used during the drilling of a given interval, it is necessary to consider the following two conditions: creating a back pressure that prevents the inflow of formation fluids into the well and preventing hydraulic fracturing [4].

The first condition has the form

ρ=minkрpплgLк, pпл+∆pрgLк                                                                   (2)

where ρ is the density of the drilling fluid, kg/m3; kр is the safety factor; pпл is the formation pressure, Pa; g is the acceleration due to gravity, m/s2; Lк is the depth to the top of the formation with the maximum formation pressure gradient, m; ∆pрis the pressure loss.

According to existing guidelines, the following values are recommended for the formation counterpressure, expressed through the safety factor kр and the differential pressure ∆pр:

kр∈1,1;1,15 ; ∆pр=1,5 МPа at Lк<1 200 м;

kр∈1,05;1,1 ; ∆pр=2,5 МPа at  1 200 м<Lк≤2 500 м;

kр∈1,04;1,07 ; ∆pр=3,5 МPа at Lк>2 500 м.

Thanks to the authors [3-5], a variety of conditions for the rheology of drilling fluid under different circumstances have been developed.

The density ρ calculated using formula (2) must be checked for compliance with the second condition, which stipulates that the pressure of the drilling fluid in the annular space opposite each formation must be less than the fracturing pressure of that formation. The second condition is expressed as follows [1]:

ρ<pг-∆pкп-(1-φ)ρшgLпφgLп                                                                  (3)

where pг is the formation fracturing (loss) pressure, Pa; ∆pкпis the pressure loss during the circulation of drilling fluid in the annular space along the path from the bottom of the considered formation to the wellhead, Pa; φ=Qπvмdс24+Q- luid content in the slurry flow without considering relative velocities; ρш - slurry density, kg/m3; Lп - depth of occurrence of the bottom of the considered layer, m; vм - mechanical drilling speed, m/s.

From condition (2), we compare and evaluate the minimum value of ρ:

kрpплgLк ˅ pпл+∆pрgLк  => kрpпл ˅ pпл+∆pр

from here

kр-1 ˅ ∆pрpпл

RESEARCH RESULTS

When Lk > 2500 m, if the reservoir pressure is less than 50 MPa, then

ρ=kрpплgLk

If the reservoir pressure exceeds 87.5 MPa, then

ρ=pпл+∆pрgLк

At 1200 m < Lk≤ 2500 m, if the reservoir pressure is less than 25 MPa, then

ρ=kрpплgLк

If the formation pressure exceeds 50 MPa, then

ρ=pпл+∆pрgLк

When Lk < 1200 m, if the reservoir pressure is less than 10 MPa, then

ρ=kрpплgLк

If the reservoir pressure exceeds 15 MPa, then

ρ=pпл+∆pрgLк

Otherwise, the required density of the drilling fluid is determined by calculations based on conditions (2) and (3).

Since the values of ∆pкп and φ varphi depend on the flow rate of the washing fluid, the second condition can only be verified after establishing the pump output. To calculate pressure losses due to friction during the movement of washing fluid without cuttings in pipes and the annular space, it is necessary to determine the flow regime, which dictates the selection of appropriate calculation formulas. For this purpose, the critical Reynolds number Reкр for the washing fluid flow is calculated, at which the transition from laminar to turbulent flow occurs.

This number for viscoplastic fluids is determined from the relationship [4].

Reкр=2 100+7,3He0,58

Where He=ρdг2τ0μ2- Hedstrom number; μ - plastic (dynamic) viscosity of the drilling fluid, Pa·s; τ0 - dynamic yield stress, Pa. When fluid flows inside the drill string, the value of dг is taken as equal to the inner diameter of the drill pipes dт. In the annular space, dг is defined as the difference between the wellbore diameter dс and the outer diameter of the drill pipes dn [4].

If the Reynolds number of fluid flow in pipes ReT or in an annular space Reкп exceeds the calculated critical value Reкр, then the flow regime is turbulent. Otherwise, the flow occurs in the structural regime. [4]

The values of ReT are calculated using the following formulas:

Reт=4ρQπdтμ

The values of Reкп are calculated using the following formulas:

Reкп=Qdгπ(dc2-dn2)4ν=4Qdс-dnπ(dc2-dn2)ν=4Qdс-dnπdс-dndс+dnν=4Qπ(dс+dn)ν

Comparing Reкпand ReT:

Reкп ˅ Reт

by substituting the corresponding values, we get

4Qπ(dс+dn)ν ˅ 4ρQπdтμ

Setting ν=μρ, we obtain

4Qπ(dс+dn)ν ˅ 4Qπdтν

simplified form

dс+dn ˅ dт

The difference between dт-dnis much smaller than dс; therefore,

dс+dn > dт

From this it follows that the following relationship is always valid:

Reкп < Reт

Means that the Reynolds number for fluid flow in pipes is always greater than the Reynolds number for fluid flow in the annular space.

If the Reynolds number of fluid flow in the annular space is greater than the calculated critical value Reкр, then the fluid flow regime in the annular space is turbulent.

From the above, it follows that to ensure a turbulent flow regime of the fluid in the annular space, it is necessary to select a drilling fluid that satisfies the following condition:

Reкп>Reкр    =>      4Qπ(dс+dn)ν>2 100+7,3He0,58

Q>πνdс+dn2 100+7,3He0,584                                                                       (4)

From formula (1) and the values of a, we always obtain:

Q≥7πdс280                                                                                                (5)

by combining the formulas (4) and (5), we obtain

7πdс280>πνdс+dn2 100+7,3He0,584

hence, it follows that according to the conditions for cleaning the bottomhole from cuttings (sm the above-mentioned relationship)

ν2 100+7,3He0,58<7dс220dс+dn                                                                 (6)

Reкп≥2 300 represents the transition period to turbulent flow regime. Therefore

Reкп=4ρQπ(dс+dn)μ≥2 300

and

4Qπ(dс+dn)ν>2 100+7,3He0,58

must correspond to each other. From the properties of the inequality it is known that

2 300>2 100+7,3He0,58

from where

He0,58<2007,3                                                                                        (7)

By combining formulas (6) and (7), we obtain the conditions for turbulent flow of washing fluids in the annulus space for cleaning the bottomhole from sludge:

He0,58<7dс2146νdс+dn-2 1007,3He0,58<2007,3

It is known that

7dс2146νdс+dn-2 1007,3>2007,3

from which we have that the kinematic viscosity must always be less than the values determined by formula (8):

ν<152,1739∙10-6dс2dс+dn                                                                                   (8)

According to GOST 33768-2015, the kinematic viscosity ν, m2s is calculated using the formula [2]:

ν=CtggH-∆ν

where C- viscometer constant,m2s2; t- flow time, s; gH=9,80665 m2s ;

g=9,7803181+0,0053024sin2φ-0,0000059sin22φ-2∙10-6h

φ - the geographical latitude of the place; h- the height above sea level, m;

∆ν=0,0166V32LCD12t2

V - capacity of the measuring tank, m3; L - length of the capillary, m; D - diameter of the capillary.

From

ν=1+0,0053024sin2φ-0,0000059sin22φ-2,0449∙10-7h1,0026923Ct-0,0166V32LCD12t2

According to passport data SPV-5

V=0,7 л=7∙10-4 м3 , L=0,1 м , D=0,0005 м

At 20°C ± 5°C:

C=15∙10-6м2с2

In Uzbekistan, the geographical latitude and altitude above sea level (intermediate values) have values within:

37°≤φ≤45°, 12,8 м≤h≤4668 м.

After the introduction of the designation

K=14,95972∙10-61+0,0053024sin2φ-0,0000059sin22φ-2,0449∙10-7h

we have

ν=Kt-0,03549t2                                                        (9)

where K is the correction coefficient for the measurement location. It is known that kinematic viscosity has no negative value, i.e.:

Kt-0,03549t2>0

The liquid flow time from SPV-5 will always be greater than:

30,03549K

Substituting He0,58=ρdг2τ0μ20,58 (7), we obtain:

ρdг2τ0μ20,58< 2007,3

It is known that dг=dс-dn и μ=νρ (when performing calculations, it is necessary to take into account the units of measurement).

ρdс-dn2τ0νρ2<2007,35029

Simplifying the inequalities, we have

ν>0,0576dс-dnτ0ρ                                                                                (10)

When selecting the kinematic viscosity of the washing fluid that ensures a turbulent flow regime of the fluid in the outer space for cleaning the bottomhole from sludge, it is necessary to take into account that its value should be in the range:

0,0576dс-dnτ0ρ<ν<152,1739∙10-6dс2dс+dn                                                       (11)

After determining the dynamic shear stress (τ0, dPa), it becomes possible to evaluate drilling fluids (in subsequent calculations, it should be noted that 1 dPa = 10 Pa).

The authors [5] assert that in the FANN viscometer, the dynamic shear stress (τ0) in dPa is calculated using the formula:

τ0=4,8φ300-μ

Accordingly, plastic viscosity

μ=φ600-φ300

where φ600 and φ300 are the values of the rotation angles on the viscometer scale at sleeve rotation frequencies of 600 and 300 min-1, respectively, measured in degrees.

By substituting τ0 into inequality (10), we obtain a quadratic inequality

ν2+0,0159dс-dn2ν-0,0159φ300ρdс-dn2>0

Solving the quadratic inequality, we have:

ν>dс-dn0,00025∙dс-dn2+0,0636φ300ρ-0,0159dс-dn22

Introductory designation

KFANN=dс-dn0,00025∙dс-dn2+0,0636φ300ρ-0,0159dс-dn22

will lead to

ν>KFANN

The authors [5] state that in the VSN-3 viscometer, the dynamic shear stress (τ0) in dPa is calculated using the formula:

when using a rotation frequency of 600 and 300 min-1, deg.:

τ0=3φ1-μ

Substituting τ0into inequality (10), we obtain a quadratic inequality

ν2+0,00995dс-dn2ν-0,00995φ1ρdс-dn2>0

Solving the quadratic inequality, we have:

ν>dс-dn0,000099∙dс-dn2+0,038φ1ρ-0,00995dс-dn22

introduction, designation

KВСN-3.1=dс-dn0,000099∙dс-dn2+0,038φ1ρ-0,00995dс-dn22

will lead to

ν>KВСN-3.1

→ when using rotation speeds of 400 and 200 min-1, degrees:

τ0=3φ1-2μ

By substituting τ0 into inequality (10), we obtain a quadratic inequality

ν2+0,00663dс-dn2ν-0,00995φ1ρdс-dn2>0

Solving the quadratic inequality, we have:

ν>dс-dn0,000044dс-dn2+0,0398φ1ρ-0,05762dс-dn22

introduction, designation

KВСN-3.2=dс-dn0,000044dс-dn2+0,0398φ1ρ-0,05762dс-dn22

will lead to

ν>KВСN-3.2

To prevent absorption and hydraulic fracturing, it is necessary to combine (3) with the obtained results. Equation of pressure balance during the movement of drilling tools in the wellbore, taking into account the stability of rocks in the wellbore [8]:

P0g∙dvdt+P0+∆P≤σ

where P0 is the hydrostatic pressure, Pa; σ is the formation's fracture strength limit, Pa; λ is the coefficient of hydraulic resistance in the annular space; g is the acceleration of gravity, ms2; v is the average fluid velocity in the annular space, ms;

∆P=P0λv22g(dс-dn)

from where

P0gdvdt+P0+P0λv22g(dс-dn)≤σ

Using the properties of the differential, we have

dvσgP0-g-λv22(dс-dn)≤dt

Let's integrate the result:

2(dс-dn)λdv2g(dс-dn)σ-P0P0λ-v2g(dс-dn)σ-P0P0λ+v≤dt

and let

2g(dс-dn)σ-P0P0λ=A

then we have

(dс-dn)AλlnA+v-lnA-v≤t

from where

v≤AeAλt(dс-dn)-1eAλt(dс-dn)+1

or

v≤2gdс-dnσ-P0P0λ∙e2gλσ-P0P0dс-dnt-1e2gλσ-P0P0dс-dnt+1                                                     (12)

The combination of the model given in [8] (or (12)) and condition (3) represents a mathematical model for preventing water rupture.

ρ<P0-∆Pкп-(1-φ)ρшgLпφgLпv≤2gdс-dnσ-P0P0λ∙e2gλσ-P0P0dс-dnt-1e2gλσ-P0P0dс-dnt+1

Combining the hydraulic fracture model with the obtained results will allow for the construction of a complete mathematical model of the criteria and parameters of the flushing fluid to ensure the turbulent flow regime of the fluid in the annulus space.

Kприбор.<Kt-0,03549t2<152,1739∙10-6dс2dс+dnρ<P0-∆Pкп-(1-φ)ρшgLпφgLпv≤2gdс-dnσ-P0P0λ∙e2gλσ-P0P0dс-dnt-1e2gλσ-P0P0dс-dnt+1                                              (13)

CONCLUSIONS

This study examined and mathematically formulated the criteria and parameters of drilling fluid necessary to ensure effective cuttings removal during drilling operations in the geological conditions of Uzbekistan, with a focus on the transition from laminar to turbulent flow regime in the annular space. The main results and conclusions obtained in this work are as follows: The condition for determining the minimum density of drilling fluid has been refined based on two key requirements: preventing the inflow of formation fluids and preventing formation fracturing. Numerical boundaries were determined for selecting the calculation formula (2) depending on the depth of the formation top and formation pressure. Conditions for the transition to turbulent flow regime for viscoplastic fluids in the annular space have been established. A comparative analysis of Reynolds numbers in pipes and in the annular space showed that Reкп < Reт, and, consequently, the condition for ensuring a turbulent regime is

Reкп>Reкр

the key inequality for kinematic viscosity was obtained, which ensures a turbulent flow regime in the annulus space depending on the density, dynamic shear stress, and the well's geometric parameters (11). a complete mathematical model (13) has been developed, which combines:

- geological and technological criteria for selecting the density of the washing fluid;

- rheological criteria for ensuring turbulent flow regime;

- conditions for preventing hydraulic fracturing during the movement of drilling tools.

The proposed criterion inequalities allow for the prompt and justified selection of optimal parameters of the flushing fluid, which is an important condition for increasing the mechanical speed of drilling and reducing accidents in the complex mining and geological conditions of Uzbekistan.

REFERENCES

- T. O. Komilov and D. R. Makhamatkhodzhaev, “Improved composition of a drilling fluid to prevent the absorption of movable liquids,” Oil Industry 1167, 68–74 (2021). https://doi.org/10.24887/0028-2448-2021-2-68-72

- Sh. K. Umedov, T. O. Komilov, and E. E. Sanetullaev, “Research of features of structure and components of flushing liquids,” in Proc. II Int. Sci.-Pract. Conf. “Bulatov Readings”, Vol. 7 (2018), pp. 315–317.

- T. O. Komilov, F. I. Ruzmanov, A. A. Raximov, and Sh. Kh. Umedov, “Ways to create a quality wellbore for smooth casing flow,” PalArch’s J. Archaeol. Egypt/Egyptol. 17(6), 3520–3523 (2020).

- A. I. Bulatov, Yu. M. Proselkov, and S. A. Shamanov, Oil and Gas Well Drilling Equipment and Technology (Nedra–Business Center, Moscow, 2003).

- State Standard, Method for Determining Kinematic Viscosity and Calculating the Dynamic Viscosity of Transparent and Opaque Liquids (Standardinform, Moscow, 2016).

- V. P. Ovchinnikov and N. A. Aksenova, Drilling Flushing Fluids (Tyumen, 2008).

- V. P. Zozulya, N. E. Zozulya, and A. M. Magrupov, Well Flushing (Department of Drilling of Oil and Gas Wells, Tashkent, 2021).

- J. A. Akilov and M. S. Jabbarov, Mathematical Models of Hydrodynamic Processes of Oil and Gas Production (Samarkand, 2023).