Impact of Thread Density on Quality of Double-Layer Knitted Fabrics

Elyor Jurakulov 1, a), Kamoliddin Yunusov 2, Odil Toshbekov1, Baymurova Nigora1, Olimjon Kulmuminov 3, Urozov Mustafokul 3

1Termez State University, Termez, Uzbekistan

2Tashkent Insititute of Textile and Light Industry, Tashkent, Uzbekistan

3Termez State University of Engineering and Agrotechnology Termez, Uzbekistan

a) Corresponding author: elyor_jurakulov90@mail.ru

Abstract. This study investigates how the linear density of polyester threads connecting the layers of double-layer knitted fabrics affects their quality indicators. Two-layer knitted fabrics were produced using round needle knitting machines with varying linear dencities of connecting threads. The research aims to optimize the quality of these fabrics by examining indicators such as breathability, stiffness, extensibility, and abrasion resistance. The findings indicate that using a polyester thread with a linear dencity of 7,7 tex results in improved quality characteristics compared to using a 11,1 tex thread. This study provides insight into enhancing the performance of double-layer knitted fabrics by adjusting the linear dencity of connecting threads.

INTRODUCTION

Analysis of the quality indicators of knitted fabrics allows not only to study the types of fabrics obtained, but also to create various knitted fabrics with a new structure and expand the range of knitted fabrics and products [1].

In order to reduce the consumption of raw materials per unit of production, improve the quality indicators of knitted fabric and expand the range of knitted fabrics, three variants of the structure of a two-layer knitted fabric and the method of its production were developed [2].

[3] the work has developed a method for producing a two-layer knitted fabric, consisting of a rainbow element of fabric, the layers of which are fastened with lycra thread, on a circular knitting two-needle rotary machine of class 10.

A method for producing a two-layer knitted fabric for fire-resistant natural children's clothing is presented in [4]. Bamboo and modacrylic threads were used to connect the layers. According to experimental analysis, the percentage of modacrylic yarn in knitted fabric is 40-60%, which increases its fire resistance.

In order to improve the heat-retaining properties of a two-layer knitted fabric, [5] created a method for producing a two-layer fleece knitted fabric with a new structure, in which each independent layer of a two-layer knitted fabric is formed into fleece fabric and the layers are connected using an additional connecting thread.

In order to expand the range of knitted products, a method for producing lining knitted fabric based on a two-layer fabric is presented in [6]. The front and back layers of the jersey are woven from the rainbow element of the fabric. Highly elastic lycra with a linear density of 7.7 tex was used as a connecting thread to join the layers.

Aegean University scientists N. Oglakcioglu, A. Chai and B. Sarilar studied the moisture management and drying properties of two-layer knitted fabric [7]. The research work examined the influence of different types of yarn on the moisture permeability and drying parameters of two-layer knitted fabrics.

In [8] the structure and methods of manufacturing a two-layer knitted fabric with longitudinal weaving are presented. The canvas with a new structure can be used for medicine, everyday life and technical purposes. Two-layer knitwear consists of an outer and an inner layer connected by an elastic thread.

The results of the influence of yarn type on moisture permeability for a number of two-layer knitted fabrics made of cotton, polyester, acrylic, polypropylene, nylon threads used in the production of sportswear are highlighted in [9].

EXPERIMENTAL RESEARCH

To create a new knitted fabric, a round needle knitting machine was used and a project for a new knitted fabric was developed [10]. As a result of the analysis of the scientific work carried out, it became known that the influence of the linear density of the connecting thread on the quality indicators of two-layer knitted fabrics has not been studied.

In order to improve the quality of knitting, the technological capabilities of binding and knitting machines with circular needles were used [11], two-layer knitted fabrics with different linear densities of the connecting thread belong to the same class 22 with high performance characteristics. The fabric was woven on a knitting machine and quality indicators were analyzed (Fig 1).

|  |  | Place of production; Korea. Model name, SD-R2/SD-I4; SD-I4. Inch, 30~36 dyum; 36. Gauge, 18~42; 22. Feeder, 96~108 dona; 96. interlock/rib; Interlok. Number of needles; 2. Voltage frequency, V; 380 V/50~60 Hz. |
| --- | --- | --- |

- b)

FIGURE 1. a) Interlock knitting machine. b) technical classification of the vehicle

The technology of knitting samples on a circular two-needle interlock machine is presented (Fig 2.).

In system I, IV, the satin fabric element is knitted on knitting needles 1, 2, 3, 4, 5, 6, 7, 8 of a cylindrical needle and forms the front loops of the knitted fabric. In the II, V-system, on knitting needles 1', 2', 3', 4', 5', 6', 7', 8', an element of the satin fabric is knitted and the purl loops of the knitted fabric are knitted. In system III, the connecting thread forms half-loops on knitting needles 1,3,5,7 of a cylindrical needle and on knitting needles 2',4',6',8' of a disk needle [18-40]. In the VI-system, the connecting thread, in contrast to the III-system, forms half-loops in knitting needles 2,4,6,8 of a cylindrical needle and in knitting needles 1',3',5',7' of a disk needle (Fig 2a).

Samples I and II differ from each other in the linear density of the connecting thread used to connect the layers (system III, VI) (fig. 2b). As a connecting thread for sample I, a polyester thread with a density of 11.1 tex (white) and 7.7 tex (black) was used, and for sample II 7.7 tex (black).

In the analyzed samples, it is important to determine the influence of the linear density of the connecting thread on the quality indicators of two-layer knitwear. Indicators such as breathability, stiffness, extensibility, penetration and abrasion resistance can provide information about the quality of knitwear and its functional use [12].

| a) |  | b) |
| --- | --- | --- |

FIGURE 2. a) graphic symbol for knitting b) double layer knitted fabric.

RESEARCH RESULTS

It is especially important to compare the quality indicators of knitted products obtained by changing the linear density [13] of the binding thread. The quality indicators of knitted samples were determined and the results are summarized in Table 1. [14]

TABLE 1. The quality indicators of knitted samples were determined and the results

| № | Raw materials type, linear density |  | Surface density, МS, g/m2 | Thickness, T | Tensile strength, Р, N |  | Irreversible deformation, εн, % |  | Tissue entry U, % |  | Abrasion resistance, a thousand rounds |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | high | width | high | width | high | width |  |
| I | Cotton 18 tex | Polyester 11,1 tex | 203,4 | 0,75 | 312,3 | 256 | 22 | 27 | 7 | 7 | 18,5 |
| II | Cotton 18 tex | Polyester 7,7 tex | 221,2 | 0,75 | 330,8 | 292,5 | 21,1 | 18,6 | 1,5 | 2 | 25 |

The linear density of the threads connecting a two-layer knitted fabric causes a change in its quality indicators, that is, the surface density of the knitted fabric [15] in sample I was 203.4 g/m2, and in sample II - 221.2 g/m2. At the same time, the volumetric density of knitting also increases, the reason for this is that the linear density of the connecting thread in sample [16] II is 7.7 tex. Since in the process of joining the layers, ring spacers bend 7.7 tex yarn more than 11.1 tex polyester yarn, which increases the surface and volumetric density of the knitted fabric, reduces air permeability, which improves heat-retaining properties [17].

The strength of a two-layer knitted fabric varies from 312.3 N to 330.8 N along the length and from 256 N to 292.5 N across the width in samples I and II. At the same time, the indicator of irreversible deformation of the knitted fabric, fabric impregnation and resistance to friction are significantly improved in sample II compared to sample I [18]

According to the results of the analytical comparison, it is advisable to use 7.7 tex polyester thread for knitting this two-layer knitted fabric [19].

CONCLUSIONS

It has been established that it is possible to change and improve the quality characteristics of a two-layer knitted fabric by changing the linear density of the connecting thread.

In the production of two-layer knitted fabric obtained on the basis of Glad, due to changes in the linear density of the connecting thread, an expansion of the range of fabric types for lightweight outer knitted products with improved quality indicators was achieved.

REFERENCES

- I. I. Shalov, A. S. Dalidovich, and L. A. Kudryavin, Knitting Technology (Light and Food Industry, Moscow, 1984), pp. 89–98 (in Russian).

- Sh. Usmankulov, M. M. Mukimov, and K. Yunusov, “Study of the influence of the fabric structure of two-layer knitted layers on its parameters and physical and mechanical properties,” Textile Problems 3, 50–59 (2015).

- T. K. Allamuratova, M. M. Mukimov, and K. M. Kholikov, “Features of the production of two-layer knitwear on a circular machine,” Scientific and Technical Journal of FerPI 22(4), 28–31 (2018).

- M. E. Sarwar, M. S. Anas, and H. Jamshaid, “Engineering an eco-friendly hybrid bi-layer fabric having inherently flame-resistant antibacterial characteristics for children’s protective clothing,” Thermal Science and Engineering Progress 49, 102489 (2024). https://doi.org/10.1016/j.tsep.2024.102489

- Sh. Usmonkulov and M. Mukimov, Method for Producing Two-Layer Plush Knitwear, Uzbekistan Patent No. IAP 05269 (application December 26, 2013; published September 30, 2016).

- Sh. K. Usmonkulov and Kh. A. Khazratkulov, “Technology for producing lined two-layer knitwear,” European Applied Sciences 3, 63–67 (2015).

- N. Oglakeioglu, A. Cay, and B. Sari, “Moisture management and drying properties of double-face knitted fabrics,” IOP Conference Series: Materials Science and Engineering 459, 012056 (2018).

- A. S. Shapalina and A. D. Ostopenko, Two-Layer Warp Knitted Fabric, Russian Patent No. 2172366 (application July 21, 2000; published August 20, 2001).

- B. S. Babu, P. Senthil Kumar, and M. Senthil Kumar, “Effect of yarn type on moisture transfer characteristics of double-face knitted fabrics for active sportswear,” Journal of Industrial Textiles 49(8), 1078–1099 (2020).

- B. Qoraboyev, E. Jurakulov, and K. Yunusov, “Creation of a new type of single-jersey knitted fabrics on single-bed knitting machines,” AIP Conference Proceedings 2789, 040004 (2023). https://doi.org/10.1063/5.0145458

- E. Jurakulov, K. Yunusov, and B. Qoraboyev, “Technology for the production of two-layer knitwear on interlock machines,” Universe: Technical Sciences 12(105), 64–67 (2022). https://doi.org/10.32743/UniTech.2022.105.12.14715

- K. Z. Yunusov, E. N. Dzhurakulov, and B. A. Paulanov, “Analysis of the physical and mechanical properties of knitted fabric obtained from yarn and non-woven fibers,” Scientific and Technical Journal of Fergana Polytechnic Institute 27(4), 79–82 (2023).

- K. Jumaniyozov, M. Urozov, O. Toshbekov, M. Salimova, K. Raximova, and B. Khursandova, “Enhancement of energy-efficient cleaning equipment,” AIP Conference Proceedings 3331, 050007 (2025). https://doi.org/10.1063/5.0307149

- O. Toshbekov, M. Urozov, F. Sultonova, S. Raximqulova, Z. Mustanova, and G. Xulkaliyeva, “Analysis of the thermal conductivity of nonwoven fabrics made from silkworm cocoons and their influence on ambient temperature,” AIP Conference Proceedings 3331, 050005 (2025). https://doi.org/10.1063/5.0306350

- A. Kurbanov, M. Shomirzaev, S. Tursunov, O. Toshbekov, N. Mukhamadieva, B. Kambarov, and S. Mannobova, “Analysis of the state of cultivation and harvesting of mung bean and agro-biological requirements for threshing and separating its grain,” BIO Web of Conferences 105, 02010 (2024).

- T. O. Abdullayevich and S. M. A. Qizi, “Types and application areas of nonwoven fabrics obtained from coarse fibers,” Eurasian Journal of Academic Research 4(12–2), 12–17 (2024) (in Uzbek).

- T. O. Abdullayevich and S. M. A. Qizi, “Prospects for the development of technology for the production of waterproof nonwoven materials,” Eurasian Journal of Academic Research 4(12–2), 7–11 (2024) (in Uzbek).

- Safarov J., Khujakulov A., Sultanova Sh., Khujakulov U., Sunil Verma. Research on energy efficient kinetics of drying raw material. // E3S Web of Conferences: Rudenko International Conference “Methodological problems in reliability study of large energy systems” (RSES 2020). Vol. 216, 2020. P.1-5.  doi.org/10.1051/e3sconf/202021601093

- Safarov J., Sultanova Sh., Dadayev G.T., Zulponov Sh.U. Influence of the structure of coolant flows on the temperature profile by phases in a water heating dryer. // IOP Conf. Series: Materials Science and Engineering. Dynamics of Technical Systems (DTS 2020). Vol.1029, 2021. №012019. P.1-11. doi:10.1088/1757-899X/1029/1/012019

- Sultanova Sh.A., Artikov A.A., Masharipova Z.A., Abhijit Tarawade, Safarov J.E. Results of experiments conducted in a helio water heating convective drying plant. // International conference AEGIS-2021 «Agricultural Engineering and Green Infrastructure Solutions».  IOP Conf. Series: Earth and Environmental Science 868 (2021) 012045. P.1-6. doi:10.1088/1755-1315/868/1/012045

- Sultanova Sh., Safarov J., Usenov A., Samandarov D., Azimov T. Ultrasonic extraction and determination of flavonoids. XVII International scientific-technical conference “Dynamics of technical systems” (DTS-2021). AIP Conference Proceedings 2507, 050005. 2023. P.1-5. doi.org/10.1063/5.0110524

- Sultanova Sh., Safarov J., Usenov A., Raxmanova T. Definitions of useful energy and temperature at the outlet of solar collectors. // E3S Web of Conferences: Rudenko International Conference “Methodological problems in reliability study of large energy systems” (RSES 2020). Vol. 216, 2020. P.1-5.  doi.org/10.1051/e3sconf/202021601094

- Usenov A.B., Sultanova Sh.A., Safarov J.E., Azimov A.T. Experimental-statistic modelling of temperature dependence of solubility in the extraction of ocimum basilicum plants. // International conference AEGIS-2021 «Agricultural Engineering and Green Infrastructure Solutions». IOP Conf. Series: Earth and Environmental Science 868 (2021) 012047. P.1-5. doi:10.1088/1755-1315/868/1/012047

- Baratov, B.N., Umarov, F.Y., Toshov, Z.H. Tricone drill bit performance evaluation. Gornyi Zhurnal, Moscow, 2021. - № 12. - PP. 60-63. DOI:10.17580/gzh.2021.12.11.

- Toshov, J.B., Toshov, B.R., Baratov, B.N., Haqberdiyev, A.L. Designing new generation drill bits with optimal axial eccentricity | Вопросы проектирования буровых долот нового поколения с оптимальным межосевым эксцентриситетом // Mining Informational and Analytical Bulletin, 2022, (9). - PP. 133–142. DOI: 10.25018/0236_1493_2022_9_0_133

- Toshov J., Makhmudov A., Kurbonov O., Arzikulov G., Makhmudova G. Development and Substantiation of Energy-Saving Methods for Controlling the Modes of Operation of Centrifugal Pumping Units in Complicated Operating Conditions. Proceedings of the 11th International Conference on Applied Innovations in IT, (ICAIIT), November 2023, Koethen, Germany. – РР. 161-165.

- J.B. Toshov, K.T. Sherov, B.N. Absadykov, R.U. Djuraev, M.R. Sikhimbayev, Efficiency of drilling wells with air purge based on the use of a vortex tube. NEWS of the National Academy of Sciences of the Republic of Kazakhstan “Series of geology and technical sciences”. – Almaty, Volume 4, Number 460 (2023), 225–235. https://doi.org/10.32014/2023.2518-170X.331 
Toshov J., Toshov B., Bainazov U., Elemonov M. Application of Cycle-Flow Technology in Coal Mines. Proceedings of the 11th International Conference on Applied Innovations in IT, (ICAIIT), March 2023, Koethen, Germany. – РР. 279-284.

- Usmanov, E., Kholikhmatov, B., Rikhsitillaev, B., Nimatov, K. Device for reducing asymmetry // E3s Web of Conferences 461. 2023. PP, 01052, 1-5. https://doi.org/10.1051/e3sconf/202346101052

- Toshov B., Toshov J., Akhmedova L., Baratov B. The new design scheme of drilling rock cutting tools, working in rotation mode pairs. E3S Web of Conferences 383, 04069 (2023) TT21C-2023 https://doi.org/10.1051/e3sconf/202338304069

- J.B. Toshev, M.B. Norkulov, A.A. Urazimbetova and L.G. Toshniyozov. Optimization of scheme of placing cutting structures on the cone drill bit. E3S Web of Conf., Volume 402, 10039 (2023), International Scientific Siberian Transport Forum - TransSiberia 2023, https://doi.org/10.1051/e3sconf/202340210039

- Toshov J., Baratov B., Sherov K., Mussayev M., Baymirzaev B., Esirkepov A., Ismailov G., Abdugaliyeva G., Burieva J. Ways to Optimize the Kinetic Parameters of Tricone Drill Bits. Material and Mechanical Engineering Technology, №1, 2024, 35-45. https://doi.org/10.52209/2706-977X_2024_1_35

- K.T. Sherov, N.Zh. Karsakova, B.N. Absadykov, J.B. Toshov, M.R. Sikhimbayev, Studying the effect of the boring bar amplitude-frequency characteristics on the accuracy of machining a large-sized part. NEWS of the National Academy of Sciences of the Republic of Kazakhstan “SERIES OF GEOLOGY AND TECHNICAL SCIENCES”. – Almaty, Volume 2, Number 464 (2024), 217–227. https://doi.org/10.32014/2024.2518-170X.405

- Akbar, K., Javokhir, T., Lazizjon, A., Umidjon, K., Muhammad, I., Improvement of soft-start method for high-voltage and high-power asynchronous electric drives of pumping plants. AIP Conf. Proc. 3152, 040006 (2024) / III International Scientific and Technical Conference “Actual issues of Power supply systems” (ICAIPSS2023), 7–8 September 2023, Tashkent, Uzbekistan. https://doi.org/10.1063/5.0218899

- J. Toshov, L. Atakulov, G. Arzikulov, U. Baynazov, Modeling of optimal operating conditions of cyclic-flow technologies with a belt conveyor at coal mine under the "ANSYS" program. AIP Conf. Proc. 3152, 020006 (2024) / III International Scientific and Technical Conference “Actual issues of Power supply systems” (ICAIPSS2023), 7–8 September 2023, Tashkent, Uzbekistan. https://doi.org/10.1063/5.0218904

- Kholikhmatov B.B., Samiev Sh.S., Erejepov M.T., Nematov L.A. Modelling of laboratory work in the science "Fundamentals of power supply" using an educational simulator based on a programmed logic controller // E3S Web of Conferences 384. 2023. РР, 01032, 1-3. https://doi.org/10.1051/e3sconf/202338401032

- Rakhimov F, Rakhimov F, Samiev Sh, Abdukhalilov D. Justification of Technical and Economic Effectiveness of Application of 20 kV Voltage in Overhead Electric Networks //AIP Conf. Proc. 3152, 030023 (2024). https://doi.org/10.1063/5.0218921

- Taslimov A, Mo'minov V, Samiev Sh, Abdukhalilov D. Issues of Optimization of Electrical Network Parameters Medium Voltage //AIP Conf. Proc. 3331, 020007 (2025). https://doi.org/10.1063/5.0305781

- Toshbekov, O., Urazov, M., Yermatov, S., & Khamraeva, M. 2023). Yeffisient and yesonomisal yenergy use teshnology in the prosessing of domestis soarse wool fiber. In Ye3S Web of Sonferenses (Vol. 461, p. 01068). https://doi.org/10.1051/e3sconf/202346101068

- Jumaniyozov, K., Urozov, M., Toshbekov, O., Salimova, M., Raximova, K., & Khursandova, B. (2025, November). Enhancement of energy-efficient cleaning equipment. In American Institute of Physics Conference Series (Vol. 3331, No. 1, p. 050007). https://doi.org/10.1063/5.0307149

- Sultonova, F., Toshbekov, O., Urozov, M., Boymurova, N., Mustanova, Z., & Boltaeva, I. (2025, November). Enhancing and evaluating the characteristics of specialized workwear for employees in the electric power supply sector. In American Institute of Physics Conference Series (Vol. 3331, No. 1, p. 050006).  https://doi.org/10.1063/5.0306350