Experimental analysis of the programmable logic controller

Olim Alimnazarov 1, a), Abdurasul Jurayev 2

1 Termiz State Pedagogical Institute, Termez, Uzbekistan

2Termiz State University of Engineering and Agrotechnologies, Termez, Uzbekistan

a) Corresponding author: olimbekalimnazarov12@gmail.com

Abstract. This article analyses analogue temperature measurements using components of a programmable logic controller (PLC). In addition, the practical application of discrete control algorithms was examined. Real-time monitoring of temperature variations was ensured through the PLC module’s input and output channels. The analogue signal in the range of 0…10 V provided by the thermocouple sensor was linearly converted within the temperature interval of 0…60 °C. This demonstrated the correspondence between the temperature and the output signal. When the specified threshold value (TUST = 45 °C) was reached, the discrete output was switched to the TRUE state. In this case, the control algorithm demonstrated the stability of the module. The results showed the high accuracy of PLC systems in working with analogue-discrete signals. They also confirmed the efficiency of automatic monitoring of temperature processes and the potential for creating adaptive algorithms in an IoT environment. This study contributes to ensuring the real-time stability of PLC systems.

INTRODUCTION

Programmable Logic Controller (PLC) components are used in the automation of technological processes. They are applied in monitoring technological process systems. PLC systems ensure real-time control of complex technological processes. They provide the capability to monitor these processes, thereby playing an important role in maintaining overall efficiency. The stability of automated systems is ensured through the use of PLC [1].

Modern technological processes widely employ Programmable Logic Controllers (PLC). Their function is not limited to controlling only discrete processes. PLC are also required to control analogue physical quantities with high accuracy. This, of course, ensures the possibility of real-time monitoring of systems [2]. In this process, measuring analogue signals such as temperature, pressure, and flow is of great importance. Moreover, converting these signals is one of the essential tasks of PLC systems [7].

In the automatic control of temperature processes through PLCs, PID or on/off control algorithms are used. These algorithms ensure the accuracy of the control process, thereby maintaining the overall stability of the system. Temperature monitoring via PLC systems is of great significance in technological processes [8]. This system provides a stable response to environmental changes and demonstrates high efficiency [3-38].

In addition, the accuracy of analogue-discrete signal converters in PLC modules is critically important. Their accuracy plays a key role in ensuring the reliability of measurement systems. During the normalisation of analogue signals in the 0…10 V or 4…20 mA standards, the technical parameters of PLC modules become crucial. Parameters such as linearity, sensitivity, and resistance to noise are among the important aspects. The quality of process automation depends on these parameters [4].

The integration of IoT with a PLC system enables the transmission of data to the cloud in real time [9]. This creates conditions for overall monitoring of the system and the intelligent analysis of data [5]. This, of course, ensures the adaptability of technological processes to the digital ecosystem [10]. The stability and safety of PLC systems depend on the variability of technological process conditions. They are characterised by their resistance to factors such as temperature and electrical noise [6].

EXPERIMENTAL RESEARCH

The programmable logic controller (PLC) is designed for use in a technological process environment. It is a digital electronic system intended for the user. It employs programmable memory that allows storing instructions in internal memory. These instructions serve to define logic algorithms. It performs tasks such as real-time measurement and arithmetic operations. It is used for monitoring various types of technological processes through digital or analogue inputs and outputs. The PLC is most commonly used for the automation of technological processes. The PLC often operates reliably even under unfavourable environmental conditions.

The PLC is a device intended for use in real-time operation. For application in automated systems, the PLC is equipped with discrete and analogue inputs and outputs.

The discrete input (DI) is intended for supplying the PLC with parameters that have only two states: on or off, signal present or absent. These states are defined by the voltage level applied to the DI.

Depending on the model of the programmable logic controller, 5 V, 12 V, and 24 V direct current are used. In addition, 110 V and 220 V alternating current are used as the most common DI voltages.

In some cases, 24 V direct current discrete inputs are used in PLCs. If 24 volts of direct current is applied to such a DI, the PLC recognises the presence of the input signal as “logical one” (TRUE). If it is 0 volts, the PLC determines the absence of the input signal as “logical zero” (FALSE).

0 V

+24 V

Di0

Di1

PLC

+24V

0V

FIGURE 1. Operating principle of discrete inputs

Limit switches, level sensors, and other discrete sensors are connected to the DI. Some sensors are connected to several DIs simultaneously. In this case, the electromechanical manometer provides two discrete signals: the minimum and maximum pressure levels.

0 V

+24 V

Di0

Di1

PLC

+24V

0V

Pmin

Pmax

FIGURE 2. Connecting the electromechanical manometer to two discrete inputs

The discrete output (DO) in a PLC is used to switch on devices connected to it: valves, magnetic starters, and intermediate relays. In this case, the discrete output is a switch that is closed by the command of the programmable logic controller. The discrete output can have two states: open (FALSE) and closed (TRUE). If the output is in the closed state, the connected device is switched on. If it is in the open state, it is switched off.

+24V

0V

+24V

0V

Com

DQ0

DQ1

(TRUE)

(FALSE)

OFF

ON

PLC

FIGURE 3. Operating principle of discrete outputs

Procedure for conducting the experiment. Our experiments were carried out to analyse the number of inputs and outputs in the PLC and their parameters (Figure 3). In this process, a thermocouple sensor with a measurement range of 0…60 °C and an output signal range of 0…10 V was used. When the PLC module switch is turned on, the thermocouple sensor is activated. When the temperature reaches the specified value, the light indicator turns on. In this case, the output signal range was set to 0…10 V and the measurement range was set to Ain low = 0 and Ain high = 60. A value of 45 °C was assigned to the TUST variable. The switch of the technological data sensors module was turned on. The temperature processes in the system were monitored. The temperature element was adjusted by means of a regulator.

IN1

IN2

OUT1

OUT2

DO3

+15V

PLC

2

3

8

7

AI1

TC

CI

PM

HL

FIGURE 4. Connection diagram of the PLC module

TC – mini module “thermocouple”; CI – control and indicator module; PM – power module

RESEARCH RESULTS

During the experiment, temperature measurements were recorded through the thermocouple sensor connected to the analog input channel of the PLC module. According to the parameters of the analog input module, the range was set to Ain low = 0 °C and Ain high = 60 °C. The output signal range was adjusted to 0…10 V. As a result of this calibration, each 1 °C change corresponded to 0.166 V. This demonstrated the system's linear variation principle, as shown in the following graph (Fig. 5).

10

8

6

4

2

0

0	10	20	30	40	50	60

U (V)

T (CO)

FIGURE 5. Temperature and voltage graph

According to the results of our experiments, when the temperature value reached the TUST = 45 °C limit, the PLC algorithm switched the discrete output to the TRUE state. This activated the indicator module. It enabled the PLC to monitor the analog signal from the sensor in real time. When the temperature value decreased, the indicator turned off, and the system returned to the standby mode. In addition, the stability of the analog signal and the accuracy of digital conversion were analysed. No significant delays were observed in the system. This demonstrated that the PLC module performs analog-to-discrete conversion at high speed.

Overall, our experiments demonstrated the sensitivity of the system to temperature changes through the PLC component. It responded to the specified limit value. This highlighted the practical significance of automatic monitoring of temperature processes across various measurements.

CONCLUSION

These experiments studied the analysis of analog temperature measurements through programmable logic controller (PLC) components. In addition, the execution of the discrete control algorithm was examined through the experiments. The analog signal in the range of 0…10 V from the thermocouple sensor was linearly converted in the PLC’s analog input module to a temperature range of 0…60 °C. The results demonstrated a proportional increase of the output voltage with the upper temperature limit. This showed the stability of the PLC modules when working with analog signals.

During the experiment, when the specified TUST = 45 °C limit was reached, the discrete output channel was activated by the PLC. This demonstrated the PLC system’s ability to monitor analog signals in real time. In addition, it showed the capability to make control decisions with high accuracy. The timely response of the output module indicated that the control algorithm was correctly configured and that the module operated without delays.

The obtained results demonstrated that PLC systems are stable and effective in the automatic monitoring of temperature processes. The results of our experiments indicate the possibility of analysing data in an IoT-based monitoring environment. At the same time, these results provide the opportunity to develop adaptive algorithms.

REFERENCES

- M. Rahman, H. A. Banna, A. Hossain, M. Khan, S. Hossain, M. I. H. Patwary, T. Mallik, and M. Islam, “Design and implementation of a PLC-based automatic industrial drainage system,” in 2021 International Conference on Information and Communication Technology for Sustainable Development (ICICT4SD) (2021), pp. 185–188. https://doi.org/10.1109/ICICT4SD50815.2021.9396930

- Y. Birbir and H. Noğay, “Design and implementation of PLC-based monitoring control system for three-phase induction motors fed by PWM inverter,” in Proceedings of the International Conference on Automation and Control (2007).

- R. Chen, “A comprehensive analysis of PID control applications in automation systems: Current trends and future directions,” Highlights in Science, Engineering and Technology 97, 126–132 (2024). https://doi.org/10.54097/6q4xxg69

- H. Eren and E. Eren, Signal Processing in Process Control and Automation (CRC Press, Boca Raton, 2012).

- A. Banaulikar, “Real-time monitoring and control for industrial automation using PLC,” International Journal for Research in Applied Science and Engineering Technology 8(5), 1193–1200 (2020). https://doi.org/10.22214/ijraset.2020.30453

- L. Huai and C. Cheng, “Reliability design of PLC-based control system,” in 2013 Ninth International Conference on Natural Computation (ICNC) (2013), pp. 1671–1675. https://doi.org/10.1109/ICNC.2013.6818250

- M. D. Arifin, A. C. P. Oloan, and E. H. Brahmantyo, “Development of a control and monitoring system for ballast systems based on programmable logic controllers (PLC) and human–machine interface (HMI),” International Journal of Marine Engineering Innovation and Research 10(1), 242–255 (2025). https://doi.org/10.12962/j25481479.v10i1.4768

- Z. Zhang, Y. Gao, and C. Ye, “Research on design practice of PLC in automatic control system of electrical equipment,” Lifelong Education 9(5), 81–83 (2020). https://doi.org/10.18282/le.v9i5.1211

- G. Chen, L. Zhang, J. Arinez, and S. Biller, “Real-time performance analysis of production lines: A system-theoretic approach,” in IEEE International Conference on Automation Science and Engineering (2012), pp. 656–661.

- L. J. Koo, C. M. Park, C. H. Lee, S. Park, and G. N. Wang, “Simulation framework for the verification of PLC programs in automobile industries,” International Journal of Production Research 49(16), 4925–4943 (2011). https://doi.org/10.1080/00207543.2010.492404

- K. Jumaniyozov, M. Urozov, O. Toshbekov, M. Salimova, K. Raximova, and B. Khursandova, “Enhancement of energy-efficient cleaning equipment,” AIP Conference Proceedings 3331, 050007 (2025). https://doi.org/10.1063/5.0307149

- F. Sultonova, O. Toshbekov, M. Urozov, N. Boymurova, Z. Mustanova, and I. Boltaeva, “Enhancing and evaluating the characteristics of specialized workwear for employees in the electric power supply sector,” AIP Conference Proceedings 3331, 050006 (2025). https://doi.org/10.1063/5.0306350

- N. Safarov, R. Yangiboev, H. Bo‘riyev, B. Karshiev, O. Gulboyev, F. Narzullayev, and A. Qurbonov, “Study of the influence of main factors on the mass and density of saw fiber separator raw material,” AIP Conference Proceedings 3268, 020033 (2025). https://doi.org/10.1063/5.0257374

- Mahmutkhonov S., Baizhonova L., Mustayev R., Tashmatova S. Dynamic analysis of voltage-ampere characteristics and harmonic distortions in electric arc furnaces. // AIP Conference Proceedings. 3331(1), 2025. pp. 070023, 1–5. https://doi.org/10.1063/5.0305745.

- Bobojanov M., Mahmutkhonov S. Influence of the consumer to power quality at the point of connection // E3S Web of Conferences 384. 2023. РР, 01041, 1-5. https://doi.org/10.1051/e3sconf/202338401041.

- Reymov K.M., Makhmuthonov S.K., Turmanova G., Uzaqbaev Q.  Optimization of electric networks modes under conditions of  partial uncertainty of initial information // E3S Web of Conferences 289, 07023 (2021). -2021, pp: 1-4,  https://doi.org/10.1051/e3sconf/202128907023.

- Alimov, U.K., Reimov, A.M., Namazov, Sh.S., Beglov, B.M. The insoluble part of phosphorus fertilizers, obtained by processing of phosphorites of central kyzylkum with partially ammoniated extraction phosphoric acid. Russian Journal of Applied Chemistry. Russ J Appl Chem (2010) 83(3): 545–552.  https://doi.org/10.1134/S107042721030328

- Reymov, A.M., Namazov, S.S., Beglov, B.M. Effect of phosphate additives on physical-chemical properties of ammonium nitrate. Journal of Chemical Technology and Metallurgy 2013 48(4), 391-395. http://dl.uctm.edu/journal/

- Urishev, B., Fakhriddin Nosirov, and N. Ruzikulova. 2023. “Hydraulic Energy Storage of Wind Power Plants.” E3S Web of Conferences, 383. https://doi.org/10.1051/e3sconf/202338304052

- Urishev, B., S. Eshev, Fakhriddin Nosirov, and U. Kuvatov. 2024. “A Device for Reducing the Siltation of the Front Chamber of the Pumping Station in Irrigation Systems.” E3S Web of Conferences, 274. https://doi.org/10.1051/e3sconf/202127403001

- Turabdjanov, S., Sh. Dungboyev, Fakhriddin Nosirov, A. Juraev, and I. Karabaev. 2021. “Application of a Two-Axle Synchronous Generator Excitations in Small Hydropower Engineering and Wind Power Plants.” AIP Conference Proceedings. https://doi.org/10.1063/5.0130649

- L.Jing, J.Guo, T.Feng, L.Han, Z.Zhou and M.Melikuziev, "Research on Energy Optimization Scheduling Methods for Systems with Multiple Microgrids in Urban Areas," 2024 IEEE 4th International Conference on Digital Twins and Parallel Intelligence (DTPI), Wuhan, China, 2024, pp. 706-711, https://ieeexplore.ieee.org/abstract/document/10778839

- Baratov, B.N., Umarov, F.Y., Toshov, Z.H. Tricone drill bit performance evaluation. Gornyi Zhurnal, Moscow, 2021. - № 12. - PP. 60-63. DOI:10.17580/gzh.2021.12.11.

- Toshov, J.B., Toshov, B.R., Baratov, B.N., Haqberdiyev, A.L. Designing new generation drill bits with optimal axial eccentricity | Вопросы проектирования буровых долот нового поколения с оптимальным межосевым эксцентриситетом // Mining Informational and Analytical Bulletin, 2022, (9). - PP. 133–142. DOI: 10.25018/0236_1493_2022_9_0_133

- Toshov J., Makhmudov A., Kurbonov O., Arzikulov G., Makhmudova G. Development and Substantiation of Energy-Saving Methods for Controlling the Modes of Operation of Centrifugal Pumping Units in Complicated Operating Conditions. Proceedings of the 11th International Conference on Applied Innovations in IT, (ICAIIT), November 2023, Koethen, Germany. – РР. 161-165.

- J.B. Toshov, K.T. Sherov, B.N. Absadykov, R.U. Djuraev, M.R. Sikhimbayev, Efficiency of drilling wells with air purge based on the use of a vortex tube. NEWS of the National Academy of Sciences of the Republic of Kazakhstan “Series of geology and technical sciences”. – Almaty, Volume 4, Number 460 (2023), 225–235. https://doi.org/10.32014/2023.2518-170X.331 
Toshov J., Toshov B., Bainazov U., Elemonov M. Application of Cycle-Flow Technology in Coal Mines. Proceedings of the 11th International Conference on Applied Innovations in IT, (ICAIIT), March 2023, Koethen, Germany. – РР. 279-284.

- Usmanov, E., Kholikhmatov, B., Rikhsitillaev, B., Nimatov, K. Device for reducing asymmetry // E3s Web of Conferences 461. 2023. PP, 01052, 1-5. https://doi.org/10.1051/e3sconf/202346101052

- Toshov B., Toshov J., Akhmedova L., Baratov B. The new design scheme of drilling rock cutting tools, working in rotation mode pairs. E3S Web of Conferences 383, 04069 (2023) TT21C-2023 https://doi.org/10.1051/e3sconf/202338304069

- J.B. Toshev, M.B. Norkulov, A.A. Urazimbetova and L.G. Toshniyozov. Optimization of scheme of placing cutting structures on the cone drill bit. E3S Web of Conf., Volume 402, 10039 (2023), International Scientific Siberian Transport Forum - TransSiberia 2023, https://doi.org/10.1051/e3sconf/202340210039

- Toshov J., Baratov B., Sherov K., Mussayev M., Baymirzaev B., Esirkepov A., Ismailov G., Abdugaliyeva G., Burieva J. Ways to Optimize the Kinetic Parameters of Tricone Drill Bits. Material and Mechanical Engineering Technology, №1, 2024, 35-45. https://doi.org/10.52209/2706-977X_2024_1_35

- K.T. Sherov, N.Zh. Karsakova, B.N. Absadykov, J.B. Toshov, M.R. Sikhimbayev, Studying the effect of the boring bar amplitude-frequency characteristics on the accuracy of machining a large-sized part. NEWS of the National Academy of Sciences of the Republic of Kazakhstan “SERIES OF GEOLOGY AND TECHNICAL SCIENCES”. – Almaty, Volume 2, Number 464 (2024), 217–227. https://doi.org/10.32014/2024.2518-170X.405

- J. Toshov, L. Atakulov, G. Arzikulov, U. Baynazov, Modeling of optimal operating conditions of cyclic-flow technologies with a belt conveyor at coal mine under the "ANSYS" program. AIP Conf. Proc. 3152, 020006 (2024) / III International Scientific and Technical Conference “Actual issues of Power supply systems” (ICAIPSS2023), 7–8 September 2023, Tashkent, Uzbekistan. https://doi.org/10.1063/5.0218904

- Kholikhmatov B.B., Samiev Sh.S., Erejepov M.T., Nematov L.A. Modelling of laboratory work in the science "Fundamentals of power supply" using an educational simulator based on a programmed logic controller // E3S Web of Conferences 384. 2023. РР, 01032, 1-3. https://doi.org/10.1051/e3sconf/202338401032

- Rakhimov F, Rakhimov F, Samiev Sh, Abdukhalilov D. Justification of Technical and Economic Effectiveness of Application of 20 kV Voltage in Overhead Electric Networks //AIP Conf. Proc. 3152, 030023 (2024). https://doi.org/10.1063/5.0218921

- Taslimov A, Mo'minov V, Samiev Sh, Abdukhalilov D. Issues of Optimization of Electrical Network Parameters Medium Voltage //AIP Conf. Proc. 3331, 020007 (2025). https://doi.org/10.1063/5.0305781

- Toshbekov, O., Urazov, M., Yermatov, S., & Khamraeva, M. 2023). Yeffisient and yesonomisal yenergy use teshnology in the prosessing of domestis soarse wool fiber. In Ye3S Web of Sonferenses (Vol. 461, p. 01068). https://doi.org/10.1051/e3sconf/202346101068

- Jumaniyozov, K., Urozov, M., Toshbekov, O., Salimova, M., Raximova, K., & Khursandova, B. (2025, November). Enhancement of energy-efficient cleaning equipment. In American Institute of Physics Conference Series (Vol. 3331, No. 1, p. 050007). https://doi.org/10.1063/5.0307149

- Sultonova, F., Toshbekov, O., Urozov, M., Boymurova, N., Mustanova, Z., & Boltaeva, I. (2025, November). Enhancing and evaluating the characteristics of specialized workwear for employees in the electric power supply sector. In American Institute of Physics Conference Series (Vol. 3331, No. 1, p. 050006).  https://doi.org/10.1063/5.0306350