Efficiency of External Involute Spur Gearing

Oleksandr Koriak1, a)

1Kharkiv National Automobile and Highway University, 
Department of Machine Elements and Theory of Mechanisms and Machines, 61002 Kharkiv, Ukraine 

a)Corresponding author: alexanderalexkor@gmail.com

Abstract. The work is devoted to the study of the external involute spur gear efficiency, taking into account the features of single-pair and double-pair meshing phases. Special attention is paid to the double-pair meshing phase. The influence of the load distribution between pairs of teeth during the double-pair meshing phase on energy losses has been established. Increasing the load on the pre-pitch pair of teeth leads to a decrease in the efficiency of the meshing in the reduction mode, i.e., provided that the gear with fewer teeth is the driver. It is shown that the instantaneous gear efficiency does not depend on the gear wheel rotation angle only in the case of double-pair meshing, provided that the normal force is equally distributed between the pairs of teeth. Therefore, it is worth taking into account the energy losses during the contact of one pair conjugate profiles instead of determining the instantaneous efficiency. As a result of the study, calculation dependencies for determining the instantaneous and cyclic efficiency for different phases of meshing were obtained. As the literature analysis showed, the derivation of existing calculation dependencies used to determine the meshing efficiency was made with significant simplifications. Comparison of results shows, that the mentioned formulas allow to determine the meshing efficiency with sufficient accuracy for engineering calculations. This study describes, how the accuracy of existing simplified formulas can be improved.

Keywords: efficiency; involute gearing; spur gear; conjugate profiles; friction force; sliding speed.

INTRODUCTION

The efficiency coefficient (EC) of any mechanical transmission is one of the main parameters that characterizes its energy efficiency and perfection. Energy losses in transmissions negatively affect the traction-speed characteristics of cars and tractors, lead to a decrease in fuel efficiency, and reduce the service life of parts and assemblies. Therefore, when designing mechanical transmissions, one should focus on such design solutions that would ensure the highest possible efficiency.

The main elements of mechanical transmissions are gears, which provide the conversion and transmission of rotational motion from the engine to the vehicle's propulsion units. The transmission of motion in a gear mesh is carried out through the interaction of conjugate profiles, which form a higher kinematic pair. During interaction, the conjugate profiles roll and slide against each other, which is the main cause of energy losses in mechanical transmissions. Considering that several pairs of gears are involved in transmitting torque, determining the efficiency coefficient of each gear is a very important task on the way to improving the transmission.

Directions and Methods of Research on Energy Losses in Mechanical Transmissions

When performing engineering calculations and designing mechanical transmissions, one usually focuses on the average values of the efficiency of the drive mechanisms [1, 2, 3, 4]. This approach takes place when designing typical designs of mechanical transmissions, and is not intended to solve the problems of minimizing energy losses in transmissions.

A significant number of works are devoted to the study of factors that affect energy losses, as well as the search for ways to increase the efficiency of gear transmissions. It is noted [5, 6, 7, 8, 9, 10] that energy losses in gear transmissions consist of losses in the meshes, bearings, seals, as well as other losses (losses from splashing and mixing of lubricant, losses due to aerodynamic drag, etc.). In this case, some of the listed losses do not depend on the magnitude of the transmitted torque, while others depend on the load of the transmission. The first part of the losses depends on the rotational speed of the transmission parts, the quality and temperature of the lubricant, the depth of immersion of the gears in the oil bath, etc. The second part of the losses is approximately proportional to the magnitude of the torque acting on the transmission links and relates to the meshes and bearings.

In the work [10], the generation and dissipation of heat in a truck gearbox are modeled, which allows determining operating temperatures, identifying and minimizing energy losses, optimizing the design of gearboxes and improving the lubrication system. The simulation results are confirmed by experiments. It is noted that the main sources of heat are gear meshes and bearings.

Researchers pay much attention to minimizing friction by improving the quality of lubricating materials. For example, the work [8] investigates the possibility of reducing the friction coefficient using water-containing transmission fluids. The researchers conclude that water-containing lubricating fluids can significantly reduce power losses in gearboxes compared to traditional mineral and synthetic oils.

The possibility of the transmission operating under conditions of lubrication loss is also considered [11, 12]. The influence of thermal and mechanical treatment of the tooth surface on the service life of the gear transmission has been studied. It is noted that increasing the service life of gears in conditions of insufficient lubrication can be achieved by applying special coatings to the working surfaces of the gear teeth. Experimental studies have shown that the traditional method of reducing the friction coefficient by superfinishing the tooth surfaces is less effective than the method of applying coatings. Moreover, the work [12] indicates that superfinishing leads to a negative result due to poor lubricant retention on a smooth surface and scuffing.

The work [6] introduces the concept of "super-efficiency" and proposes to divide industrial gearboxes into classes according to their efficiency value. It is noted that the largest share of energy losses in mechanical transmissions is attributed to losses in the mesh under load. Ways to increase efficiency are indicated, which consist in reducing the sliding speed of conjugate profiles, as well as improving the quality and reducing the amount of lubricating material.

Modeling the movement of lubricants in a gearbox is performed in the work [9]. The implementation of this study’s results will allow calculating energy losses that do not depend on the transmission load, reduce calculation time and accelerate the development of energy-efficient gear transmission designs.

Dissipative energy losses in gearboxes lead to the heating of its parts and assemblies. It is obvious that for the reliable operation of the transmission, it is necessary to ensure sufficient heat dissipation from the gearbox to the environment. The work [13] is devoted to solving a similar problem. It investigates the process of heat transfer from gears in a gearbox operating under dry lubrication conditions. It has been established that the geometry of the gears significantly affects the airflows inside the gearbox. It is noted that helical gears, unlike spur gears, create more complex, multidirectional airflows. As a result, the use of helical gears leads to a significantly higher heat transfer and improves the thermal operating mode of the transmission.

The work [7] is dedicated to developing the method for predicting power loss and heat generation during steady-state and transient operating modes of a gear transmission. The results obtained using the proposed model were verified and confirmed by experimental studies.

The authors of [14] warn researchers against errors when determining the mechanical transmission’s instantaneous efficiency and point to the necessity of accounting for the elastic and mass-inertial parameters of the transmission.

In the work [5], a detailed analysis of the factors leading to efficiency reduction in gear transmissions is carried out, the contributions of structural elements to the total energy losses are investigated, and the division of energy losses into losses under load and no-load losses is systematized. The authors of the work point to the possibility of a significant increase in transmission efficiency under the condition of complex design optimization.

An analysis of literature sources has shown that among the dissipative losses that occur during the operation of gear transmissions under load, the most significant are energy losses in the gear meshes.

Currently, involute gears are predominantly used in power mechanical transmissions, which is due to their manufacturability and high operational performance. For a comparative assessment of the efficiency coefficient of a cylindrical involute gearing, a convenient formula is usually used, the proof of which is given in a textbook on the theory of mechanisms and machines [15]:

,						(1)

where 	ε – face overlap ratio;

f – combined average friction coefficient for gears;

z1; z2 – the number of teeth of the gear and wheel, respectively (the upper sign is taken for external, and the lower sign for internal gearing).

Analogues of formula (1) are also found in the literature. For example, in [16] the efficiency of an external involute gear is determined by the formula:

,						(2)

where 	αW – pressure angle.

In [17], a similar formula has the form:

,					(3)

where 	γ – coefficient that takes into account the gear shift coefficients (for zero gears γ=1).

The derivation of formula (1) was based on averaging the power losses during single-pair meshing. Thus, when determining the sliding speed of conjugate profiles, the average value of the distance from the pitch point to the tooth contact point was used. It was assumed that the entire load is transmitted by one pair of teeth. The presence of a double-pair meshing phase was taken into account only when determining the length of the working section of the line of action, without considering the influence of this operating mode on meshing efficiency. Obviously, these simplifications should affect the accuracy of determining the meshing efficiency coefficient.

The aim of the research is to determine the efficiency of an external involute spur gear mesh, taking into account the features of the single-pair and double-pair meshing phases. To achieve this goal, the following tasks must be solved:

- obtain calculation dependencies for determining the efficiency coefficient during the single-pair and double-pair meshing phases;

- establish the influence of the load distribution between the pairs of teeth during the double-pair operating mode on the meshing efficiency;

- evaluate the acceptability of formulas (1)-(3) for determining the efficiency coefficient of an external involute spur gear.

DETERMINATION OF THE MESHING EFFICIENCY COEFFICIENT

Fig. 1 shows several positions of the tooth of the involute gear 1 in different meshing phases. The tooth profile enters into meshing at point A (the beginning of the working section of the line of action) and occupies position 11. At the same time, due to the face overlap, the adjacent tooth profile of the gear 1 is in position 21 and contacts the conjugate profile of the gear 2 (not shown in the figure) at point C. The simultaneous meshing of the two pairs of teeth will continue until the adjacent profile takes a position  that corresponds to its exit from meshing. At this time, the first profile will be in position  and will contact the conjugate profile at point D.

Figure 1. Calculation scheme

During the double-pair meshing phase, the distance between the contact points of two adjacent profiles does not change. It is equal to the circular pitch of the teeth on the main circle, i.e.

,						(4)

where 	m – gear module;

α – the basic rack profile angle (standard value is).

Let us determine the magnitude of the normal force in the case of a double-pair meshing, i.e. at an angle φ of rotation of the first gear wheel, which lies within φA≤φ≤φD (see Fig. 1). This meshing phase at a certain value of the rotation angle φ= φKb1 is shown in Fig. 2.

Figure 2. Kinematic and force characteristics in double-pair meshing

Let kFn be the fraction of the normal force that falls on a pair of teeth that have entered into meshing and are in contact on the pre-pitch section of the line of action, . Then  is the fraction of the normal force that falls on the previous pair of teeth that contact on the post-pitch section of the line of action, .

Friction forces that arise in the meshing of pairs of teeth:

;						(5)

,						(6)

where 	f – friction coefficient of the first kind.

Let us formulate the equilibrium equation, taking into account expressions (5) and (6), as well as the directions of the relative velocity vectors  and  (Fig. 2):

,		(7)

where	rb1 – base circle radius of the driving gear.

From expression (7) we obtain:

.					(8)

Due to the properties of the involute of a circle, the length of the segment N1K is equal to the length of the arc  on the main circle of gear wheel 1, i.e.

.							(9)

Similarly,

,				(10)

where 	 – is the angular pitch of the teeth of gear wheel 1 with the number of teeth z1.

For an arbitrary rotation angle φ of gear wheel 1 (φA≤φ≤φD), the normal force in a double-pair meshing, taking into account equations (8), (9), (10), is

.				(11)

Substituting (11) into (5) and (6), we obtain expressions for determining friction forces:

;				(12)

.				(13)

Let us determine the sliding speeds of the contact points of the conjugate profiles in the external mesh. The speed of the point K1, which belongs to the profile 11 of the first gear wheel, relative to the point K2, which belongs to the profile 12 of the second gear wheel (Fig. 2):

.						(14)

Similarly, the speed of point M1, which belongs to profile 21 of the first gear, relative to point M2, which belongs to profile 22 of the second gear, is

.						(15)

The distances from the contact points to the pitch point (point P) can be found by taking into account the properties of the involute of the circle and the geometric parameters of the involute meshing (Fig. 2). Therefore:

;				(16)

,		(17)

The power of friction forces, taking into account (12), (13), (14), (15), (16), (17):

(18)

Let us assume that during the double-pair meshing phase the normal force is equally distributed between the pairs of teeth, i.e. kFn=0.5. In this case, according to (18), we obtain:

.						(19)

As follows from equation (19), at kFn=0.5 the power of friction forces does not depend on the rotation angle φ of gear wheel 1.

After the previous pair of teeth disengages (profiles 21 and 22 in Fig. 2), the single-pair meshing phase begins. In the case of contact of conjugate profiles on the pre-pitch section of the line of action, it is necessary to set kFn=1 in formula (18). As a result, we obtain:

.					(20)

If the contact of the conjugate profiles occurs on the post-pitch section of the line of action, it is necessary to substitute kFn=0 in formula (18) and keep in mind that the previous pair of teeth has already disengaged. In this case, the power of friction forces:

.					(21)

Instantaneous gear efficiency (rolling friction is neglected):

,						(22)

where 	 – power on the drive gear.

In the general case, taking into account (18) and (22), as well as the fact that ω2/ω1= z1/z2, the gear efficiency is

.		(23)

Let us assume that during the double-pair meshing phase, the pairs of teeth are loaded equally, i.e. kFn=0.5. Then, according to (23), the meshing efficiency is

.						(24)

In this case, as follows from expression (24), the efficiency of the gearing does not depend on the angle of rotation of gear wheel 1.

Let us consider the single-pair meshing phase. Let the contact of the conjugate profiles occur on the pre-pitch section of the line of action. In this case, the power of the friction forces is determined by formula (20), and the instantaneous efficiency of the meshing, taking into account (22), is

.					(25)

Let during the single-pair meshing phase the contact of the conjugate profiles occurs on the post-pitch section of the line of action. Then, the instantaneous meshing efficiency, taking into account (21) and (22):

.					(26)

Expressions (23), (24), (25), (26) indicate that the instantaneous efficiency of the gearing does not depend on the angle φ only in the case of double-pair meshing at kFn=0.5, i.e. under the condition of equal loading of the pairs of teeth. In all other cases, the instantaneous efficiency of the gearing depends on the rotation angle of the driving gear. Therefore, instead of determining the instantaneous efficiency, it is advisable to take into account the energy losses during the meshing of one pair of conjugate profiles.

Taking into account (22), the instantaneous meshing loss coefficient is

,						(27)

where 	 – the torque that must be applied to the drive gear to overcome the frictional forces in the gearing.

We will assume that the torque T1 and the angular velocity ω1 are constant. In the general case, the integral loss coefficient during the double-pair meshing phase is

.					(28)

Taking into account (18) and (28), we obtain after integration:

.		(29)

Efficiency coefficient, which takes into account energy losses during the double-pair meshing phase is

. 	(30)

It should be noted that the function (30) is defined and continuous for all values of the variable kFn. However, at kFn=0.5 it is quite difficult to determine the efficiency by formula (30), since it is necessary to solve the problem of finding the limit of the function. In this case, it is better to use formula (24), since, as already noted, at kFn=0.5 the gear efficiency loses its dependence on the rotation angle of the gear wheel 1.

Fig. 3 presents curves 1, constructed according to formula (30) at f = 0.1. They reflect the dependence of the meshing efficiency on the normal force distribution coefficient kFn during the double-pair meshing phase for two numbers of teeth of the driven gear. For comparison, graphs constructed according to formulas (1), (2) and (3) are also attached (lines 2, 3 and 4, respectively).

z1=17; z2=34					z1=17; z2=85

а)						б)

1 – efficiency according to formula (30); 2 – efficiency according to formula (1);

3 – efficiency according to formula (2); 4 – efficiency according to formula (3)

Figure 3. Efficiency of the double-pair meshing phase

Curves 1 illustrate a decrease in the efficiency of the gear mesh with an increase in the proportion of the normal force acting on the pre-pitch pair of teeth in the reduction mode of operation, that is, under the condition that the gear with the smaller number of teeth serves as the driving member. This occurs due to the greater average distance of the contact point of the pre-pitch pair of teeth from the pitch point of mesh, as well as a slight increase in the total normal force Fn resulting from the growth of the friction force component  acting on the pre-pitch pair of teeth.

When designing and manufacturing gears, the aim is to evenly distribute the load between pairs of teeth. In this case, the gear efficiency will be approximately at the level of the midpoint of curves 1. It is obvious that for a double-pair meshing phase, simplified formulas (1), (2) and (3) give somewhat overestimated values of the gear efficiency, especially formula (3).

Let us determine the efficiency during the single-pair pre-pitch phase of meshing. In this case, kFn=1, the integration limits are the angles φD and φP (Fig. 1), and the integral loss coefficient, taking into account (28) and (29), is

.				(31)

Efficiency factor, which takes into account energy losses in meshing during the single-pair pre-pitch phase, is

. 			(32)

Let us consider the case of a single-pair post-pitch meshing phase, when the angle of rotation of the driving gear wheel is within φР≤φ≤φС. Let us determine the integral loss coefficient, taking into account (21) and (28):

.		(33)

Efficiency factor, which takes into account energy losses in meshing during the single-pair post-pitch meshing phase:

.				(34)

Let us determine the integral loss coefficient during the meshing of one pair of teeth:

,			(35)

where 	 – integral loss coefficient of meshing during the rotation of the driving gear from φС to φВ.

Efficiency factor, which takes into account energy losses during the meshing of one pair of teeth:

(36)

At the angle of rotation of the driving gear from φС to φВ, the load is transmitted by two pairs of teeth, and . Therefore,  can be determined by formula (29), taking into account the possible difference in the normal force distribution coefficient kFn.

Let us assume that during the double-pair meshing phase the normal force is equally distributed between the pairs of teeth. As already mentioned, at kFn=0.5 the efficiency does not depend on the rotation angle of the gear wheel. In this case, taking into account (24), (31), (33), (36), the formula for determining the efficiency of the meshing has the form:

.	(37)

Fig. 4 presents graphs of spur involute gear efficiency versus the gear ratio, constructed using different formulas at f = 0.1 and the number of teeth of the driving gear z1=17.

1 – efficiency according to formula (37); 2 – efficiency according to formula (1);

3 – efficiency according to formula (2); 4 – efficiency according to formula (3)

Figure 4. Efficiency of external spur involute gearing

Curve 1 corresponds to the exact formula (37) within the accepted assumptions. It can be stated that curve 2, constructed according to the simplified formula (1), almost coincides with curve 1. Even simpler formulas (2) and (3), which do not require the determination of the face overlap ratio and are represented by curves 3 and 4, respectively, demonstrate a somewhat greater discrepancy in the results. Calculation according to formula (2) leads to an underestimated value of the efficiency, and calculation according to formula (3), on the contrary, leads to an overestimated value of the gear efficiency. However, they can also be used, since in the considered range of gear ratios the maximum relative error does not exceed 0.5%.

Formulas (2) and (3) can be written in a more general form:

,						(38)

where 	k – is a some multiplier.

In formula (2) (curve 3 in Fig. 4)  (at  k ≈ 2.75), and in formula (3) for meshing of zero wheels k = 2.3 (curve 4 in Fig. 4). Therefore, taking into account the position of curves 3 and 4 relative to curve 1, the discrepancy of the results obtained by formulas (37) and (38) can be reduced if we take k = 2.5. However, it should be borne in mind that curve 1 reflects a partial case, since the derivation of formula (37) was based on the assumption of an equal distribution of the normal force between pairs of teeth during the double-pair meshing phase.

CONCLUSION

1. As a result of the study, analytical expressions for determining the efficiency of an external involute spur gear, considering the features of the single-pair and double-pair meshing periods, were derived. Rolling friction was not taken into account.

2. In the case of a double-pair meshing, the efficiency value depends on the load distribution between the pairs of teeth. Increasing the load on the pre-pitch pair of teeth leads to a decrease in the efficiency of the meshing in the reduction mode, i.e., provided that the gear with fewer teeth is the driver. With an equal distribution of the normal force between the pairs of teeth, the efficiency does not depend on the rotation angle of the gear wheel and remains constant during the double-pair meshing phase.

3. Simplified formulas (1), (2) and (3) allow determining the gear efficiency with sufficient accuracy for engineering calculations. For the assumed assumption of equal distribution of normal force between pairs of teeth, formula (1) provides the greatest accuracy of calculations, but it, unlike formulas (2) and (3), requires determining the gear face overlap ratio.

Acknowledgments

The author would like to express their gratitude to Kharkiv National Automobile and Highway University and the organizing committee of the Modern Automotive Industry, Transport and Road Infrastructure (MAITRI) conference.

Conflict of interests

The author declares that there is no conflict of interests regarding the publication of this paper.

References

[1] V. T. Pavlyshche, Fundamentals of Design and Calculation of Machine Elements, 2nd ed. (Afisha, Lviv, 2003), p. 560.

[2] A. V. Haidamaka, Machine Elements. Fundamentals of Theory and Calculations (NTU “KhPI”, Kharkiv, 2020), p. 275.

[3] L. V. Kurmaz, Fundamentals of Design of Machine Elements (Textbook of NTU “KhPI”, Kharkiv, 2010), p. 532.

[4] V. O. Malashchenko and V. V. Yankiv, Machine Elements. Design of Mechanical Drive Components (Novyi Svit-2000, Lviv, 2018), p. 264.

[5] K. Michaelis, B.-R. Höhn, and M. Hinterstoißer, Ind. Lubr. Tribol. 63, 46 (2011); https://doi.org/10.1108/00368791111101830.

[6] T. Lohner and C. Paschold, Forsch. Ingenieurwes. 89, 2 (2025); https://doi.org/10.1007/s10010-024-00768-w.

[7] M. Autiero, M. Cirelli, G. Paoli, and P. P. Valentini, Lubricants 11, 303 (2023); https://doi.org/10.3390/lubricants11070303.

[8] M. Yilmaz, T. Lohner, K. Michaelis, and K. Stahl, Forsch. Ingenieurwes. 83, 327 (2019); https://doi.org/10.1007/s10010-019-00373-2.

[9] F. Concli and M. N. Mastrone, Forsch. Ingenieurwes. 87, 1181 (2023); https://doi.org/10.1007/s10010-023-00698-z.

[10] M. Häggström, “Thermal modelling of a truck gearbox,” Master’s thesis (Luleå Univ. Technol., 2017), p. 79.

[11] L. Braumann, V. S. de Viteri, B. Morhard, T. Lohner, J. Ochoa, and H. Amri, J. Mater. Sci. Mater. Eng. 20, 29 (2025); https://doi.org/10.1186/s40712-024-00204-5.

[12] B. Morhard, T. Lohner, and K. Stahl, J. Mater. Sci. Mater. Eng. 19, 1 (2024); https://doi.org/10.1186/s40712-024-00143-1.

[13] L. Hildebrand, F. Dangl, C. Paschold, T. Lohner, and K. Stahl, Appl. Sci. 12, 10386 (2022); https://doi.org/10.3390/app122010386.

[14] J. Li, C.-C. Li, and J. Huang, in Proc. 3rd Annu. Int. Conf. Mech. Mech. Eng. (MME 2016), Chengdu, China, 2016 (Atlantis Press, 2017), p. 298; https://doi.org/10.2991/mme-16.2017.41.

[15] K. I. Zablonskyi, I. M. Bilokoniev, and V. M. Shchokin, Theory of Mechanisms and Machines (Vyshcha shkola, Kyiv, 1989), p. 376.

[16] I. Hofman, P. Sergeant, A. Van den Bossche, S. Koroglu, and S. Kesler, Int. J. Veh. Des. 72, 262 (2016); https://doi.org/10.1504/IJVD.2016.080592.

[17] S. I. Kryshhtopa, L. I. Kryshhtopa, I. M. Mykytii, M. M. Hnyp, and F. V. Kozak, Naftohazova Enerhetyka 36(2), 106 (2021); https://doi.org/10.31471/1993-9868-2021-2(36)-106-119.