Effect of Magnetic Field on the Energy of Light Holes at the Valence Band Ceiling of A Narrow-Gap Quantum Well

Ulugbek Erkaboev, Rustamjon Rakhimova), Muzaffar Dadamirzaev,                        Jasurbek Mirzaev, Nozimjon Sayidov, Ulugbek Negmatov

Namangan State Technical University, Namangan, Uzbekistan

a)Corresponding author: rgrakhimov @gmail.com

Abstract: In this paper develops a method for determining the dependence of the allowed bands ,  of narrow-gap quantum well semiconductors on the magnetic field B and temperature T, and uses this method to determine the dependence of the forbidden zone width of the narrow-gap quantum well on the magnetic field B and temperature T. For this purpose, the effect of the magnetic field on the energy of light holes at the valence band ceiling of the narrow-gap quantum well, the effect of the magnetic field on the energy of free electrons at the bottom of the conduction band of the narrow-gap quantum well, the dependence of the two-dimensional energy density of states in the allowed bands of the narrow-gap quantum well on temperature and magnetic field, and the dependence of the forbidden band width of the narrow-gap quantum well on temperature and magnetic field were studied.

Keywords: semiconductor, quantum well, magnetic field, temperature, forbidden zone, energy.

INTRODUCTION

The main fundamental physical quantity of bulk and small-sized semiconductor structures is the band gap (Eg3d,Eg2d), the energy width of which allows us to predict the operational parameters of a semiconductor-based device. Therefore, determining Eg3d and Eg2d (if the band gap of newly created materials is not known) is one of the main tasks of semiconductor heterostructure technology. In addition, another important feature of Eg is its strong susceptibility to external influences, and even a change in Eg as a result of this influence radically changes the physical and chemical properties of a semiconductor device. There are a number of methods for determining the dependence of the band gap of semiconductors on external factors. In particular, in a number of works, a method for determining Eg(T) from the temperature dependence model of the surface density of states has been developed. In addition, a mechanism for explaining Eg(T) was created by the temperature change of the density of states, through the penetration of its “tail” into the bandgap. However, in these works, the effect of the quantizing magnetic field was not studied. In some works, the dependence of the bandgap of bulk and quantum-scale semiconductors on temperature, magnetic field, and hydrostatic pressure was studied. In particular, in works [1-5], the Eg3d(B,T) was calculated theoretically by changing the allowed bands of bulk semiconductor structures with temperature and magnetic field. In these works, it was proven that when the  condition is met, it turns into Eg3d(B,T)→Eg(T), that is, classical methods. This theory is mainly suitable for wide-bandgap 3D materials. In works [6-9], a model for Eg2d(B,T,d) quantum well semiconductors obeying the parabolic dispersion law was developed. A mechanism for the change of the bottom of the conduction band and the ceiling of the valence band (the boundaries of the allowed bands) of a rectangular quantum well with magnetic field and temperature was developed. As a result of the penetration of the tail of Ncs2d(B,T,d) and Nvs2d(B,T,d) into the Eg2d, the Eg2d(B,T,d) model was proposed. In this case, Eg2d(B,T,d) quantum well semiconductors with wide bandgap were also determined.

In addition, in works[10-15], the dependence of the energy density of states of the conduction band of narrow-gap semiconductor materials on magnetic field and temperature was theoretically determined. In this work, an analytical expression Ncs3d(n)(B,T) for the nonparabolic dispersion law was derived. The experimental results were interpreted for different T using the obtained theoretical reports. However, in these works, a perfect mathematical model for determining Nvs2d(n), Ncs2d(n)was not developed.

The main aim of this research work is to develop a method for determining the dependence of the allowed bands of narrow-gap quantum well semiconductors on the magnetic field B and temperature T, and to determine the dependence of the forbidden band width of the narrow-gap quantum well on the magnetic field B and temperature T using this method.

METHODS

This problem is solved by the three-band approximation. This approximation is a convenient solution of the equation for semiconductors with narrow bandgap quantum wells. In the considered approximation, the 8 X 8 interaction matrix can be written using and H1 as follows [1]

(1)

Here,

(2)

The right-hand column in the matrix (2) represents the energy states associated with H.

If we calculate the initial energy with the bottom of the conduction band of the quantum well, (Ec=0), then the following equation is obtained:

(3)

Here, .

From the formula (3) it is known that for k=0 the function E`(k) has four eigenvalues: E1=0; E2= E3= -Eg; and E4=- Eg-Δ. The condition E1=0 means that the counting starts from the bottom of the conduction band. The energies E2 and E3 denote the ceilings of the valence bands of heavy (Ev1) and light (Ev2) holes. The energy E4 indicates the spin orbital effect on the ceiling of the valence band.

To solve the problem, we use the following approximations:

1. Ignoring the spin orbital effect on the allowed bands of a narrow-gap semiconductor.

2. Ignoring the interaction with heavy holes, since the effective masses of light holes are close to the effective masses of free electrons.

In this case, Eg>>Δ and according to, the energy of light holes is determined as follows:

(4)

It can be seen from expression (4) that k2p2 (a P-matrix element, which is equal to P=-iℏm0<SPx>) is considered as the energy, since the condition that the k2p2Eg2 term is a dimensionless quantity must be met. In this case, taking into account the above matrix element formula and the dimensionless quantities k2p2Eg2, (4) can be written as follows:

(5)

In expression (5), the energy of light holes in narrow-gap bulk semiconductors at the valence band ceiling depends mainly on the effective masses and wave numbers of the light hole in the XYZ axes. In this case, the band gap (Eg) of the narrow-gap bulk semiconductor is considered constant. From formula (5), natural questions arise:

1. How to use (5) if the material under the influence of the magnetic field is a quantum-enclosed narrow-gap, heterostructure semiconductor?

2. It is known that the Eg of narrow-gap bulk or small-sized semiconductors is very sensitive to external factors. In this case, how are  Ev3d(k,Eg) and Ev2d(k,Eg,d) determined?

3. How does a change in Ev2d(k,Eg,d) affect the energy density of states in the valence band of the quantum-enclosed?

To solve these problems, it is necessary to create a new mathematical model.

By applying expression (5) to narrow-gap semiconductors with quantum wells, the following expression is obtained:

(6)

Formula (6) is the energy of light holes in a narrow-gap quantum well at the valence band ceiling.

Here, nz is the number of quantum wells, d is the thickness of the narrow-band quantum well, and m* is the effective mass of the light well.

As can be seen from formula (6), the light well energy at the valence band ceiling of the quantum well depends on the quantum well thickness, the light well effective mass, and the number of quantum wells.

Now, let us consider the effect of a strong magnetic field on a narrow-band quantum well. In particular, let the direction of the magnetic field induction vector  be along the Z axis and perpendicular to the XOY plane. This is called the longitudinal quantizing magnetic field. In this case, according to Landau theory and the laws of the quantizing magnetic field, the  terms of the free light well at the valence band ceiling of the quantum well are replaced by the  term.

Here NLv is the number of Landau levels in the valence band of a narrow-band quantum well, ɷcv is the cyclotron frequency of light holes.

It follows that formula (6) under the influence of a longitudinal quantizing magnetic field takes the following form:

Ev2dB,d,nz,NLv=-Eg21+1+4NLv+12ℏωcv+π2ℏ22md2nz2Eg21/2                                (7)

From the derived formula (7) it is clear that the light hole energy at the valence band ceiling of the quantum well is transformed into discrete energy levels in all directions. This, in turn, makes the light hole energy analogous to the quantum dot energy. However, it is also necessary to consider another important physical quantity, Eg, which depends on B and d. The reason is that the change in the function Eg(B,d) is considered monotonically. From this, the function  becomes . In this case, the formula takes the following form:

(8)

The obtained formula (8) expresses the dependence of the light hole energy in the valence band gap of a quantum well on the magnetic field, the band gap width, the thickness of the quantum well, and the number of dimensional levels. Let us analyze expression (8) numerically and graphically. In works [13], the Joseph oscillations of a narrow-gap InAs quantum well semiconductor were determined. Here, the InAs quantum well thickness d=4 nm, B=0÷12 Tl, Eg(0)=0.35 eV, and nz were taken as.

By substituting these experimental values ​​into (8), we can obtain a graph of . Fig.1 shows the dependence of the energy of light holes in the vacancy ceiling of an InAs quantum well on the magnetic field for different Landau levels. As can be seen from this figure, the curve of the graph is reflected in the non-quadratic dispersion law of a narrow-gap InAs quantum well.

In addition, using formula (8), it is possible to calculate the two-dimensional energy density of states in the valence band of a narrow-gap quantum well.

FIGURE 1. The dependence of the energy of light holes in the valence ceiling of an InAs quantum well on the magnetic field for Landau levels.

CONCLUSION

In the process of carrying out this research work, the following conclusions were reached:

- A mathematical model was developed to determine the dependence of the light hole energy Ev2dEg(B,d),B,d,nz,NLv at the valence band ceiling of a narrow-gap quantum well and the free electron energy En2d,nonparab(Eg(B,d),B,d,nz,NLc) at the bottom of the conduction band of the quantum well on the magnetic field, the band gap width, the quantum well thickness, and the number of dimensional levels.

- A method was proposed to determine the dependence of the allowed bands Nvs2d(n), Ncs2d(n) of narrow-gap quantum well semiconductors on the magnetic field B and temperature T.

- Using this method, the dependence of the band gap width of a narrow-gap quantum well on the magnetic field B and temperature T was determined.

REFERENCES

- Tsidilkovskii, I.M. (1972). Electrons and holes in semiconductors. Energy spectrum and dynamics. Nauka Publishing House, Main editorial board of physical and mathematical literature, Moscow, 264-266.

- Sharibaev, N., Jabborov, A., Rakhimov, R., Korabayev, S., Sapayev, R. (2024). A new method for digital processing cardio signals using the wavelet function. BIO Web of Conferences, 130, 04008. https://doi.org/10.1051/bioconf/202413004008\

- Mirzabayev, B., et al. (2025). Controlling tensile strength in yarn using an ultrasonic water vaporizer. AIP Conference Proceedings, 3304, 030047. https://doi.org/10.1063/5.0269506

- Erkaboev, U.I., Rakhimov, R.G. (2023). Determination of the dependence of the oscillation of transverse electrical conductivity and magnetoresistance on temperature in heterostructures based on quantum wells. East European Journal of Physics, 2023(3), 133-145. https://doi.org/10.26565/2312-4334-2023-3-10

- Gulyamov, G., Erkaboev, U.I., Rakhimov, R.G., Mirzaev, J.I., Sayidov N.A. (2023). Determination of the dependence of the two-dimensional combined density of states on external factors in quantum-dimensional heterostructures. Modern Physics Letters B, 37(10), 2350015. https://doi.org/10.1142/S021798492350015X

- Alemu, G., & Menberu, M. (2019). Magneto-Stark effect on exciton in parabolic bandGaAs/〖Ga〗_(0.7) 〖Al〗_(0.3) As quantum well heterostructure. International Journal of the Physical Sciences, 14(7), 55–61. https://doi.org/10.5897/ijps2018.4736

- Erkaboev U., Rakhimov, R., Mirzaev, J., Sayidov, N., Negmatov, U., Mashrapov, A. (2023). Determination of the band gap of heterostructural materials with quantum wells at strong magnetic field and high temperature. AIP Conference Proceedings, 2789(1), 040056. https://doi.org/10.1063/5.0145556

- Başer, P. (2021). Effect of pressure, temperature, and magnetic field on the binding energy of the electron-hole system in III-V group semiconductors. Cumhuriyet Science Journal, 42(2), 403–412. https://doi.org/10.17776/csj.872732

- Erkaboev, U.I., Sayidov, N.A., Negmatov, U.M., Mirzaev J.I., Rakhimov, R.G. (2023). Influence temperature and strong magnetic field on oscillations of density of energy states in heterostructures with quantum wells HgCdTe/CdHgTe. E3S Web of Conferences, 401, 01090.

https://doi.org/10.1051/e3sconf/202340101090

- Minkov, G. M., Rut, O. E., Sherstobitov, A. A., Dvoretski, S. A., Mikhailov, N. N., & Aleshkin, V. Y. (2023). Energy spectrum of valence band in HgTe quantum wells on the way from a two to the three dimensional topological insulator. arXiv (Cornell University). https://doi.org/10.48550/arxiv.2308.01745

- Erkaboev U., Rakhimov, R., Mirzaev, J., Sayidov, N., Negmatov U., Abduxalimov M. (2023). Calculation of oscillations in the density of energy states in heterostructural materials with quantum wells. AIP Conference Proceedings, 2789(1), 040055. https://doi.org/10.1063/5.0145554

- Landau, L. (1930). Diamagnetismus der Metalle. Zeitschrift für Physik. 64, 629-637.

https://doi.org/10.1007/BF01397213

- Joseph, Y., Mehdi, H., Brenden, A.M., William, M., Matthieu, C.D., Kasra, S., Kaushini, S.W., Giti, A.K., Yasuhiro, H.M., Yoshimitsu, K., Zhuo, Y., Sunil, T., Christopher, J.S., Javad, S. (2020). Experimental measurements of effective mass in near-surface InAs quantum wells. Physical review B. 101, 205310- 205317. https://doi.org/10.1103/PhysRevB.101.205310

- Sharibaev, N., Jabborov, A., Rakhimov, R., Korabayev, S., Sapayev, R. (2024). A new method for digital processing cardio signals using the wavelet function. BIO Web of Conferences, 130, 04008. https://doi.org/10.1051/bioconf/202413004008

- Patricio, M. a. T., Villegas-Lelovsky, L., De Oliveira, E. R. C., Marques, G. E., LaPierre, R. R., Toropov, A. I., & Pusep, Y. A. (2023). Magnetic field effects on the valence band of AlGaAs and InGaAsP parabolic quantum wells. Physical Review. B./Physical Review. B, 108(3). https://doi.org/10.1103/physrevb.108.035416